`

PROPOSED MINISTRY OF JUSTICE AND COURTS ADMINSTRATION’S (MJCA) NEW COURTHOUSE & JUDGES RESIDENCE AT SALELOLOGA, SAVAII

Architectural Technical Specification

November 2022

Revision Status

| Revision | Date | Description | Author Name Position Title |  | Approver Name Position Title |  |
| --- | --- | --- | --- | --- | --- | --- |
| A | 25/11/2022 | Draft Tender For Client review | Velda S.T Caffarelli | Architect / Project Manager |  |  |

TABLE OF CONTENTS

TABLE OF CONTENTS	1

0131B PRELIMINARIES	1

0171B GENERAL REQUIREMENTS	12

0221 SITE PREPARATION	36

0181 ADHESIVES, SEALANTS AND FASTENERS

0183B METALS AND PREFINISHES

0193 BUILDING ACCESS SAFETY SYSTEMS

0315 CONCRETE FINISHES

0382 LIGHT TIMBER FRAMING

0423P ROOF SHEETING

0451B WINDOWS AND GLAZED DOORS

0453B DOORS AND ACCESS PANELS

0455 DOOR HARDWARE

0454B OVERHEAD DOORS

0456 LOUVRE WINDOWS

0471 THERMAL INSULATION AND PLIABLE MEMBRANES

0511B LINING

0525B CUBICLE SYSTEMS

0551B JOINERY

0552B METALWORK - FABRICATED

0611 RENDERING AND PLASTERING

0621 WATERPROOFING – WET AREAS

0631B CERAMIC TILING

0671P PAINTING

0811S SANITARY FIXTURES

0812S TAPWARE

0815P BOILING, CHILLED AND FILTERED WATER DISPENSERS

0581B SIGNAGE

0195P TACTILE INDICATORS AND STAIR NOSINGS

0457 EXTERNAL SCREENS

APPENDICES : EXTRA INFORMATION (MANUFACTURER’S):-

APPENDIX A – RECOMMENDED CEILING SYSTEM

APPENDIX B – WON-DOOR FOLDING PARTITION

APPENDIX C – COURTROOM FURNITURE & EQUIPMENT

APPENDIX D - HOLDING CELL TECHNICAL INFORMATION (CELL SECURITY LTD CATALOGUE)

42

47

49

51

54

58

69

79

85

91

93

99

103

106

122

127

133

139

148

156

158

161

165

175

178

A B C D

STRUCTURAL, CIVIL AND BUILDING SERVICES SPECIFICATION ATTACHED AS SEPARATE DOCUMENTS.

The drawings and specifications are not to be read as mutually exclusive documents. The specifications, drawings and contract complement each other and form the contract set.

0131B PRELIMINARIES

- GENERAL

  - SCOPE OF WORK

The project encompasses the construction of the New Courthouse & Judges Residence and auxiliary works at the Salelologa site in Savaii, Samoa. The Specification shall be read in conjunction with the Samoa Bidding Document (SBD). The works shall include all activities as noted in the SBD and Specifications and necessary to construct the building in compliance with the Construction Drawings and Specifications. These works includes but not limited to;

    - General building construction

Electrical (Power, Lighting and Backup) & Data reticulation

    - Audio / Sound System (Courtrooms) & CCTV Security System

    - AC & Ventilation mechanical systems

    - Sanitary Plumbing including fixtures

    - Joinery (Window, doors, general miscellaneous joinery)

    - Joinery (fixed furniture)

    - And auxiliary services as listed below.

Auxiliary works to service the new MJCA Courthouse & Judges Residence Compound. The works shall include all activities necessary to construct the auxiliary services in compliance with the Construction Drawings and Specifications. These works includes but not limited to;

    - Vehicle entry cross over and Entry gates.

    - Pedestrian entry cross over, gates and fencing.

    - General vehicle parking areas

    - Emergency genset  (~145KVA generator), diesel storage tank, bund and Shelter

    - Transformer and HV/LV cables to and from the street to the buildings.

    - Soakage pits for showers, sinks & miscellaneous drainage.

    - Septic tanks & absorption fields for sanitary drainage.

    - Perimeter Fencing

    - Site Electrical

The tender documents will request each respective tenderer to price the project based on the above breakup to allow each tender to be assessed. Refer to the Volume 1 RFP document for additional information.

General conditions Contract:

The contract used will be the Government of Samoa - Conditions of Contract and Particular Conditions of Contract and the whole of the clauses of same are to be read in conjunction with this specification, drawings, schedules and bidding documents.

Interpretation

General: The words principal, and contract administrator have the same meaning, respectively, as owner and Supervisor , unless the context requires otherwise.

Cross reference: The clause INTERPRETATION, in the 0171 General requirements work section, also applies.

Contract Preparation Fee - Not Applicable

Nature of the contract – Lump Sum

  - DISCREPANCIES IN THE DOCUMENTS Mutually Explanatory Documents

The several documents which constitute or evidence the Contract shall be taken as mutually explanatory and anything contained in one but not in the other shall be equally binding as if contained in all. Any discrepancies between the drawings and specification shall imply the most expensive option is to be considered for evaluation of the allowance in the Contract and the matter referred to the Supervisor for a decision.

Minor Items:

The Contractor shall without adjustment to the Contract Sum, supply and execute minor items not expressly mentioned in the Contract but necessary for the satisfactory completion and performance of the work under the Contract.

  - THE SITE Site restrictions

Entry permits: Make available, to persons entering designated secure areas, valid entry. Make sure these persons comply with conditions of entry.

Designated secure areas: MJCA Site. Conditions of entry: NA

List: NA.

Site limitations: Comply with the following restrictions on the use of the site:

The main works vehicle access will be off the Salelologa Sasae Drive. The contractor must ensure this access through Salelologa Sasae Drive is kept clear and in good condition at all times. The contractor is to allow for traffic management throughout the day to ensure the public is safe throughout the works contract.

Occupied premises

General: For the parts of the site, documented in the Occupied premises schedule:

- Allow occupants to continue in secure possession and occupancy of the premises for the required period.

- Maintain safe access for occupants.

- Arrange work to minimise nuisance to occupants and for their safety.

- Protect occupants against weather, dust, dirt, water or other nuisance, by such means as temporary screens.

Proposals: Submit details of proposed methods.

| Occupants | Occupied premises | Period of occupancy |
| --- | --- | --- |

Purpose of submission: Information only. Occupied premises schedule - Not Applicable.

Protection of persons and property

Temporary works: Provide and maintain required hoardings, barricades, guards, fencing, shoring, temporary roadways, footpaths, signs, lighting, watching and traffic flagging to protect the works from unauthorized access.

Public viewing area: N/A

Access ways, services: Do not obstruct or damage roadways and footpaths, drains and watercourses and other existing services in use on or adjacent to the site. Determine the location of such services.

Property: Do not interfere with or damage property which is to remain on or adjacent to the site, including adjoining property encroaching onto the site, and trees.

Rectification

Access ways, services: Rectify immediately any obstruction or damage to roadways and footpaths, drains and watercourses and other existing services in use on or adjacent to the site. Provide temporary services whilst repairs are carried out.

Property: Rectify immediately any interference or damage to property which is to remain on or adjacent to the site, including adjoining property encroaching onto the site, and trees.

Existing services: Not Applicable Site Security

Security of the site at all times is to be provided to the satisfaction of the Superintendent. In addition to

agreed security measures the Contractor shall:

  - not allow and shall not approve the entry of any person onto the Site who is not concerned with the execution of the Work unless prior written instruction has been obtained from the Superintendent on the approval of the Principal;

  - restrict the number of vehicles on site to the minimum necessary to execute the work. Non- essential vehicles (including those of the Contractors employees) are to be kept off the works site;

  - not permit to be taken, photographs or sketches of the works, nor shall issue or permit to be issued, any information by way of advertisement or publication about the Works unless prior written approval is obtained from the Principal; and

  - not divulge to any unauthorized person information, documents and the like furnished to the Contractor by the Principal or the Superintendent unless prior written instruction has been obtained from the Superintendent on the approval of the Principal.

  - Supply, erect & remove a temporary (to Engineers approval) site security fence (1.8m high) around the perimeter of the work site and allow for temporary vehicle and separate pedestrian access points.

Site Safety

All matters and conditions which concern the safety, health and welfare of persons working on the Site covered by this Contract shall be the responsibility of the Contractor, shall be at the Contractor’s expense, and shall be to the satisfaction of the Superintendent. The Contractor is to comply with all relevant statutory occupational health and safety rules and regulations, and shall:

  - continuously maintain a suitable First Aid Kit in a dedicated First Aid Room, and shall ensure that one of his employees on the Site at all times during working hours is the holder of a current First Aid Certificate;

  - ensure that all his employees and his Subcontractor’s employees wear suitable protective clothing and safety gear at all times while engaged upon work on the Site;

  - make available for inspection, testing and approval by the Superintendent or Principal all electrical power tools that will be used by the Contractor for the execution of the works. The Superintendent, on the approval of Principal, has the right to confiscate any equipment which in his opinion is faulty or defective and if necessary have such equipment repaired at the Contractor’s or Subcontractor’s expense;

  - obtain the prior written approval of the Superintendent for the use of “Ramset” guns and other explosive devices on the site. Such devices shall only be operated by fully qualified and licensed personnel, and in accordance with the rules and regulations governing their use; and

  - provide appropriate instructions in safety measures and accident prevention to all employees, subcontractors and the like engaged in the execution of the works.

  - Accidents: Promptly notify the superintendent (within 24 hours) of the occurrence of the following:

  - Accidents involving death or personal injury;

  - Accidents involving loss of time; and

  - Incidents with accident potential such as equipment failure.

  - Accident reports: Submit reports of accidents.

  - Environmental Protection

  - Comply with the Government of Samoa Environmental Policy and any other environmental protection provisions in the Contract and with the requirements of any statute, by-law, standard and the like related to environmental protection, including control of the following:

  - excavation, silt control and drainage;

  - disposal of contaminants;

  - disposal of refuse;

  - trucking;

  - storage of fuels and hazardous compounds;

  - suppression of dust; and

  - suppression of construction noise.

  - Comply with all notices and instructions issued by the Superintendent in relation to such Rules and Regulations.

  - CONSTRUCTION PLANT Access roads

Owner’s existing roads: Use only designated roads.

Location: Use access roads designated for the future cross overs. Allow for a temporary site fence and access gates as required.

Parking

Provide parking on site for site vehicles. Use of existing services

General: Existing services may be used as temporary services for the performance of the contract

subject to conditions of use as documented in the Existing services schedule. Existing services schedule

| Service | Conditions of use |
| --- | --- |
| Water | Supply and install a temporary sub water meter to record the usage and pay for use based on commercial rates available through Samoa Water Authority. The contractor will pay the cost to supply, install, use and remove the sub meter. |
| Power | Supply and install a sub power meter to record the usage and pay for use based on commercial rates available through EPC. The contractor will pay the cost to supply, install, use and remove the sub meter. |
| Telephone and Internet Service Connection | Supply and install temporary phone and data connection to the site office. The contractor will pay the cost to supply, install, use and remove the connection. |

Contractors site office

To comply with standard construction industry requirements, the Contractor shall provide his own temporary site accommodation and proper facilities for the safe keeping and laying out of his copies of the Contract documents. Remove site accommodation at ‘Practical Completion’ of the Works. The Contractor shall provide his own telephone and pay all costs and charges in connection therewith.

Workmen's Amenities:

Provide all statutory and necessary amenities and sanitary facilities for the workmen and other persons lawfully upon the site and remove them at ‘Practical Completion’ of the Works. Occupation of any part of the works and site for the provision of workmen’s’ amenities shall not be permitted without prior approval by the Superintendent

Protective clothing

Protective clothing: Make available protective clothing for the use of visitors.

- Safety helmets: To AS/NZS 1801, Type 1.

- Certification: Required.

. Certification provider: An organization accredited by the Joint Accreditation System of Australia and New Zealand (JAS-ANZ).

Number of helmets: 4 Temporary services

Requirement: Provide temporary phone and internet service within the site office. The cost to install,

use and disconnect the service will be borne by the contractor. Project signboards

General: Provide project-specific signboards and the following:

- Locate where directed.

- Maintain in good condition for duration of the work.

- Obtain permission for removal.

- Remove on completion. Project signboard description

Location: Located on Salelologa Sasae Drive.

Size: 1200mm H x 2400mm W Text, graphics: TBA

Other details: Colour image of the proposed works and text noting the contractor, client, client logo, project manager and consultants names including their roles and logos.

Adjoining Buildings Survey:

Contractor is to undertake and dilapidation survey of any adjoining structure, driveway or building which may be affected by the works. This survey shall include photographs, and notes outlining the existing condition prior to the work being under taken. Please provide 3 copies of this document to the client prior to the works beginning.

Statues and Subordinate Legislation:

The Contractor shall remain responsible for compliance with all relevant Statutory Requirements in accordance with the Contract Conditions.

Inspection and Provision of Insurances

Before commencing work under the contract, the Contractor shall provide evidence to the satisfaction and approval in writing of the Owner of the insurance's having been taken out to comply with the relevant clauses in the Conditions of Contract.

  - BUILDING THE WORKS Survey marks

Definition: The term survey mark means a survey peg, bench mark, reference mark, signal, alignment, level mark or any other mark used or intended to be used for the purpose of setting out, checking or measuring the work.

Care of survey marks:

Preserve and maintain the owner’s survey marks in their true positions. Rectification: If the owner’s survey marks are disturbed or obliterated, immediately give notice and rectify the disturbance or obliteration.

Safety

Accidents: Promptly notify the Superintendent of the occurrence of the following:

- Accidents involving death or personal injury.

- Accidents involving loss of time.

- Incidents with accident potential such as equipment failure, slides and cave-ins. Accident reports: Submit reports of accidents.

- Purpose of submission: Information only. Contractor's representative

General: Must be accessible, and fluent in English and technical terminology.

Subcontracting

General: Submit a complete list of proposed subcontractors and suppliers. Workmanship and materials

- All workmanship and materials to be the best of their respective kinds.

- The Contractor shall ensure he/she orders material within sufficient time to ensure his/her supplies guarantee delivery of these items at times to accord with the building programme. Any lapse in this respect will not be accepted as a just cause for any extension of time to the contract, unless satisfactory evidence is produced that delays in delivery are beyond the Contractor's and the supplier's control.

- The Contractor shall ensure he orders material within sufficient time to ensure his supplies Superintendent .

- All materials supplied to the works shall be suitably stored and protected to the approval of the Superintendent

- Materials are to be stored so as not to be accessible to the public. Smoking on construction sites

The Contractor shall at all times ensure that all workmen and visitors on the construction site comply

with the following policy on smoking.

In respect of construction sites, smoking is prohibited on/in the following:

  - In site offices, lunchrooms or enclosed toilet facilities.

  - Inside existing premises that are designated as “no smoking” areas.

Failure to comply with this requirement may result in the offender being ordered permanently off the site.

  - NOMINATED AND SPECALIST SUBCONTRACTORS Nominated and Specialist subcontract work

There are no Nominated Sub Contractor for the works.

The Contractor is to allow for in their Tender the any required specialist sub-contractor works.

The Contractor is responsible for administering, coordinating, supervising and generally attending upon the execution of work under the Contract by the selected Nominated and specialist Subcontractor. The Contractor shall provide for each Subcontractor all normal facilities for the proper performance of the Subcontract Work under the Contract including the following:

    - access to the Site

    - storage areas

    - use of water (Nominated Subcontractors shall provide their own hoses and fittings) –

    - use of electric light and power supply (Subcontractors shall provide their own cables and portable lamps)

    - use of scaffolding and hoisting facilities as provided for the Contractor’s own use while in position and at reasonable times including operators, but not the labour for loading or unloading such hoisting facilities ( Subcontractors shall pay the Contractor for the use of such scaffold and hoisting facilities at rates agreed between them)

    - sanitary conveniences

    - statutory amenities for drinking water, messing and changing

    - storage of tools

    - first aid and safety measures

    - rubbish removal from site Any additional facilities required and any cutting of holes, chases, making good or other builders work required in connection with the work of any Subcontractor, shall be the responsibility of the Subcontractor concerned, unless otherwise specified or shown on the drawings.

Nominated and Specialist subcontract direction

Where in any Nominated Subcontract it is specified that any matter or thing is to be referred, submitted, supplied, handed over or notified by the Nominated Subcontractor to the Superintendent or application is to be made to the Superintendent then the Nominated Subcontractor shall as and when required initiate such action with the Contractor and thereafter the Contractor shall as soon as practicable take the matter or thing or application to the Superintendent together with any amendments pursuant to the Subcontract as the Contractor may consider necessary.

Where in any Nominated Subcontract document it is specified that any action or approval is required of the Superintendent or that any work of the Nominated Subcontractor is to be done to the satisfaction of the Superintendent then notification of such action, approval or work done to the satisfaction of the Superintendent or not approved or done as the case may be, shall be given by the

Superintendent to the Contractor and thereafter the Contractor shall as soon as practicable give the notification to the Subcontractor.

Subcontractor with such additional requirements, directions or instructions pursuant to the Subcontract as the Contractor may consider necessary.

Program of work

Construction program: Show the following:

- Sequence of work.

- Critical paths of activities related to the work.

- Long lead time items inclusive of order and delivery dates.

- Allowance for holidays.

- Activity inter-relationships.

- External dependencies including provision of access, document approvals and work by others.

- Periods within which various stages or parts of the work are to be executed. Time scale: Working days based on 5 days a week

Updated program: Identify changes since the previous issue, and show the estimated percentage of completion for each item of work.

Program chart: Display in the contractor’s site office an up-to-date bar chart and network diagram based on the construction program.

Site meetings

General: Hold and attend site meetings throughout the contract and arrange attendance of appropriate subcontractors, the Superintendent , and appropriate consultants.

Frequency: Fortnightly.

Minutes: Keep minutes of site meetings. Within 5 working days after each meeting, submit to each party written copies of the minutes.

- Purpose of submission: Review.

Contacts: At the first site meeting, submit names and telephone numbers of responsible persons who may be contacted after hours during the course of the contract.

- Purpose of submission: Information only. Progress photographs

General: Take colour progress photographs within 5 working days before each site meeting. At each

site meeting submit 2 sets of glossy prints, and the digital files. Identify the project, date, time, location and orientation.

Purpose of submission: Information only. Minimum frequency: Weekly

Minimum number: 50 Format: JPEG

  - WORKPLACE SAFETY AND HEALTH

Compliance

The Contractor shall comply with the Occupational Safety & Health Act 2002 (OSH Act) and associated the regulations and with any amendments that may be made to the Act and regulations from time to time. The Act, regulations, Codes of Practice and other safety information can be accessed from.

The Contractor is deemed to have control of the Site for the purposes of executing the work under the Contract. Accordingly the Contractor is responsible for ensuring that, wherever practicable, its employees and all other persons entering and moving about the Site, for whatever purpose, are not exposed to hazards.

Occupational Safety and Health Information

Prior to the commencement of work on Site, or at any other time the Superintendent directs, the Contractor shall consult with the Owner and the Superintendent for the purposes of ensuring that, as far as practicable, the construction work can be done without risk to the health and safety of either those doing the work, or anyone else who may be at or near the construction site. The Contractor shall ensure that the following information is recorded, reviewed and updated regularly, and kept until the Works are completed:

- The identification of hazards to which a person at the construction site is likely to be exposed;

- An assessment of the risk of injury or harm to a person resulting from those hazards; and

- The risk control measures through which the risk of injury or harm may be eliminated or otherwise controlled. If the Contractor becomes aware that a change in the design of the Works could better eliminate or control a risk of injury or harm to those doing the construction work, or anyone else who may be at or near the Site, the Contractor shall ensure that this information is passed on to the Owner and the Superintendent.

If the Contractor becomes aware that a change in the design of the Works could better eliminate or control a risk of injury or harm to those doing the construction work, or anyone else who may be at or near the Site, the Contractor shall ensure that this information is passed on to the Owner and the Superintendent.

The Contractor shall ensure that any Occupational Safety and Health information it receives from the Owner or the Superintendent is incorporated into its hazard identification, risk assessment and risk control measures.

Safety Management Plan:

Prior to the commencement of the work on Site, the Contractor shall prepare and implement a Safety Management Plan relevant to the works under the Contract. The Safety Management Plan shall be maintained, and where necessary updated, throughout the Contract. The Safety Management Plan shall be appropriate to the risks associated with the work under the Contract and shall contain provision for, but not be limited to, the following elements:

- Occupational Safety and Health induction for new employees;

- listing of competencies required for specialist work (eg. rigger, scaffolder)

- the arrangements for managing occupational safety and health incidents on the Site, including accident/incident reporting and investigation

- the safety rules at the Site and description of the arrangements for ensuring that all persons working at or visiting the Site are informed of the rules;

- hazard identification, risk assessment and risk control including routine inspection processes;

- plant/equipment inspection processes;

- pre-job planning, procedural issues and JSA's (Job Safety Analyses, also known as Safe Work Method Statements). Within the Safety Management Plan, particular attention is to be given to identifying hazardous activities including, but not limited to, work in confined spaces, asbestos removal, demolition work, excavation work, working near power lines and live conductors and working at heights;

- emergency response and evacuation procedures;

- methods of communicating and consulting with employees and transmitting new work procedures to employees;

- hazardous substances exposure management;

- site security;

- purchasing/hiring controls (to avoid unknowingly bringing hazards onto the Site;

- quantitative performance measures (application to be determined by contract size and duration;

- and any other matters that the Superintendent may direct from time to time. Each element of the Safety Management Plan shall specifically address:

- the person on the Site who shall take responsibility for the successful implementation of each element;

- the hierarchical structure by which the responsibility is performed, and;

- the specific manner by which the element is performed.

The Contractor shall prepare the Safety Management Plan in conjunction with a competent person suitably experienced and qualified in safety matters. The Owner may direct that the Contractor prepare the Safety Management Plan in conjunction with the Consultant. Prior to the commencement of the Works, the Contractor shall certify to the Superintendent that its Safety Management Plan

    - has been prepared

    - has been provided to each person doing construction work at the Site (where this is practical) or is otherwise available for inspection on the Site; and

    - has been implemented on Site. Occupational Health and Safety Induction training:

The Contractor shall not permit its employees, the employees of other parties or other persons to

commence work on the Site until they have been inducted. Such induction shall include but not necessarily be limited to:

- familiarization with the Safety Management Plan;

- reporting of accidents and incidents which shall include the type of events to be reported, how an event is reported and to whom the event is reported;

- emergency procedures - which shall cover the procedure for a medical emergency and for evacuation of the Site in the event of a life threatening situation arising; d) personal protective equipment (PPE) - the standard requirements for the Site;

- lifting and manual handling skills

- sun protection;

- avoidance of noise induced hearing loss;

- location of and access to First Aid on the Site;

- legislative framework - an employee’s rights and responsibilities under the Act and Regulations;

- procedure for the resolution of safety issues at the workplace in accordance with the Act and regulations of the Government of Samoa (OSHA Laws and regulations).

First Aid, Workplace Amenities and PPE for practical guidance for the provision of first aid, workplace amenities and access to these amenities. The Contractor shall induct its employees, the employees of other parties or other persons working on the Site with regard to JSA’s and shall prepare “Training Session Attendance” sheets signed by each attendee verifying that such induction has occurred.

Upon commencement of work on the Site, the Contractor shall further induct each employee, the employees of other parties or other persons working on the Site with regard to all significant hazards associated with their particular activity and area of employment on the Site and where relevant shall include the use of powered plant, tools and equipment.

Safe Work Method Statements:

Where construction work on the Site is high risk construction work, the Contractor shall ensure that a person having day-to-day, on site control of the high-risk work at the site gives the Contractor a written Safe Work Method Statement before the high risk construction work commences.

The Contractor shall ensure that the Safe Work Method Statement is kept up to date. For the purposes of this clause, high risk construction work includes:

- work involving a risk of a person falling 2 metres or more; or

- work on telecommunications towers; or

- the demolition of any existing structure; or

- disturbing or removing asbestos; or

- the alteration to a structure that requires the structure to be temporarily supported to prevent its collapse; or

- work within a confined space;

- the excavation to a depth of more than 1.5 metres; or

- the construction of tunnels; or

- the use of explosives; or

- work on or near pressurized gas pipes (including distribution mains); or

- work on or near chemical, fuel or refrigerant lines; or

- work on or near energized electrical installations and lines (whether overhead or underground); or

- work in an area that may have a contaminated or flammable atmosphere; or

- work involving tilt-up or precast concrete; or

- work on or adjacent to roads or railways that are in use; or

- work on a construction site where there is movement of powered mobile plant; or

- work in an area where there are artificial extremes of temperature; or

- work in, over or adjacent to water or other liquids if there is a risk of drowning; or

- work involving diving.

The Contractor shall ensure that a Safe Work Method Statement will cover all high-risk construction work done at the site, and that the high risk construction work is carried out in accordance with the statement. Where work is carried out other than in accordance with the Safe Work Method Statement, the Contractor shall ensure that the work ceases (when it is safe to do so) and does not resume until the statement is complied with.

The Contractor shall ensure that the Safe Work Method Statement shall be kept up to date and describe: - each high-risk construction work activity that is or includes a hazard to which a person at the construction site is likely to be exposed; - the risk of injury or harm to a person resulting from any such hazards;

    - the safety measures to be implemented to reduce the risk, including the control measures to be applied to the activity or hazards; and

    - a description of the equipment used in the work activity; and

    - the qualifications and training (if any) required for persons doing the work to do it safely.

  - MAINTENANCE MANUAL ON COMPLETION

The Contractor at Practical Completion is to provide three copies of the maintenance manual and operating instructions. The maintenance and operating manual shall have the names, addresses, phone numbers, facsimile numbers and email addresses of the Superintendent , consultants, Contractor, and all Subcontractors, listing trade or product supplied, name of subcontractor & contact number and be to the approval of the Superintendent .

The Contractor shall provide a schedule of any warranties and instruction booklets supplied. The Contractor shall provide a separate appendix listing the final colour schedule including any changes as directed on site by the Superintendent after issue of colour schedule.

Refer to the General Requirements section of this specification for warrantee requirements.

Note: Final payment is conditional upon receipt of the maintenance manual and operating instruction in a condition acceptable to the Superintendent and within 30 days of Practical Completion. Non- compliance will result in the Superintendent commissioning the relevant consultants to prepare the material with cost being deducted from Retention.

  - COMPLETION OF THE WORKS Final cleaning

General: Before the date for practical completion, clean throughout, including interior and exterior surfaces exposed to view. Vacuum carpeted and soft surfaces. Clean debris from the site, roofs, gutters, downpipes and drainage systems. Remove waste and surplus materials.

Samples: Remove non-incorporated samples, prototypes and sample panels. Reinstatement

General: Before the date for practical completion, clean and repair damage caused by installation or

use of temporary work and restore existing facilities used during construction to original condition. Adjoining property

Evaluation: At practical completion, for properties described in the Adjoining properties to be

recorded schedule inspect the properties with the Superintendent and owners and occupants of the properties, recording any damage that has occurred since the pre-commencement inspection.

Certificate of occupancy

The Contractor shall liaise with the Local Authority for issue of the Certificate of Occupancy including but not limited to the following all to comply with Authority requirements.

Pest eradication

General: Employ suitably qualified pest exterminators. At practical completion submit a certificate stating that completed works are free of pest types documented in the Pest eradication treatments schedule.

Pest eradication treatments schedule

| Pest type to be treated | Eradication method |
| --- | --- |
| Rodent | Lockable rodent bait stations and tracking powders |

Removal of plant

General: Within 10 working days after practical completion, remove temporary works and construction plant no longer required. Remove the balance before the end of the defects liability period.

  - PAYMENT FOR THE WORKS Anticipated progress claims schedule

General: At commencement of the works, submit a schedule of anticipated progress claims for the contract period. Submit a revised schedule with each progress claim.

Purpose of submission: Information only. Progress claims

Break-down: With each progress claim, submit a statement of amounts claimed in respect of each

work section or trade heading designated in the specification. Purpose of submission: Review.

Method of measurement

General: In conformance with the principles of the Australian Standard Method of Measurement of Building Works (ASMM).

Other civil engineering work: To AS 1181.

  - MISCELLANEOUS

Contractor and owner to observe confidentiality

Publicity: Do not issue information concerning the project for publication in the media without prior written approval of the owner. Refer to the owner, enquiries from the media concerning the project.

Compliance with the law

Requirements of authorities: The owner, before entering into the contract, has given the notices, paid the fees, and obtained the permits, approvals and other authorizations.

Insurances

Contractor must maintain valid and current Workers Compensation in accordance with the laws and regulations. The Contractor must also maintain Contract Works insurance and Public Liability insurance throughout the contract for the period stated within the General Conditions and Particular conditions of contract.

Prior to the works starting the contractor much provide certificates of currency to the superintendent for confirmation and insure these certificates are updated and issued prior to the expiry date.

Permits and Development Approvals

Requirements of authorities: Contractor must have a valid building permit and development application for the works prior to mobilizing.

0171B GENERAL REQUIREMENTS

- GENERAL

  - PRECEDENCE General

Worksections and referenced documents:

- The requirements of other worksections of the specification override conflicting requirements of this worksection.

- The requirements of the worksections override conflicting requirements of their referenced documents. The requirements of the referenced documents are minimum requirements.

  - CROSS REFERENCES Common requirements

Requirement: Conform to the following work sections:

- 0181 Adhesives, sealants and fasteners..

- 0183 Metals and prefinishes.

- 0185 Timber products, finishes and treatment. Cross referencing styles

General: Within the text, titles are cross referenced using the following styles:

- Work section titles are indicated by Italicized text.

- Subsection titles are indicated by BOLD text.

- Clause titles are indicated by BOLD text.

- Subclause titles are indicated by Bold text.

  - REFERENCED DOCUMENTS Contractual relationships

General: Responsibilities and duties of the principal, contractor and contract administrator are not altered by requirements in the documents referenced in this specification.

Current editions

General: Use referenced documents which are the editions, with amendments, current 3 months before the closing date for tenders, except where other editions or amendments are required by statutory authorities.

  - INTERPRETATION Documentation conventions

Imperative mood and streamlined language: The words shall or shall be are implied where a colon is used following a keyword or within a sentence or sentence fragment.

Subject of sentences and phrases: Specification requirements are to be performed by the contractor, unless stated otherwise.

Abbreviations

General: For the purposes of this specification the following abbreviations apply:

- NBCS: National Building Code of Samoa 1991

- AS: Australian Standard.

- BCA: National Construction Code Series Volume One: Building Code of Australia Class 2 to 9 Buildings and Volume Two: Building Code of Australia Class 1 and Class 10 Buildings.

- GRP: Glass Reinforced Plastic.

- IP: Ingress protection.

- NATA: National Association of Testing Authorities.

- NCC: National Construction Code.

- NZS: New Zealand Standard.

- PCA: National Construction Code Series Volume 3: Plumbing Code of Australia.

- PVC: Polyvinyl Chloride.

- PVC-U: Unplasticized Polyvinyl Chloride. Also known as UPVC.

- SDS: Safety data sheets.

- VOC: Volatile Organic Compound.

- WHS: Work Health and Safety. Definitions

General: For the purposes of this specification, the following definitions apply:

- Access for maintenance: Includes access for maintenance, inspection, measurement, operation, adjustment, repair, replacement and other maintenance related tasks.

- Accessible, readily: Readily accessible, easily accessible, easy access and similar terms mean capable of being reached quickly and without climbing over or removing obstructions, mounting upon a chair, or using a movable ladder, and in any case not more than 2.0 m above the ground, floor or platform.

- Attendance: Attendance, provide attendance and similar expressions mean give assistance for examination and testing.

- Contract administrator: Has the same meaning as architect or superintendent and is the person appointed by the owner or principal under the contract.

- Contractor: Has the same meaning as builder and is the person or organization bound to carry out and complete the work under the contract.

- Default: Specified value, product or installation method which is to be provided unless otherwise documented.

- Design life: The period of time for which it is assumed, in the design, that an asset will be able to perform its intended purpose with only anticipated maintenance but no major repair or replacement being necessary.

- Documented: Documented, as documented and similar terms mean contained in the contract documents.

- Economic life: The period of time from the acquisition of an asset to the time when the asset, while still physically capable of fulfilling its function and with only anticipated maintenance, ceases to be the lowest cost alternative for satisfying that function.

- Electricity distributor: Any person or organization that provides electricity from an electricity distribution system to one or more electrical installations. Includes distributor, supply authority,

network operator, local network service provider, electricity retailer or electricity entity, as may be appropriate in the relevant jurisdiction.

- Fire Hazard Properties: To BCA A2.4.

- Geotechnical site investigation: The process of evaluating the geotechnical characteristics of the site in the context of existing or proposed construction.

- Give notice: Give notice, submit, advise, inform and similar expressions mean give notice (submit, advise, inform) in writing to the contract administrator.

- High level interface: Systems transfer information in a digital format using an open system interface.

- Hot-dip galvanized: Zinc coated to AS/NZS 4680 after fabrication with coating thickness and mass to AS/NZS 4680 Table 1.

- Ingress protection: IP, IP code, IP rating and similar expression have the same meaning as IP Code in AS 60529.

- Joints:

. Construction joint: A joint with continuous reinforcement provided to suit construction sequence.

. Contraction joint: An opening control joint with a bond breaking coating separating the joint surfaces to allow independent and controlled contraction of different parts or components, induced by shrinkage, temperature changes or other causes. It may include unbound dowels to assist vertical deflection control.

. Control joint: An unreinforced joint between or within discrete elements of construction which allows for relative movement of the elements.

. Expansion joint: A closing control joint with the joint surfaces separated by a compressible filler to allow axial movement due to thermal expansion or contraction with changes in temperature or creep. It may include unbound dowels to assist vertical deflection control.

. Sealant joint: A joint filled with a flexible synthetic compound which adheres to surfaces within the joint to prevent the passage of dust, moisture and gases.

. Structural control joint: A control joint (contraction, expansion and isolation) in structural elements when used with applied material and finishes.

. Substrate joint: A joint in the substrate which includes construction joints and joints between different materials.

. Weakened plane joint: A contraction joint created by forming a groove, extending at least one quarter the depth of the section, either by using a grooving tool, by sawing, or by inserting a premoulded strip.

- Local (government) authority: A body established for the purposes of local government by or under a law applying in a state or territory.

- Low level interface: Systems transfer information via terminals and voltage free contacts.

- Manufacturer’s recommendations: Recommendations, instructions, requirements, specifications (and similar expressions) provided in written or other form by the manufacturer and/or supplier relating to the suitability, use, installation, storage and/or handling of a product.

- Metallic-coated: Steel coated with zinc or aluminium-zinc alloy as follows:

. Metallic-coated steel sheet: To AS 1397. Metal thicknesses specified are based metal thicknesses.

. Ferrous open sections zinc coated an in-line process: To AS/NZS 4791.

. Ferrous hollow sections zinc coated by a continuous or specialized process: To AS/NZS 4792.

- Network utility operator: The entity undertaking the piped distribution of drinking water or natural gas for supply or is the operator of a sewerage system or external stormwater drainage system.

- Obtain: Obtain, seek and similar expressions mean obtain (seek) in writing from the contract administrator.

- Pipe: Includes pipe and tube.

- Practical completion or defects free completion: The requirements for these stages of completion are defined in the relevant building contract for the project.

- Principal: Principal has the same meaning as owner, client and proprietor and is the party to whom the contractor is legally bound to construct the works.

- Professional engineer: As defined by the BCA.

- Proprietary: Identifiable by naming the manufacturer, supplier, installer, trade name, brand name, catalogue or reference number.

- Prototype: A full size mock-up of components, systems or elements to demonstrate or test construction methods, junctions and finishes, and to define the level of quality.

- Provide: Provide and similar expressions mean supply and install and include development of the design beyond that documented.

- Record drawings: Record drawings has the same meaning as as-installed drawings, as-built drawings and work-as-executed drawings.

- Referenced documents: Standards and other documents whose requirements are included in this specification by reference.

- Registered testing authority:

. An organization registered by the National Association of Testing Authorities (NATA) to test in the relevant field; or

. An organization outside of Australia registered by an authority recognized by NATA through a mutual recognition agreement; or

. An organization recognized as being a Registered Testing Authority under legislation at the time the test was undertaken.

- Required: Required by the contract documents, the local council or statutory authorities.

- Rock Breaking: Defined as solid rock which requires a 20 tonne excavator to remove the rock using a rock breaking implement.

- If required: A conditional specification term for work which may be shown in the documents or is a legislative requirement.

- Sample: A physical example that illustrates workmanship, materials or equipment, and establishes standards by which the work will be judged. It includes samples, prototypes and sample panels.

- Statutory authority: A public sector entity created by legislation, that is, a specific law of the Commonwealth, State or Territory.

- Supply: Supply, furnish and similar expressions mean supply only.

- Tests – completion: Tests carried out on completed installations or systems and fully resolved before the date for practical completion, to demonstrate that the installation or system, including components, controls and equipment, operates correctly, safely and efficiently, and meets performance and other requirements. The superintendent may direct that completion tests be carried out after the date for practical completion.

- Tests – pre-completion: Tests carried out before completion tests, including:

. Production: Tests carried out on a purchased item, before delivery to the site.

. Progressive: Tests carried out during installation to demonstrate performance in conformance with this specification.

. Site: Tests carried out on site.

. Type: Tests carried out on an item identical with a production item, before delivery to the site.

- Tolerance: The permitted difference between the upper limit and the lower limit of dimension, value or quantity.

- Verification: Provision of evidence or proof that a performance requirement has been met or a default exists.

  - CONTRACT DOCUMENTS Services diagrammatic layouts

General: Layouts of service lines, plant and equipment shown on the drawings are diagrammatic only, except where figured dimensions are provided or calculable.

Before commencing work:

- Obtain measurements and other necessary information.

- Coordinate the design and installation in conjunction with all trades. Levels

General: Spot levels take precedence over contour lines and ground profile lines.

Drawings and manuals for existing services

- It will be the contractors responsibility to locate the existing services, advise the super intendent of the service and appropriately relocated as necessary to avoid impeding on the progress of the works. Contractor is to mark on the final as-built drawings the location of any relocated services.

Warranty: No warranty is given as to the completeness or accuracy of drawings and/or manuals of existing services.

  - SUBMISSIONS Requirement

General: Submit the following, as documented:

- Authority approvals: Notes of meetings with authorities whose requirements apply to the work and evidence that notices, fees and permits have been sought and paid, that authority connections are complete and that statutory approvals by the authorities whose requirements apply to the work have been received.

- Building penetrations: Details of the methods to maintain the required structural, fire and other properties to EXECUTION, BUILDING PENETRATIONS.

- Certification: Certification of conformance to documented requirements, including certification that the plant and equipment submitted meets all requirements of the contract documents and that each installation is operating correctly.

- Execution details: Execution programs, schedules and details of proposed methods and equipment. For building services include the following:

. Fixing of services: Typical details of locations, types and methods of fixing services to the building structure.

- Marking and labelling: Samples and schedules of proposed marking and labels to EXECUTION, MARKING AND LABELLING.

- Operation and maintenance manuals: For the whole of the work to EXECUTION, OPERATION AND MAINTENANCE MANUALS.

- Products: Products and materials data, including manufacturer’s technical specifications and drawing, evidence of conformance to product certification schemes, performance and rating tables and installation and maintenance recommendations.

- Prototypes: Prototypes of components, systems or elements.

- Records: As-built documents, photographs, system diagrams, schedules and logbooks to EXECUTION, RECORD DRAWINGS.

- Samples: Representative of proposed products and materials and including proposals to incorporate samples into the works, if any to EXECUTION, SAMPLES.

- Shop drawings: To EXECUTION, SHOP DRAWINGS.

- Substitutions: To PRODUCTS, GENERAL, Substitutions.

- Tests:

. Inspection and testing plan consistent with the construction program including details of test stages and procedures.

. Certificates for type tests.

. Fire hazard properties: Evidence of conformance of proposed proprietary products to documented requirements for fire hazard properties.

. Test reports for testing performed under the contract to EXECUTION, TESTS.

- Warranties: To EXECUTION, WARRANTIES.

Contractor review: Before submissions, review each submission item and check for coordination with other work of the contract and conformance to contract documents.

Submit to: Superintendent or Architect. Submission times

Default timing: Make submissions at least 5 working days before ordering products or starting

installation of the respective portion of the works.

Submission response times: Allow in the construction program for at least the following times:

- Shop drawings Certificate approved by Structural Consultants: 10 working days

- Certification of Samples and prototypes: 10 working days

- Manufacturers’ or suppliers’ recommendations: 10 working days

- Product data: 5 working days

- Product/design substitution or modification: 10 working days

Proposed products schedules: If major products are not specified as proprietary items, submit a schedule of those proposed for use within 3 weeks of site possession.

Identification

Requirement: Identify the project, contractor, subcontractor or supplier, manufacturer, applicable product, model number and options, as appropriate and include relevant contract document references. Include service connection requirements and product certification.

Non-conformance: Identify proposals that do not conform with project requirements, and characteristics which may be detrimental to successful performance of the completed work.

Errors

Errors: If a submission contains errors, make a new or amended submission as appropriate, indicating changes made since the previous submission.

Electronic submissions

Electronic copies file format: PDF, DWG and MS Office (Word, Excel etc.) Transmission medium: email

Hard copy submissions

Hard copy quantity: 2 bound copies Standard contract drawing size: A3

  - INSPECTION

Notice

Concealment: If notice of inspection is required for parts of the works that are to be concealed, advise when the inspection can be made before concealment.

Tests: Give notice of the time and place of documented tests. Minimum notice: As documented in the Notices schedule.

Light levels

Requirements: To AS/NZS 1680.2.4. Attendance

General: Provide attendance for documented inspections and tests.

- PRODUCTS

  - GENERAL

Manufacturers’ or suppliers’ recommendations

General: Provide and select, if no selection is given, transport, deliver, store, handle, protect, finish, adjust and prepare for use the manufactured items in conformance with the recommendations of the manufacturer or supplier.

Proprietary items/systems/assemblies: Assemble, install or fix to substrate in conformance with the recommendations of the manufacturer or supplier.

Project modifications: Advise of activities that supplement, or are contrary to the recommendations of the manufacturers or supplier.

Sealed containers

General: If materials or products are supplied by the manufacturer in closed or sealed containers or packages, bring the materials or products to point of use in the original containers or packages.

Prohibited materials

General: Do not provide the following:

- Materials, exceeding the limits of those listed, in the Safe Work Australia Hazardous Substances Information System (HSIS).

- All materials which contain some form of Asbestos.

- Materials that use chlorofluorocarbon (CFC) or hydro chlorofluorocarbon (HCFC) in the manufacturing process.

Substitutions

Identified proprietary items: Identification of a proprietary item does not necessarily imply exclusive preference for the identified item, but indicates the necessary properties of the item.

Alternatives: If alternatives to the documented products, methods or systems are proposed, submit sufficient information to permit evaluation of the proposed alternatives, including the following:

- Evidence that the performance is equal to or greater than that specified.

- Evidence of conformity to a cited standard.

- Samples.

- Essential technical information, in English.

- Reasons for the proposed substitutions.

- Statement of the extent of revisions to the contract documents.

- Statement of the extent of revisions to the construction program.

- Statement of cost implications including costs outside the contract.

- Statement of consequent alterations to other parts of the works.

Availability: If the documented products or systems are unavailable within the time constraints of the construction program, submit evidence.

Criteria: If the substitution is for any reason other than unavailability, submit evidence that the substitution:

- Is of net enhanced value to the principal.

- Is consistent with the contract documents and is as effective as the identified item, detail or method.

  - MATERIALS AND COMPONENTS Consistency

General: For each material or product use the same manufacturer or source and provide consistent type, size, quality and appearance.

Corrosion resistance

General: Conform to the following atmospheric corrosivity category as defined in AS 4312 and the AS/NZS 2312 series.

Exterior atmospheric corrosivity category: C4 Interior atmospheric corrosivity category: C4 Galvanizing

Severe conditions: Galvanize mild steel components (including fasteners) to AS 1214 or AS/NZS 4680

as appropriate, if:

- Exposed to weather.

- Embedded in masonry.

- Exposed to or in air spaces behind the external leaf of masonry walls.

- In contact with chemically treated timber, other than copper chrome arsenate (CCA).

- EXECUTION

  - SAMPLES General

Incorporation of samples: Only incorporate samples in the works which have been endorsed for inclusion. Do not incorporate other samples.

Retention of samples: Keep endorsed samples in good condition on site, until the date of practical completion.

Unincorporated samples: Remove on completion.

  - SHOP DRAWINGS General

Documentation: Include dimensioned drawings showing details of the fabrication and installation of structural elements, building components, services and equipment, including relationship to building structure and other services, cable type and size, and marking details.

Diagrammatic layouts: Coordinate work shown diagrammatically in the contract documents, and prepare dimensioned set-out drawings.

Record drawings: Amend all documented shop drawings to include changes made during the progress of the work and up to the end of the defects liability period.

Services coordination: Coordinate with other building and service elements. Show adjusted positions on the shop drawings.

Space requirements: Check space and access for maintenance requirements of equipment and services indicated diagrammatically in the contract documents.

Submission medium: PDF and Hardcopy Drawing size: A3

  - OFF-SITE DISPOSAL

Removal of material

General: Dispose of building waste material off site to the requirements of the relevant authorities.

  - WALL CHASING Holes and chases

General: If holes and chases are required in masonry walls, make sure structural integrity of the wall is maintained. Do not chase walls nominated as fire-resistance or acoustic rated.

Parallel chases or recesses on opposite faces of a wall: Not closer than 600 mm to each other.

Chasing in blockwork: Only in core-filled hollow blocks or in solid blocks which are not designated as structural.

Concrete blockwork chasing table

| Block thickness (mm) | Maximum depth of chase (mm) |
| --- | --- |
| 190 | 35 |
| 140 | 25 |

  - FIXING General

Suitability: If equipment is not suitable for fixing to non-structural building elements, fix directly to structure and trim around penetrations in non-structural elements.

Fasteners

General: Use proprietary fasteners capable of transmitting the loads imposed, and sufficient for the rigidity of the assembly.

  - SERVICES CONNECTIONS Connections

General: Connect to network distributor services or service points. Excavate to locate and expose connection points. Reinstate the surfaces and facilities that have been disturbed.

Network distributors’ requirements

General: If the network distributor elects to perform or supply part of the works, make the necessary arrangements. Install equipment supplied, but not installed, by the authorities.

  - SERVICES INSTALLATION General

Fixing: If non-structural building elements are not suitable for fixing services to, fix directly to structure and trim around holes or penetrations in non-structural elements.

Installation: Install equipment and services plumb, fix securely and organise reticulated services neatly. Allow for movement in both structure and services.

Concealment: Unless otherwise documented, conceal all cables, ducts, trays and pipes except where installed in plant spaces, ceiling spaces and riser cupboards. If possible, do not locate on external walls.

Lifting: Provide heavy items of equipment with permanent fixtures for lifting as recommended by the manufacturer.

Suspended ground floors: Keep all parts of services under suspended ground floors at least 150 mm clear of the ground surface. Make sure services do not impede access.

Arrangement: Arrange services so that services running together are parallel with each other and with adjacent building elements.

Dissimilar metals

General: Join dissimilar metals with fittings of electrolytically compatible material. Temporary capping

Pipe ends: During construction protect open ends of pipe with metal or plastic covers or caps.

Piping

General: Install piping in straight lines at uniform grades without sags. Arrange to prevent air locks. Provide sufficient unions, flanges and isolating valves to allow removal of piping and fittings for maintenance or replacement of plant.

Spacing: Provide at least 25 mm clear between pipes and between pipes and building elements, additional to insulation.

Changes of direction: Provide long radius elbows or bends and sets where practicable, and swept branch connections. Provide elbows or short radius bends where pipes are led up or along walls and then through to fixtures. Do not provide mitred fittings.

Vibration: Arrange and support piping so that it remains free from vibration whilst permitting necessary movements. Minimise the number of joints.

Embedded pipes: Do not embed pipes that operate under pressure in concrete or surfacing material. Valve groupings: If possible, locate valves in groups.

Pressure testing precautions: Isolate items not rated for the test pressure. Restrain pipes and equipment to prevent movement during pressure testing.

Differential movement

General: If the geotechnical site investigation report predicts differential movements between buildings and the ground in which pipes or conduits are buried, provide control joints in the pipes or conduits, as follows:

- Arrangement: Arrange pipes and conduits to minimize the number of control joints.

- Magnitude: Accommodate the predicted movements.

  - BUILDING PENETRATIONS Penetrations

Requirement: Maintain the required structural, fire and other properties when penetrating or fixing to the following:

- Structural building elements including external walls, fire walls, fire doors and access panels, other tested and rated assemblies or elements, floor slabs and beams.

- Membrane elements including damp-proof courses, waterproofing membranes and roof coverings. If penetrating membranes, provide a waterproof seal between the membrane and the penetrating component.

Sealing

Fire-resisting building elements: Seal penetrations with a system conforming to AS 4072.1.

Non fire-resisting building elements: Seal penetrations around conduits and sleeves. Seal around cables within sleeves. If the building element is acoustically rated, maintain the rating.

Sleeves

General: If piping or conduit penetrates building elements, provide metal or PVC-U sleeves formed from pipe sections as follows:

- Movement: Arrange to permit normal pipe or conduit movement.

- Diameter (for non fire-resisting building elements): Sufficient to provide an annular space around the pipe or pipe insulation of at least 12 mm.

- Prime paint ferrous surfaces.

- Terminations:

. If cover plates are fitted: Flush with the finished building surface.

. In fire-resisting and acoustic rated building elements: 50 mm beyond finished building surface.

. In floors draining to floor wastes: 50 mm above finished floor.

. Elsewhere: 5 mm beyond finished building surface.

. Termite management: To AS 3660.1.

- Thickness:

. Metal: 1 mm or greater.

. PVC-U: 3 mm or greater.

Sleeves for cables: For penetrations of cables not enclosed in conduit through ground floor slabs, beams and external walls provide sleeves formed from PVC-U pipe sections.

  - SUPPORT AND STRUCTURE General

Requirement: Provide incidental supports and structures to suit the services.

  - PIPE SUPPORTS Support systems

General: Provide proprietary support systems of metallic-coated steel construction.

Vertical pipes: Provide anchors and guides to maintain long pipes in position, and supports to balance the mass of the pipe and its contents.

Saddles: Do not provide saddle type supports for pipes greater than DN 25.

Dissimilar metals: If pipe and support materials are dissimilar, provide industrial grade electrically non- conductive material securely bonded to the pipe to separate them. Provide fixings of electrolytically compatible material.

Uninsulated pipes: Clamp piping supports directly to pipes. Insulated pipes:

- Spacers: Provide spacers at least as thick as the insulation between piping supports and pipes. Extend either side of the support by at least 20 mm.

- Spacer material: Rigid insulation material of sufficient strength to support the piping and suitable for the temperature application.

Support spacing

Cold and heated water pipes: To AS/NZS 3500.1 Table 5.6.4. Provide additional brackets, clips or hangers to prevent pipe movement caused by water pressure effects.

Sanitary plumbing: To AS/NZS 3500.2 Table 10.2.1. Fuel gas: To AS/NZS 5601.1 Table 5.5.

| Nominal pipe size (DN) | Minimum hanger diameter for single hangers (mm) |
| --- | --- |
| 50 maximum | 9.5 |
| 65 to 90 | 12.7 |
| 100 to 125 | 15.8 |
| 150 to 200 | 19.0 |

Other pipes: To AS/NZS 3500.1 Table 5.6.4. Hanger size table

  - ACCESS FOR MAINTENANCE Clearances

Minimum clearances for access: Conform to the following:

- ≥ 2100 mm clear vertically above horizontal floors, ground and platforms.

- Preferably ≥ 750 m clear, but in no case less than 600 mm horizontally between equipment or between equipment and building features including walls.

- If tools are required to operate, adjust or remove equipment, provide sufficient space so that the tools can be used in their normal manner and without requiring the user to employ undue or awkward force.

- If equipment components are hinged or removable, allow the space recommended by the manufacturer.

- Within plant items: Conform to the preceding requirements, and in no case less than the clearances recommended in BS 8313.

Facilities for equipment removal and replacement

Requirement: Provide facilities to permit removal from the building and replacement of plant and equipment, including space large enough to accommodate it and any required lifting and/or transportation equipment. Arrange plant so that large and/or heavy items can be moved with the minimum of changes of direction.

Removal of components: Allow sufficient space for removal and replacement of equipment components including air filters, tubes of shell and tube heat exchangers, removable heat exchanger bundles, coils and fan shafts. Provide access panels or doors large enough to permit the safe removal and replacement of components within air handling units.

Facilities for access

Equipment behind hinged doors: Provide doors opening at least 150°.

Equipment behind removable panels: Provide panels with quick release fasteners or captive metal thread screws.

Removable panels: Provide handles to permit easy and safe removal and replacement. Insulated plant and services: If insulation must be removed to access plant and services provide

access for maintenance, arranged so it can be repeatedly removed and replaced without damage. Piping

Requirement: Conform to the following:

- Provide access and clearance at fittings which require maintenance, inspection or servicing, including control valves and joints intended to permit pipe removal.

- Arrange piping so that it does not interfere with the removal or servicing of associated equipment or valves or block access or ventilation openings.

- Preferably run piping, conduits, cable trays and ducts at high level and drop vertically to equipment. Electrical and controls

Electrical equipment: Provide clearances and access space to AS/NZS 3000.

Switchboards and electrical control equipment: Locate near the main entrance to plant space. Arrange plant so that, to the greatest extent possible, switchboards are visible from the plant being operated.

Control panels: Locate near and visible from the plant controlled.

  - MARKING AND LABELLING General

Requirement: Mark and label services and equipment for identification purposes as follows:

- Locations exposed to weather: Provide durable materials.

- Pipes, conduits and ducts: To AS 1345 throughout its length, including in concealed spaces.

- Cables: Label to indicate the origin and destination of the cable.

Consistency: Label and mark equipment using a consistent scheme across all services elements of the project.

Label samples and schedules

Submission timing: Before marking or labelling.

Schedule: For each item or type of item include the following:

- A description of the item or type of item for identification.

- The proposed text for marking or labelling.

- The proposed location of the marking and labelling. Electrical accessories

Circuit identification: Label isolating switches and outlets to identify circuit origin.

Operable devices

Requirement: Mark to identify the following:

- Controls.

- Indicators, gauges, meters.

- Isolating switches.

Equipment concealed in ceilings

Location: Provide a label on the ceiling, indicating the location of each concealed item requiring access for routine inspection, maintenance and/or operation. In tiled ceilings, locate the label on the ceiling grid closest to the item access point. In flush ceilings, locate adjacent to closest access panel. Items to be labelled include but are not limited to:

- Fan coil units and terminal equipment (e.g. VAV terminals).

- Fire and smoke dampers.

- Isolating valves not directly connected to items otherwise labelled.

- Motorised dampers.

- Wall mounted equipment in occupied areas: Provide labels on wall mounted items in occupied areas including the following:

. Services control switches.

. Temperature and humidity sensors.

Points lists

Automatic control points: Provide plasticized, fade-free points lists for each automatic control panel. Store in a pocket on the door of the panel. Lists to include terminal numbers, point addresses, short and long descriptors.

Pressure vessels

General: Mount manufacturer’s certificates in glazed frames on a wall next to the vessel. Valves and pumps

General: Label to associate pumps with their starters and valves. Screw fix labels to body or attach

label to valve handwheels with a key ring. Labels and notices

Materials: Select from the following:

- Cast metal.

- For indoor applications only, engraved two-colour laminated plastic.

- Proprietary pre-printed self-adhesive flexible plastic labels with machine printed black lettering.

- Stainless steel or brass minimum 1 mm thick with black filled engraved lettering. Emergency functions: To AS 1319.

Colours: Generally to AS 1345 as appropriate, otherwise black lettering on white background except as follows:

- Danger, warning labels: White lettering on red background.

- Main switch and caution labels: Red lettering on white background. Edges: If labels exceed 1.5 mm thickness, radius or bevel the edges.

Labelling text and marking: To correspond to terminology and identifying number of the respective item as shown on the record drawings and documents and in operating and maintenance manuals.

Lettering heights:

- Danger, warning and caution notices: Minimum 10 mm for main heading, minimum 5 mm for remainder.

- Equipment labels within cabinets: Minimum 3.5 mm.

- Equipment nameplates: Minimum 40mm.

- Identifying labels on outside of cabinets: Minimum 5 mm.

- Isolating switches: Minimum 5 mm.

- Switchboards, main assembly designation: Minimum 25 mm.

- Switchboards, outgoing functional units: Minimum 8 mm.

- Switchboards, sub assembly designations: Minimum 15 mm.

- Valves: Minimum 20 mm.

- Self-adhesive flexible plastic labels:

. Labels less than 2000 mm above floor: 3 mm on 6 mm wide tape.

. Labels minimum 2000 mm above floor: 8 mm on 12 mm wide tape.

. Other locations: Minimum 3 mm.

Label locations: Locate labels so that they are easily seen and are either attached to, below or next to the item being marked.

Fixing: Fix labels securely using screws, rivets, proprietary self-adhesive labels or double-sided adhesive tape and as follows:

- If labels are mounted in extruded aluminium sections, use rivets or countersunk screws to fix the extrusions.

- Use aluminium or monel rivets for aluminium labels. Vapour barriers: Do not penetrate vapour barriers.

  - WARRANTIES

General

Requirement: If a warranty is documented, name the principal as warrantee. Register with manufacturers as necessary. Retain copies delivered with components and equipment.

Warranty period: Start warranty periods at acceptance of installation.

Approval of installer: If installation is not by manufacturer, and product warranty is conditional on the manufacturer’s approval of the installer, submit the manufacturer’s written approval of the installing firm.

  - RECORD DRAWINGS General

Requirement: Show the following:

- Installed locations of building elements, services, plant and equipment.

- Off-the-grid dimensions and depth if applicable.

- Any provisions for the future. Recording, format and submission

Progress recording: Keep one set of drawings on site at all times, expressly for the purpose of marking

changes made during the progress of the works.

Drawing layout: Use the same borders and title block as the contract drawings. Quantity and format: Conform to SUBMISSIONS.

Endorsement: Sign and date all record drawings.

Accuracy: If errors in, or omissions from, the record drawings are found, amend the drawings and re- issue in the quantity and format documented for SUBMISSIONS.

Date for submission: Not later than 2 weeks after the date for practical completion. Services record drawings

General: To General and Recording, format and submission and the following:

- Extensions and/or changes to existing: If a drawing shows extensions and/or alterations to existing installations, include sufficient of the existing installation to make the drawing comprehensible without reference to drawings of the original installation.

- Detention: If on-site detention tanks or pondage are provided, include the volume required on the drawing and the permitted flow rate to the connected system.

- Domestic cold water or fire mains: Show the pressure available at the initial connection point and the pressure available at the most disadvantaged location on each major section of the works.

- Stormwater: If storm water pipes are shown, include the pipe size and pipe grade together with the maximum acceptableflow and the actual design flow.

Diagrams: Provide diagrammatic drawings of each system including the following:

- Controls.

- Piping including all valves and valve identification tags.

- Principal items of equipment.

- Single line wiring diagrams.

- Acoustic and thermal insulation.

- Access provisions and space allowances.

- Fixings.

- Fixtures.

- Switchgear and control gear assembly circuit schedules including electrical service characteristics, controls and communications.

- Charts of valve tag numbers, with location and function of each valve, keyed to flow and control diagrams.

CAD base drawings: DWG and/or DXF format

  - OPERATION AND MAINTENANCE MANUALS General

Authors and compilers: Personnel experienced in the maintenance and operation of equipment and systems installed, and with editorial ability.

Referenced documents: If referenced documents or technical work sections require that manuals be submitted, include corresponding material in the operation and maintenance manuals.

Subdivision: By installation or system, depending on project size. Contents

Requirement: Include the following:

- Table of contents: For each volume. Title to match cover.

- Directory: Names, addresses, and telephone and facsimile numbers of principal consultant, subconsultants, contractor, subcontractors and names of responsible parties.

- Record drawings: Complete set of record drawings, full size.

- Drawings and technical data: As necessary for the efficient operation and maintenance of the installation. Include:

. Switchgear and control gear assembly circuit schedules including electrical service characteristics, controls and communications.

. Charts of valve tag numbers, with location and function of each valve, keyed to flow and control diagrams.

- Installation description: General description of the installation.

- Systems descriptions and performance: Technical description of the systems installed and mode of operation, presented in a clear and concise format readily understandable by the principal’s staff. Identify function, normal operating characteristics, and limiting conditions.

- Systems performance: Technical description of the mode of operation of the systems installed.

- Baseline data: To AS 1851 and AS/NZS 1668.1.

- Documentation to AS 1851 including the schedule of essential functionality and performance requirements.

- Digital photographic records to Underground services.

- Equipment descriptions:

. Name, address, email address and telephone and facsimile numbers of the manufacturer and supplier of items of equipment installed, together with catalogue list numbers.

. Schedules (system by system) of equipment, stating locations, duties, performance figures and dates of manufacture. Provide a unique code number cross-referenced to the record and diagrammatic drawings and schedules, including spare parts schedule, for each item of equipment installed.

. Manufacturers’ technical literature for equipment installed, assembled specifically for the project, excluding irrelevant matter. Mark each product data sheet to clearly identify specific products and component parts used in the installation, and data applicable to the installation.

. Supplements to product data to illustrate relations of component parts. Include typed text as necessary.

- Certificates:

. Certificates from authorities.

Certificates from consultants.

. Copies of manufacturers’ warranties.

. Product certification.

. Test certificates for each service installation and all equipment.

. Test reports

. Test, balancing and commissioning reports.

. Control system testing and commissioning results.

- 7 day record of all trends at commissioning.

- Operation procedures:

. Manufacturers’ technical literature as appropriate.

. Safe starting up, running-in, operating and shutting down procedures for systems installed.

Include logical step-by-step sequence of instructions for each procedure.

. Control sequences and flow diagrams for systems installed.

. Legend for colour-codes services.

. Schedules of fixed and variable equipment settings established during commissioning and maintenance.

. Procedures for seasonal changeovers.

. If the installation includes cooling towers, a water efficiency management plan.

- Maintenance procedures:

. Detailed recommendations for periodic maintenance and procedures, including schedule of maintenance work including frequency and manufacturers’ recommended tests.

. Manufacturer’s technical literature as appropriate. Register with manufacturer as necessary.

Retain copies delivered with equipment.

. Safe trouble-shooting, disassembly, repair and reassembly, cleaning, alignment and adjustment, balancing and checking procedures. Provide logical step-by-step sequence of instructions for each procedure.

. Schedule of spares recommended to be held on site, being those items subject to wear or deterioration and which may involve the principal in extended deliveries when replacements are required. Include complete nomenclature and model numbers, and local sources of supply.

. Schedule of normal consumable items, local sources of supply ,and expected replacement intervals up to a running time of 40 000 hours. Include lubrication schedules for equipment.

. Schedules for recording recommissioning data so that changes in the system over time can be identified.

. Instructions for use of tools and testing equipment.

. Emergency procedures, including telephone numbers for emergency services, and procedures for fault finding.

. Safety data sheets (SDS).

. Instructions and schedules conforming to AS 1851, AS/NZS 3666.2, AS/NZS 3666.3 and AS/NZS 3666.4.

- Maintenance records:

. Prototype service records conforming to AS 1851 prepared to include project specific details.

. Prototype periodic maintenance records and report to AS/NZS 3666.2, AS/NZS 3666.3 and AS/NZS 3666.4 as appropriate, prepared to include project specific details.

. For hard copies: In binders which match the manuals, loose leaf log book pages designed for recording completion activities including operational and maintenance procedures, materials used, test results, comments for future maintenance actions and notes covering the condition of the installation. Include completed log book pages recording the operational and maintenance activities performed up to the time of practical completion.

. Number of pages: The greater of 100 pages or enough pages for the maintenance period and a further 12 months.

Format – electronic copies

Scope: Provide the same material as documented for hardcopy in electronic format. Quantity and format: Conform to Submissions – electronic submissions.

Printing: Except for drawings required in the RECORD DRAWINGS clause provide material that can be legibly printed on A4 size paper.

Format – hard copy

General: A4 size loose leaf, in commercial quality, 4 ring binders with hard covers, each indexed, divided and titled. Include the following features:

- Cover: Identify each binder with typed or printed title OPERATION AND MAINTENANCE MANUAL, to spine. Identify title of project, volume number, volume subject matter, and date of issue.

- Dividers: Durable divider for each separate element, with typed description of system and major equipment components. Clearly print short titles under laminated plastic tabs.

- Drawings: Fold drawings to A4 size with title visible, insert in plastic sleeves (one per drawing) and accommodate them in the binders.

- Pagination: Number pages.

- Ring size: 50 mm maximum, with compressor bars.

- Text: Manufacturers’ printed data, including associated diagrams, or typewritten, single-sided on bond paper, in clear concise English.

Number of copies: 3. Date for submission

Draft submission: The earlier of the following:

- 2 weeks before the date for practical completion.

- Commencement of training on services equipment.

Final submission: Within 2 weeks after practical completion.

  - PERIODIC MAINTENANCE OF SERVICES Maintenance records

General: Record in binders provided with the Operation and maintenance manuals.

Referenced documents: If referenced documents or technical work sections require that log books or records be submitted, include this material in the maintenance records.

Certificates: Include test and approval certificates.

Service visits: Record comments on the functioning of the systems, work carried out, items requiring corrective action, adjustments made and name of service operator. On completion of the visit, obtain the signature of the principal's designated representative on the record of the work undertaken.

Site control

General: Report to the principal’s designated representative on arriving at and before leaving the site.

  - POST-CONSTRUCTION MANDATORY INSPECTIONS AND MAINTENANCE General

Requirement: For the duration of the defects liability period, provide inspections and maintenance of safety measures required by the following:

- AS 1851.

- Other statutory requirements applicable to the work. Records: Provide mandatory records.

Certification: Certify that mandatory inspections and maintenance have been carried out and that the respective items conform to statutory requirements.

Annual inspection: Perform an annual inspection and maintenance immediately before the end of the defects liability period.

- SELECTIONS

Warranty schedule

| Warranty | Form | Period |
| --- | --- | --- |
| Electrical Services | Supplier standard warrantee for material, equipment and installation | 12 months from the date of Practical completion |

| Warranty | Form | Period |
| --- | --- | --- |
| Hydraulics Services | Supplier standard warrantee for material, equipment and installation | 12 months from the date of Practical completion |
| Hydrant Services | Supplier standard warrantee for material, equipment and installation | 12 months from the date of Practical completion |
| Mechanical services | Supplier standard warrantee for material, equipment and installation | 12 months from the date of Practical completion |
| Roof Safety Access | Installers standard warrantee for material, equipment and installation | Installer maximum duration |
| Roofing | Manufacturers warrantee for material and installation | 10 years |
| Cladding | Manufacturers warrantee for material and installation | 5 years |
| Doors and Door hardware | Manufacturers warrantee for material and installation | 5 years |
| External Screens (Including frames and Crime safe screen and frames) | Manufacturers warrantee for material and installation | Manufacturers maximum Duration |
| Painting | Manufacturers warrantee for material and installation | Manufacturers maximum Duration |

  - MAINTENANCE

Maintenance requirements schedule

| Provision | Maintenance period (months) | Requirement |
| --- | --- | --- |
| Mechanical Services | 12 months | Every 3 months in accordance with the requirements of the services specification |
| Electrical Services | 12 months | In accordance with the requirements of the services specification |
| Hydraulic Services | 12 months | In accordance with the requirements of the services specification |

Not Used

Not Used

.

l

.

.

.

.

.

.

| : |  |  |  |
| --- | --- | --- | --- |
| Class | Requirement |  | Ownership |
| Recovered items for re-use in the works | Recover without damage items identified in the Recovered items for re-use in the works schedule |  | Principal/proprietor |
| Recovered items for delivery to the principal | Recover without damage items identified in the Recovered items for delivery to the principal schedule |  | Principal/proprietor |
| Demolished material for recycling in the works | Stockpile material identified in the Demolished material for recycling in the works schedule |  | Contractor |
| Demolished material for recycling off site | Demolish and deliver for recycling material identified in the Demolished material for recycling off-site schedule |  | Contractor |
| Dismantle for relocation as part of the works | Dismantle without damage and store items identified in the Dismantle for relocation schedule |  | Principal/proprietor |
| Demolished for removal | Remove from the site demolished materials identified in the Demolish for removal schedule. Do not burn or bury on site Transit: Prevent spillage of demolished materials in transit |  | Contractor |

.

.

.

.

Individual protection: Protect the following items in their existing positions:

- Power generation equipment and associated cabling and controls

- All other existing equipment associated with the normal function of the building.

.

Sequence of demolition: Contractor is to provide a Safety management plan and safe work statements as outlined within the Preliminary Section of the specification.

General: Do not use explosives.

l

.

I

0221 SITE PREPARATION

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide site preparation, as documented. Incidental works

Generally: Undertake the following:

- Reinstatement: Reinstate undeveloped ground surfaces to the condition existing at the commencement of the contract.

- Minor trimming: As required to complete the works, as documented.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - INTERPRETATION Definitions

General: For the purposes of this work section the following definitions apply:

- Authorities: Any authority or agency covering statutory requirements relating to the project, including clearances for work in that particular area.

- Clearances: A formal certificate, approval or condition issued by an authority to allow work to be carried out in a particular area.

- Network utility operator: The entity undertaking the piped distribution of drinking water or natural gas for supply or is the operator of a sewerage system or external stormwater drainage system.

  - SUBMISSIONS Execution details

Requirement: Submit details of methods and equipment proposed for the following:

- Clearing and grubbing.

- Tree removal and transplanting.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Enclosures around trees to be retained.

- Trees to be removed.

- EXECUTION

  - COMMUNITY LIAISON Notification

General: Notify residents about construction activities which will affect access to, or disrupt the use of, their properties.

Notice: Minimum 5 working days, unless the work is of an urgent nature with safety implications. Notification content:

- The nature of the work.

- The reason for it being undertaken.

- The expected duration.

- Changes to traffic arrangements and property access.

- The 24-hour contact number of the representative responsible.

  - EXISTING SERVICES General

Requirement: NA

Utility services: Contact MNRE and SFESA to identify location of underground utility services pipes and cables.

Excavation: Do not machine excavate within 1 m of existing underground services.

Existing service lines: If required, divert services detected during excavation to new routes, clear of the building, and reconnect to the network utility operator’s requirements.

  - SITE CLEARING Extent

Contractor’s site areas: If not included within the areas documented above, clear generally only to the

extent necessary for the performance of the works. Clearing and grubbing

Clearing: Remove everything on or above the site surface, including rubbish, scrap, grass, vegetable

matter and organic debris, scrub, trees, timber, stumps, boulders and rubble.

Grubbing: Grub out stumps and roots over 75 mm diameter to a minimum depth of 500 mm below subgrade under buildings, embankments or paving, or 300 mm below finished surface in unpaved areas. Backfill holes remaining after grubbing with sand material to prevent ponding of water. Compact the material to the relative density of the existing adjacent ground material.

Redundant/decommissioned works: Remove works, including slabs, foundations, pavings, drains and access chambers covers found on the surface.

Rock Breaking Not Applicable .

Rock breaking is noted as the removal of solid rock through the use of a 20 tonne excavator with a rock breaking implement.

Batters

Temporary protection: Where change in level between crest and toe is more than 1.5 m, protect from erosion with geofabric, a hessian and tar or heavy duty black polythene sheet waterproof cover. Seal joints and securely fix down at crest and toe.

Surplus material

Topsoil and excavated material: Continually remove unwanted stripped soil and other material from the site as the work proceeds, including any material dropped on footpaths or roadways.

  - STORMWATER AND SEDIMENT CONTROL General

Ensure Erosion and sediment control measures are put in place in accordance with MNRE policies and regulation

Waterways and drains

Waterways: Temporarily divert, as necessary, ditches, field drains and other waterways affected by excavation and reinstate on completion.

Stormwater drains: Divert drains detected during excavation to new routes, clear of the building, and reconnect to the network utility operator’s requirements.

Environmental Management Plan will be issued during tender for the contractor to implement during construction.

  - EXISTING WORKS TO BE RETAINED Marking

Requirement: Mark out works with 1 m high 50 x 50 mm timber stakes with yellow plastic tapes attached to prevent accidental damage.

  - COMPLETION

Temporary works Remove at completion:

Clean up

Progressive cleaning: Keep the work included in the contract clean and tidy as it proceeds and regularly remove from the site waste and surplus material arising from execution of the work, including any work performed during the defects liability period or the plant establishment period.

Removal of plant: Within 10 working days of the date of practical completion, remove temporary works, construction plant, buildings, workshops and equipment which does not form part of the works, except what is required for work during the defects liability period or the plant establishment period.

Remove these on completion.

Environmental Management Plan will be issued during tender for the contractor to implement during construction.

Vermin management

Requirement: Employ an approved firm of pest exterminators and provide a certificate from the firm stating that the completed works is free of vermin.

.

.

.

.

Chemical: Bifenthrin Aqua and Spectrum 200SC Imidacloprid or Equally approved.

.

s

.

s

Minimum period: 12months

Form of warranty: Written and submitted as part of O&M Manuals

.

.

•

.

.

0181 ADHESIVES, SEALANTS AND FASTENERS

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide adhesives, sealants and fasteners, as documented. Performance

Requirements: Conform to the following:

- Fitness for purpose: Capable of transmitting imposed loads, sufficient to maintain the rigidity of the assembly, or integrity of the joint.

- Finished surface: That will not cause discolouration.

- Compatibility: Compatible with the products to which they are applied.

- Sealant replacement: Capable of safe removal without compromising the application of the replacement sealant for future refurbishment.

- Movement: If an adhered or sealed joint is subject to movement, select a system certified to accommodate the projected movement under the conditions of service.

- Fasteners: Accredited for the particular use, capable of transmitting imposed loads and maintaining the rigidity of the assembly.

  - PRECEDENCE General

Order of precedence:

- The requirements of other worksections of the specification override conflicting requirements of this worksection.

- The requirements of the worksections override conflicting requirements of their referenced documents. The requirements of the referenced documents are minimum requirements.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - SUBMISSIONS Samples

Visible joint sealants: Submit colour samples. Products and materials

Sealants: Submit technical data sheets.

Tests

Compatibility testing: Submit adhesion and compatibility testing data demonstrating that adhesive, sealant or fastener is compatible with materials to be fixed and is suitable for the project conditions.

Warranties

Manufacturer’s warranty: Submit the manufacturer’s published product warranties.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of prepared joints and penetrations for each sealant.

- PRODUCTS

  - ADHESIVES Standards

Mastic adhesive: To AS 2329.

Polymer emulsion adhesive for timber: To AS 2754.2, not inferior to Type 3.

  - SEALANTS Standards

General: To ISO 11600.

Lightweight building element joints

Joints subject to rapid changes of movement: Provide sealants that accommodate the movement of the contact materials.

  - FASTENERS General

Masonry anchors: Proprietary expansion or bonded type anchors conforming to SELECTIONS,

ANCHORS.

Plain washers: To AS 1237.1.

- Provide washers to the heads and nuts of bolts, and the nuts of coach bolts. Plugs: Proprietary purpose-made plastic.

Powder-actuated fasteners: To AS/NZS 1873.4. Stainless steel fasteners: To ASTM A240/A240M. Steel nails: To AS 2334.

- Length: At least 2.5 times the thickness of the member being secured, and at least 4 times the thickness if the member is plywood or building board less than 10 mm thick.

Unified hexagon bolts, screws and nuts: To AS/NZS 2465. Fasteners in CCA treated timber: Epoxy coated or stainless steel. Bolts

Coach bolts: To AS/NZS 1390.

Hexagon bolts Grades A and B: To AS 1110.1. Hexagon bolts Grade C: To AS 1111.1.

Corrosion resistance

Atmospheric corrosivity category: To the 0171 General requirements worksection.

Steel products: Conform to the Corrosion resistance table or provide proprietary products with metallic and/or organic coatings of equivalent corrosion-resistance.

Corrosion resistance table

| Atmospheric corrosivity category to AS 4312 | Threaded fasteners and anchors |  | Powder actuated fasteners |
| --- | --- | --- | --- |
|  | Material | Minimum local metallic coating thickness (µm) | Material |
| C1 and C2 | Electroplated zinc or Hot- dip galvanized | 30 | Stainless steel 316 |
| C3 | Hot-dip galvanized | 50 | Stainless steel 316 |
| C4 and T | Stainless steel 316 | - | Stainless steel 316 |

Finishes Electroplating:

- Metric thread: To AS 1897.

- Imperial thread: To AS 4397. Galvanizing:

- Threaded fasteners: To AS 1214.

- Other fasteners: To AS/NZS 4680. Mild steel fasteners: Galvanize if:

- Embedded in masonry.

- In external timbers.

- In contact with chemically treated timber other than CCA treated timber. Epoxy coated: CCA treated timber.

Nuts

Hexagon chamfered thin nuts Grades A and B: To AS 1112.4. Hexagon nuts Grade C: To AS 1112.3.

Hexagon nuts Style 1 Grades A and B: To AS 1112.1. Hexagon nuts Style 2 Grades A and B: To AS 1112.2. Screws

Coach screws: To AS/NZS 1393.

Hexagon screws Grades A and B: To AS 1110.2. Hexagon screws Grade C: To AS 1111.2.

Hexagon socket screws: To AS 1420 and AS/NZS 1421. Machine screws: To AS/NZS 1427.

Self-drilling screws: To AS 3566.1. Self-tapping screws:

- Crossed recessed countersunk (flat – common head style): To AS/NZS 4407.

- Crossed recessed pan: To AS/NZS 4406.

- Crossed recessed raised countersunk (oval): To AS/NZS 4408.

- Hexagon: To AS/NZS 4402.

- Hexagon flange: To AS/NZS 4410.

- Hexagon washer: To AS/NZS 4409.

- Slotted countersunk (flat – common head style): To AS/NZS 4404.

- Slotted pan: To AS/NZS 4403.

- Slotted raised countersunk (oval – common head style): To AS/NZS 4405. Blind rivets

Description: Expanding end type with snap mandrel.

Type: Closed end for external application, open end for internal application. End material:

- Aluminium base alloy for metallic-coated or prepainted steel.

- Stainless steel for stainless steel sheet.

- Copper for copper sheet. Size:

- For sheet metal to sheet metal: 3mm.

- For sheet metal to supports, brackets and rolled steel angles: 4.8 mm.

- EXECUTION

  - ADHESIVES General

Requirement: Install to the manufacturer’s recommendations. Preparation

Substrates: Conform to the following:

- Remove any deposit or finish which may impair adhesion.

- If framed or discontinuous, provide support members in full lengths without splicing.

- If solid or continuous, remove excessive projections.

- If previously painted, remove cracked or flaking paint and lightly sand the surface.

  - JOINT SEALING General

Requirement: Install to the manufacturer’s recommendations. Joint preparation

Cleaning: Cut flush joint surface protrusions and rectify if required. Mechanically clean joint surfaces

free of any deposit or finish which may impair adhesion of the sealant. Immediately before sealant application, remove loose particles from the joint, using oil-free compressed air.

Bond breaking: Install bond breaking backing material.

Taping: Protect the surface on each side of the joint using 50 mm wide masking tape or equivalent means. On completion of sealant application, remove the tape and remove any stains or marks from adjacent surfaces.

Primer: Apply the recommended primer to the surfaces in contact with sealant materials. Sealant joint proportions

General weatherproofing joints (width:depth):

- 1:1 for joint widths less than 12 mm.

- 2:1 for joint widths greater than 12 mm. Sealant application

General: Apply the sealant to dry joint surfaces using a pneumatic applicator gun. Make sure the

sealant completely fills the joint to the required depth, provides good contact with the full depth of the sides of the joint and traps no air in the joint. Do not apply the sealant outside the recommended working time for the material or the primer.

Weather conditions

Two pack polyurethanes: Do not apply the sealant if ambient conditions are outside the following:

- Temperature: Less than 5°C or greater than 40°C.

- Humidity: To the manufacturer’s recommendations. Joint finish

General: Force the sealant into the joint and finish with a smooth, slightly concave surface using a tool

designed for the purpose.

Excess sealant: Remove from adjoining surfaces using cleaning material nominated by the sealant manufacturer.

Protection

General: Protect the joint from inclement weather during the setting or curing period of the material. Rectification

General: Cut out and remove damaged portion of joint sealant and reinstall so repaired area is

indistinguishable from undamaged portion.

  - TESTING Installed sealant tests

Sampling: For each sealant test, take 3 samples of installed and cured sealant, each at least 50 mm long, from completed joints.

Reinstatement: Repair-as-new the joints from which the samples were taken.

  - FASTENERS General

Requirement: Install to the manufacturer’s recommendations. Fastening to wood and steel

Timber substrates: To AS 1720.1 Section 4.

Self drilling screws: To AS 3566.1 for timber and steel substrates. Masonry anchors

Installation: To the manufacturer’s recommendations.

| Application | Product | Relevant worksections |
| --- | --- | --- |
| Metal flashings and rainwater goods | Sika Blackseal BT or similarly approved | 0421 Roofing – combined, 0423 Roofing – profiled sheet metal, 0424 Roofing – seamed sheet metal, 0425 Roofing – shingles and shakes, 0426 Roofing – slate, 0427 Roofing – tiles |
| Louvres and external doors and block walls | Sika Acrlyic or similarly approved | 0432 Curtain walls, 0451 Windows and glazed doors, 0453 Doors and access panels, 0463 Glass blockwork |

SELECTIONS

  - SEALING, POINTING AND BEDDING Application schedule

0183B METALS AND PREFINISHES

- GENERAL

  - RESPONSIBILITIES General

Requirements: Provide metal and prefinishes as documented. Performance

Requirement: Provide metals in sections of strength and stiffness suited to their required function,

finish and method of fabrication.

  - PRECEDENCE General

Order of precedence:

- The requirements of other work sections of the specification override conflicting requirements of this  work section.

- The requirements of this work section override conflicting requirements of its referenced documents.

- The requirements of the referenced documents are minimum requirements.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - SUBMISSIONS Samples

General: Submit samples of the following:

- Stainless steel: One sample of every documented surface finish.

- Anodising: One sample of every colour and finishing option.

- PRODUCTS

  - METALS Coated steel

Electrogalvanized (zinc) coating on ferrous hollow and open sections: To AS 4750. Metallic-coated: Steel coated with zinc or aluminium-zinc alloy as follows:

- Ferrous open sections by an in-line process: To AS/NZS 4791.

- Ferrous hollow sections by a continuous or specialised process: To AS/NZS 4792.

- Metallic-coated steel sheet: To AS 1397. Metal thicknesses specified are base metal thicknesses. Steel wire: To AS/NZS 4534.

- EXECUTION

  - GENERAL Metal separation

Incompatible sheet metals: Provide separation by one of the following:

- Apply an anti-corrosion low moisture transmission coating such as alkyd zinc phosphate primer or aluminium pigmented bituminouspaint to contact surfaces.

- Insert a concealed separation layer such as polyethylene film, adhesive tape, or bituminous felt. Incompatible fixings: Do not use.

Incompatible service pipes: Install lagging or grommets. Do not use absorbent, fibrous or paper products.

Brazing

General: Make sure brazed joints have sufficient lap to provide a mechanically sound joint.

Butt joints: Do not use butt jointing for joints subject to load. If butt joints are used, do not rely on the filler metal fillet only.

Filler metal: To AS/NZS 1167.1. Finishing

Visible joints: Finish visible joints made by welding, brazing or soldering using methods appropriate to

the class of work (including grinding or buffing) before further treatment such as painting, galvanizing or electroplating. Make sure self-finished metals are without surface colour variations after jointing.

Preparation

General: Before applying decorative or protective prefinishes to metal components, complete welding, cutting, drilling and other fabrication, and prepare the surface using a suitable method.

Standard: To AS 1627 series.

Priming steel surfaces: If site painting is documented to otherwise uncoated mild steel or similar surfaces, prime as follows:

- After fabrication and before delivery to the works.

- After installation, repair damaged priming and complete the coverage to unprimed surfaces. Welding

Aluminium: To AS 1665.

Stainless steel: To AS/NZS 1554.6. Steel: To AS/NZS 1554.1.

  - COMPLETION

Damage

Damaged prefinishes: Remove and replace items, including damage caused by unauthorised site cutting or drilling.

Repair

Metallic-coated sheet: If repair is required to metallic-coated sheet or electro galvanizing on inline galvanized steel products, clean the affected area and apply a two-pack organic primer to AS/NZS 3750.9.

0193 BUILDING ACCESS SAFETY SYSTEMS

- GENERAL

  - RESPONSIBILITIES General

Contractor shall allow to install a load rated ladder clip to the lower entry roof and at the low end of the main roof complete with roof safety anchors to allow safe access to the equipment listed below:

    - Roof Gutter

    - External Screens

The system must be maintained throughout the 12 month defects period. Roof Anchor Safety:

All anchor points to be used should be independently tested by accredited tested for common roof structures and profiles and meet or exceed the requirements of Australian Standards AS/NZS 1891.4:2000, AS/NZS 5532:2013, and AN/NZS 4488:1997.

    - Minimum of 22KN rated two person energy absorption anchor.

Placement of the anchors should be in accordance with AS/NZS 1891.4:2000 to ensure the user is safeguarded again excess fall and pendulum movement due to the fall.

The contractor shall provide a A3 Plan indicating the all safety points and swings to ensure the system is compliant prior to ordering.

Construction Safety Access:

Installation, maintenance and removal of temporary safety railing, scaffolding, access harness, lanyards, static lines throughout the construction is the responsibility of the contractor

0315 CONCRETE FINISHES

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide finishes to formed and unformed concrete surfaces, as documented. Performance

Requirement: Compatible with documented finishes.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - STANDARDS General

Formed surfaces: To AS 3610.1.

  - INTERPRETATION Definitions

General: For the purposes of this work section the following definition applies:

- Green concrete: Concrete which has set but not appreciably hardened.

  - TOLERANCES Formed surfaces

Quality of the surface finish: To AS 3610.1 Table 3.3.2. Unformed surfaces

Unformed surfaces flatness: To the Flatness tolerance class table, using a straightedge placed

| Class | Measurement | Maximum deviation (mm) |
| --- | --- | --- |
| A | 2 m straightedge | 4 |
| B | 3 m straightedge | 6 |
| C | 600 mm straightedge | 6 |

anywhere on the surface in any direction, for the documented class of finish. Flatness tolerance class table

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Evaluation of the off-form finishes.

- Evaluation of surface finish.

- PRODUCTS

  - MATERIALS

Surface hardeners, sealants and protectors

Supply: If documented, provide proprietary products to the manufacturer’s recommendations. Slip resistance treatment

Slip resistance classification: To AS 4586.

- EXECUTION

  - SURFACE MODIFIERS General

Application: Apply to clean surfaces to the manufacturer’s recommendations.

  - FORMED SURFACES General

Surface finish: Provide formed concrete finishes as documented in the Formed surface finishes schedule.

Damage: Do not damage concrete works through premature removal of formwork. Curing

General: If formwork is stripped before the minimum curing period for the concrete has elapsed,

continue curing the exposed faces as soon as the stripping is completed. Evaluation of formed surfaces

General: If evaluation of formed surface tolerance or colour is required, complete the evaluation before

surface treatment. Surface repairs

Method: If surface repairs are required, submit proposals.

Finishing methods

Details: If soffits of concrete elements or faces of concrete columns are to have a finish other than an off-form finish, provide finishes as documented.

Floated finishes:

- Grout floated finish: Remove the vertical face formwork while the concrete is green. Dampen the surface and spread a slurry, using hessian pads or sponge rubber floats. Remove surplus slurry and work until a uniform colour and texture are produced.

  - UNFORMED SURFACES General

Surface finish: As documented in the Unformed surface finishes schedule.

Finished levels: Strike off, screed and level slab surfaces to finished levels and to the flatness tolerance class documented.

Surface repairs

Method: If surface repairs are required, submit proposals. Finishing methods – primary finish

Machine float finish:

- After levelling, consolidate the surface using a machine float.

- Cut and fill and refloat immediately to a uniform, smooth, granular texture.

- Hand float in locations inaccessible to the machine float. Steel trowel finish: After machine floating finish, as follows:

- Use power or hand steel trowels to produce a smooth surface relatively free from defects.

- When the surface has hardened sufficiently, re-trowel to produce the final consolidated finish free of trowel marks and uniform in texture andappearance.

Wood float finish: After machine floating, use wood or plastic hand floats to produce the final consolidated finish free of float marks and uniform in texture and appearance.

Fine Broom finish: After machine floating and steel trowelling use a broom across the surface to produce a coarse even-textured transverse-scored surface.

Finishing methods – supplementary finish

Wood float finish: use wood or plastic hand floats to produce the final consolidated finish free of float marks and uniform in texture and appearance.

.

- SELECTIONS

Formed surface finishes schedule

| Location | Insitu beams and columns and external instiu Wall |
| --- | --- |
| Surface finish class to AS 3610.1 | Class 2 |
| Formwork lining type | Form Ply or equivalent |
| Surface finish type | Smooth |
| Evaluation | Visual from 1, away in accordance with AS3610 |

Unformed surface finishes schedule

| Location | External Steps, Ramp and Verandah Slab, footpaths |
| --- | --- |
| Flatness tolerance class | +/- 3mm over 3m |
| Primary finish | Fine Broom finish |
| Supplementary finish | 75mm wide smooth edge trowel to either side of the verandah joints and perimeter of ramps |
| Colour: | natural |
| Slip resistance treatment | Fine broom finish across the surface |
| Slip resistance classification | R11 |

0382 LIGHT TIMBER FRAMING

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide light timber floor, wall and roof framing, as documented.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

- 0181 Adhesives, sealants and fasteners.

- 0185 Timber products, finishes and treatment.

  - INTERPRETATION Definitions

General: For the purposes of this work section the definitions given in the AS 1684 series apply.

  - STANDARDS General

Framing: To AS 1684.2, AS 1684.3 or AS 1684.4, as appropriate. Design: To AS 1720.1.

  - TOLERANCES

Floors

| Property | Permitted deviation |
| --- | --- |
| Generally: Verticality in 2 m | 1:500 |
| Generally: Flatness1 in 2 m | 3 mm |
| Features2: Verticality in 2 m | 1:1000 |
| Features2: Horizontality in 2 m | 1:1000 |
| - Flatness: Measured under a straightedge laid in any direction on a surface. - Features: Conspicuous horizontal or vertical lines including external corners, parapets, reveals, heads, sills. |  |

Maximum deviation from a 3 m straightedge laid in any direction on the floor framing: 5 mm. Walls tolerances table

  - SUBMISSIONS Certification

Requirement: Submit certification by a professional engineer of the design, documentation and erected work to AS 1684 and AS 1720.1. Include the following:

- Reactions: Provide location and magnitude of reactions to be accommodated by the support structure.

- Floor, wall and roof frame member sizes: A schedule of proposed member sizes, certified as meeting stated project requirements for spans, spacings, loadings and deflections.

- Species and stress grade. Products and materials

Identification: Submit a supplier’s certificate (which may be included on an invoice or delivery docket)

verifying that the timber conforms to the documented requirements.

Inspection: Submit the inspection authority’s certificate verifying that the timber conforms to the documented requirements.

Moisture content: Submit records of moisture content.

CCA treated timber: If proposed to be used, provide details. Shop drawings

General: Submit shop drawings, to a scale that best describes the detail, certified by a professional

engineer stating that the design has been carried out to AS 1684 and AS 1720.1 requirements for the documented configurations and loadings. Include the following:

- Prefabricated roof trusses:

. Marking plans.

. Truss plan layout.

. Elevations, with the arrangement of members allowing for the accommodation of in-roof services and the size and section type of each member.

. Camber of all elements.

. The method of assembly, connection, lifting, holding down and bracing.

Subcontractors

Prefabricated items: Submit the name and contact details of proposed manufacturers, suppliers and installers.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Prefabricated units before installation.

- Fabricated items before priming or water-repellent treatment.

- Bolts after final tightening.

- Timber work after erection but before it is covered.

- PRODUCTS

  - GENERAL Storage and handling

General: Do not distort or damage timber or timber products.

Moisture content: Maintain the equilibrium moisture content of seasoned timber.

Protection from weather: Provide temporary protection for members until permanent covering is in place.

Marking

Branding: Permanently mark structural timber under the authority of a recognized product certification program applicable to the product. Locate the brand mark on faces or edges which will be concealed in the works. For timbers not covered by the branding provisions of Australian standards or regulations for which branding is required, include the following data:

- Stress grade.

- Method of grading.

- If seasoned, the word, SEASONED or DRY, or an abbreviation of seasoned, such as SEAS or S.

- The certification mark of the product certification program.

- The applicable standard.

Trusses: Permanently mark each truss to show:

- Project identification.

- Manufacturer.

- Tag or number.

- Location.

- Support points.

.

  - COMPONENTS Fasteners

General: Conform to the 0181 Adhesives, sealants and fasteners work section. Installation: Do not split or otherwise damage the timber.

Coating: Before placing bolts in contact with CCA treated timber, coat the shank of the bolt in a grease or bituminous coating.

Damp-proof course Material: To AS/NZS 2904.

Type: Therma Kraft Super course 500 DPC or equally approved.

- EXECUTION

  - WALL FRAMING Bracing

Bracing material: Treated Timber H3.1 min. Additional support

Requirement: Provide additional support in the form of noggings, trimmers and studs for fixing lining,

cladding, hardware, accessories, fixtures and fittings, as required. Spacing of noggings: Maximum 1350 mm centres.

Vermin barriers

Requirement: Provide vermin barriers as follows:

- Brick veneer barrier: Close nail 10 mm galvanized steel wire mesh to the underside of the bottom plate of external stud walls, extending across the cavity for building into brickwork.

Damp-proof course

Requirement: Provide damp-proof courses under the bottom plate of stud walls built off slabs or masonry dwarf walls, as follows:

- External walls (not masonry veneer): Turn up at least 75 mm on the inside and tack. Project 10 mm beyond the external slab edge or dwarf wall and turn down at 45°.

- Walls of bathrooms, shower rooms and laundries: Turn up at least 150 mm on the wet side and tack to studs.

Installation: Lay in long lengths. Lap full width at angles and intersections and at least 150 mm at joints.

Junctions: Preserve continuity at junctions of damp-proof courses, sarkings and waterproof membranes.

Flashings

Location: Provide flashings to external openings to prevent the entry of moisture. Form trays at the ends of sill flashings.

Masonry veneer construction: Extend flashing across cavities and build into brickwork. 3.2

  - CEILING FRAMING

Wall plates

Fixing: Fix timber wall plates to masonry, with straps, bolts or both. Fixing plates

Requirement: Provide 45 mm minimum thick timber fixing plates to transfer the design loads where

timber joists, rafters or purlins bear on or into steel members. Bolt to the steel member at maximum 500 mm centres and at maximum 100 mm from the end of the fixing plate.

Beam framing

Ridge straps: Butt ends of rafters together at ridge, and strap each pair together with 900 mm long steel strap passing over the ridge, triple nail to each rafter.

Water tank or heater in roof space: Provide a support platform to AS/NZS 3500.4 clause 5.5.

Additional support: Provide a frame member behind every joint in fibre cement sheeting or

lining Completion

Tightening

Requirement: Tighten bolts, screws and other fixings so that joints and anchorages are secure at the date of practical completion.

  - COMPLETION Tightening

Requirement: Tighten bolts, screws and other fixings so that joints and anchorages are secure at the date of practical completion.

0423P ROOF SHEETING

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide and install roof cladding made from COLORBOND® steel and associated work.

  - COMPANY CONTACTS

BlueScope technical contacts Website: www.steel.com.au/support

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - MANUFACTURER'S DOCUMENTS Technical manuals

Website: www.steel.com.au/library.

  - SUBMISSIONS

Operation and maintenance manuals

On completion: Submit a manual of recommendations from the roofing manufacturer or supplier for the maintenance of the roofing system including, frequency of inspection and recommended methods of access, inspection, cleaning, repair and replacement.

Products and materials

Type tests: As appropriate for the project, submit evidence of conformance to the following:

- Metal roofing: Roof sheeting and fastenings to AS 1562.1 Section 5 for resistance to concentrated load and to wind pressure.

Samples

Requirement: Submit samples of the following:

- Custom profiled flashings and cappings.

- Sheet metal finishes showing the range of variation available.

- Sealants.

- Trims and accessories with a colour finish. Warranties

Roofing materials: Submit the manufacturer’s published product warranties.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Roof supports.

- Parts of the roofing, sarking, vapour barrier, insulation and roof plumbing installation before covering up or concealing.

- PRODUCTS

  - GENERAL Product substitution

Other products: Conform to PRODUCTS, GENERAL, Substitutions in the 0171 General requirements worksection. Alternative equally approved products maybe presented as substitutes, however refer the generals section for substitution process.

Storage and handling

Storage: Store metal roofing materials away from uncured concrete and masonry, on a level base. Do not store materials in contact with other materials which may cause staining, denting or other surface damage.

Handling: Handle roofing materials as follows:

- Use gloves when handling precoated metal roofing material.

- Use soft soled shoes when fixing or working on roofs.

- Protect edges and surfaces from damage. Do not drag sheets across each other or over other materials.

Marking

Identification: Marked to show the following:

- Manufacturer’s identification.

- Product brand name.

- Product type.

- Quantity.

- Product reference code and batch number.

- Date of manufacture.

- Material composition and characteristics such as volatility, flash point, light fastness, colour and pattern.

  - COMPONENTS Fasteners

Finish: Prefinished exposed fasteners with an oven baked polymer coating to match the roofing material.

Fastenings to timber battens: Fastenings long enough to penetrate the thickness of the batten without piercing the underside.

Profiled  fillers

Type: Purpose-made closed cell polyethylene foam profiled to match the roofing profile. Location: Profiled fillers under flashings to the following:

- Ridges.

- Eaves.

- Lapped joints in roof sheeting. Safety mesh

Standard: To AS/NZS 4389.

  - SHEET METAL ROOFING Standards

Design, installation and materials: To AS 1562.1. Stainless steel: To ASTM A240/A240M.

Roofing  product

Product brand: BHP Colorbond

Profile: Trimdek or equally approved Product material type: Colour Bond ULTRA Thickness, Base Metal Thickness (BMT) (mm): 0.55

Colour: TBC

  - ROOF PLUMBING

General

Standard: To AS/NZS 3500.3.

Requirement: Provide the flashings, cappings, gutters, rainwater heads, outlets and downpipes required to complete the roof system.

Materials

Metal rainwater goods: To AS/NZS 2179.1.

Metal: 0.55 BMT Colourbond Ultra or equally an approved product. PVC-U rainwater goods and accessories: To AS/NZS 3500.3.

Flashings and cappings Standard: To AS/NZS 2904.

Product: 0.55 BMT Colourbond Ultra or equally an approved product. Material and colour: Match roof sheeting.

Rib notching: Match roof sheeting.

Flashing and capping types: 0.55 BMT Colourbond Ultra or equally an approved product. Ridge and barge cappings

Product: 0.55 BMT Colourbond Ultra or equally an approved product. Material and colour: Match roof sheeting.

Capping types: 0.55 BMT Colourbond Ultra notched to match roofing profile or equally an approved

product.

Eaves gutters

Product: 0.55 BMT Colourbond Ultra or equally an approved product. Type: Refer to contract drawings for size.

Material and colour: Match roof sheeting.

Matching fascia/barge: If the selected eaves gutter is a proprietary high front pattern forming part of a combined system of gutter, fascia and barge, provide the matching proprietary fascias and barge cappings to roof verges and edges.

Downpipes

Product: Nylex or Marley or equally an approved product. Material: uPVC

Colour: Painted– Dulux Paints Profile: Round

Size: Refer drawings

  - ROOF PLANT ACCESS Description

General: Refer to Building Access section.

- EXECUTION

  - INSTALLATION Protection

General: Keep the roofing and rainwater system free of debris and loose material during construction, and leave them clean and unobstructed on completion. Repair damage to the roofing and rainwater system.

Thermal movement

Requirement: Allow for thermal movement in the roof installation and the structure, including movement in joints and fastenings.

Pan type sheets

Removal: Install sheets so that individual sheets can be removed without damage.

Metal separation

Requirement: Prevent direct contact between incompatible metals, and between green hardwood or chemically treated timber and aluminium or coated steel, by one of the following methods:

- Applying an anti-corrosion, low moisture transmission coating to contact surfaces.

- Inserting a separation layer. Tolerances

Requirement: To AS 1562.1 clause 4.2.

  - BUILDING ELEMENTS Ridges and eaves

Sheet ends: Treat as follows:

- Project sheets 50 mm into gutters.

- Close off ribs at bottom of sheets using mechanical means or with purpose-made fillers or end caps.

- Turn pans of sheets up at tops and down into gutters by mechanical means.

- Pre-cut notched eaves flashing and birdproofing, if required.

- Close off ridges with purpose-made ridge fillers of closed cell polyethylene foam. Ridge and barge

Capping: Finish off along ridge and verge lines with purpose-made ridge capping or barge rolls.

End laps

General: If end laps are unavoidable, and the sheet profile is not suitable for interlocking or contact end laps, construct a stepped type lap.

  - ROOF PLUMBING

Jointing sheet metal rainwater goods

Butt joints: Make joints over a backing strip of the same material. Soldered joints: Do not solder aluminium or aluminium/zinc-coated steel.

Sealing: Seal fasteners and mechanically fastened joints. Fill the holes of blind rivets with silicone sealant.

Flashings

Installation: Flash roof junctions, upstands, abutments and projections through the roof. Preform to required shapes if possible. Notch, scribe, flute or dress down as necessary to follow the profile of adjacent surfaces. Mitre angles and lap joints 150 mm in running lengths. Provide matching expansion joints at 6 m maximum intervals.

Upstands: Flash projections above or through the roof with two part flashings, consisting of a base flashing and a cover flashing, with at least 100 mm vertical overlap. Provide for independent movement between the roof and the projection.

Large penetrations in low pitch roofs: Extend the base flashing over the roofing ribs to the ridge to prevent ponding behind the penetrating element.

Wall abutments: Where a roof abuts a wall, provide overflashing as follows:

- Raking in concrete: Turn 25 mm into joints or grooves, wedge at 200 mm centres with compatible material and point up.

Fixing to pipes: Solder or seal with neutral cured silicone rubber and secure with either of the following:

- Clamping ring.

- Proprietary flexible clamping shoe with attached metal surround flashing. Gutters

General:. Form stop ends, downpipe nozzles, bends and returns. Dress downpipe nozzles into outlets.

Provide overflows to prevent back-flooding. Gutter support: Provide support gutters.

Support: Colour bond Ultra 0.55 BMT Gutter at 450 crs, or equally an approved product. Downpipes

General: Prefabricate downpipes to the required section and shape where possible. Connect heads to

gutter outlets and, if applicable, connect feet to rainwater drains.

Access cover: Provide a removable watertight access cover at the foot of each downpipe stack. Downpipe support: Provide supports and fixings for downpipes.

  - TESTING

Site tests

Internal downpipes: Test each stack hydrostatically in stages. Run each test over two storeys high for two hours. Remedy defects and retest if necessary.

  - COMPLETION Cleaning

Remove: Excess debris, metal swarf, solder, sealants and unused materials.

Exposed metal surfaces: Clean surfaces of substances that interfere with uniform weathering or oxidisation.

Replace: Materials that have been damaged or deteriorated.

Roof plumbing: Clean out spoutings, gutters and rainwater pipes after completion of roof installation. Warranties

Requirement: Cover materials and workmanship in the terms of the warranty in the form of interlocking

warranties from the supplier and installer.

- Form: Against failure of materials and execution under normal environment and use conditions.

- Period: As offered by the supplier/manufacturer.

0451B WINDOWS AND GLAZED DOORS

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide windows and glazed doors, as documented. Performance

Product design: Provide windows with sashes capable of being opened to satisfy the documented

maintenance requirements.

  - CROSS REFERENCES

  - GENERAL

Requirement: Conform to the following:

- 0171 General requirements. Manufacturer’s documents

Manufacturer’s and supplier’s documents related to this work section are: Designer nominated

Glass and Aluminium systems and Breezeway or equally approval on supply on product information and confirmation of suitability.

  - STANDARDS General

Selection and installation: To AS 2047. Glazing

Glass type and thickness: To AS 1288, if no glass type or thickness is nominated.

Materials and installation: To AS 1288.

Quality requirements for cut-to-size and processed glass: To AS/NZS 4667. Terminology for work on glass: To AS/NZS 4668.

  - SUBMISSIONS

Certification

Conformance: Submit evidence that window and door assemblies conform to AS 2047.

Sealant compatibility: Submit statements from all parties to the installation certifying the compatibility of sealants and glazing systems to all substrates.

Ceramic-coated spandrel glass: Submit a report, from the manufacturer, certifying that the glass meets the Fallout Resistance Test requirements of ASTM C1048.

Opacified glass: Submit a report, from the manufacturer, certifying that the proposed method of opacifying the glass will not be detrimental to the glass or affect the glass product warranty.

Operation and maintenance manual

Window and door assemblies: Submit the window and glazed door manufacturer’s published instructions for operation, care and maintenance.

Hardware: Submit the manufacturer’s published recommendations for use, care and maintenance. Products and materials

Type tests: Submit results, as follows:

- Fire-resistance level.

- Weighted sound reduction index (Rw): To AS/NZSISO 717.1. Samples

Window and door framing: Submit the following:

- Accessory and hardware items documented descriptively or by performance (i.e. not documented as proprietary items) including locks, latches, handles, catches, sash operators, anchor brackets and attachments, masonry anchors and weather seals (pile or extruded).

- Colour samples of prefinished production material (e.g. anodised or organic coated extrusions and sheet), showing the limits of the range of variation in the selected colour.

- Joints made by proposed techniques.

- Sections proposed to be used for frames, sashes, louvres and slats.

- Label each sample, giving the series code reference and date of manufacture.

Glazing: Submit samples of glazing materials, each at least 200 x 200 mm, showing documented visual properties and the range of variation, if any, for each of the following types of glass or glazing plastics:

- Tinted or coloured glass or glazing plastics.

- Surface modified or surface coated glass.

- Patterned or obscured glass or glazing plastics.

- Ceramic coated glass.

- Wired glass.

- Mirror glass.

Hardware: Submit samples of generic hardware, not documented as proprietary items, as follows:

- Particular samples required:

  - Window Screen and frame system for all windows.

  - Part Door Leaf sample for Metal faced and Internal Solid Core door Shop drawings

Submit shop drawings, to a scale that best describes the detail, showing the following:

- Full size sections of members.

- Hardware, fittings and accessories including fixing details.

- Junctions and trim to adjoining surfaces.

- Layout (sectional plan and elevation) of the window assembly.

- Lubrication requirements.

- Methods of assembly.

- Methods of installation, including fixing, caulking and flashing.

- Provision for vertical and horizontal expansion.

- Method of glazing, including the following:

. Rebate depth.

. Edge restraint.

. Clearances and tolerances.

. Glazing gaskets and sealant beads.

Subcontractors

General: Submit names and contact details of proposed manufacturers and installers. Evidence of experience: Over 10 years’ experience in Samoa.

Warranties

General: Submit warranties as documented.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Openings prepared to receive windows (where windows are to be installed in prepared openings).

- Fabricated window assemblies at the factory ready for delivery to the site.

- Fabricated window assemblies delivered to the site, before installation.

- Commencement of window installation.

- PRODUCTS

  - GENERAL

Fire-resistance of building elements Fire-resistance level: To AS 1530.4. Marking

Identification: Marked to show the following:

- Manufacturer’s identification.

- Product brand name.

- Product type.

- Quantity.

- Product reference code and batch number.

- Date of manufacture.

- Material composition and characteristics such as volatility, flash point, light fastness, colour and pattern.

  - GLASS

Glass and glazing materials

Glass: Free from defects which detract from appearance or interfere with performance under normal conditions of use.

Glazing plastics: Free from surface abrasions, and warranted by the manufacturer for 10 years against yellowing or other colour change, loss of strength and impact resistance, and general deterioration.

Safety glasses

Standard: To AS/NZS 2208. Certification: Required.

Certification provider: An organization accredited by the Joint Accreditation System of Australia and New Zealand (JAS-ANZ).

Type: Grade A to AS 1288. Heat soaking

Requirement: All toughened glass products.

Standard: To EN 14179-1.

Unacceptable blemishes in heat-treated flat glass (including tinted and coated glass) Standard: To AS/NZS 4667.

Insulating glass units (IGUs)

Selection and installation: To AS/NZS 4666.

  - GLAZING MATERIALS General

Glazing materials: Provide including putty, glazing compounds, sealants, gaskets, glazing tapes, spacing strips, spacing tapes, spacers, setting blocks and compression wedges appropriate for the conditions of application and the required performance.

Jointing materials

Requirement: Provide jointing and pointing materials to manufacturer's recommendations which are compatible with each other and with the contact surfaces and non-staining to finished surfaces. Do not provide bituminous materials on absorbent surfaces.

Elastomeric sealants

Sealing compound (polyurethane, polysulfide, acrylic): To ASTM C920 or ISO 11600. Sealing compound (silicone): To ASTM C920 or ISO 11600.

Sealing compound (butyl): To ASTM C1311. Priming

Application: Apply the manufacturer's recommended primer to the surfaces in contact with sealant

materials.

Control  joints

Depth of elastomeric sealant: One half the joint width or 6 mm, whichever is the greater.

Foamed materials (in compressible fillers and backing rods): Closed-cell or impregnated types which do not absorb water.

Bond breaking: Provide backing rods, and other back-up materials for sealants, which do not adhere to the sealant.

  - GLASS IDENTIFICATION

Safety glazing materials Identification: To AS 1288.

Noise reducing glazed assemblies

Labelling: Label each panel with a legible non-permanent mark, stating and certifying the Rw rating, and identifying the testing authority. Remove when directed.

  - INSECT SCREENS Fixed screens

General: Provide fixed screens to the window frames with a clipping device which permits removal for cleaning.

Hinged screens

General: Hinge at the top to give access to opening sash. Roll up screens

General: Provide a proprietary retractable insect screen, comprising aluminium frame with baked

enamel finish, fiberglass mesh beaded into the frame, and a retraction system including tension spring, nylon bearings, positive self-locking device, and plastic sealing strip at sill.

Sliding screens

General: Provide a matching aluminium head guide, sill runner, and frame stile sections for screens not part of the window frame.

Hardware: Nylon slide runners and finger pull handle. Provide pile strip closers against sash where necessary to close gaps.

Aluminium framed screens

General: Provide aluminium extruded or folded box frame sections with mesh fixing channel, mitred, staked and screwed at corners. Provide an extended frame section where necessary to adapt to window opening gear.

Mesh: Bead the mesh into the frame channel with a continuous resilient gasket, so that the mesh is taut and without distortion.

  - SECURITY WINDOW GRILLES General

Requirement: Provide proprietary metal security grille screens, or operable screen and frame, fixed to the building structure with tamper resistant fastenings.

Security window grilles: To AS 5039. Installation: To AS 5040.

  - ALUMINIUM FRAME FINISHES

Powder coatings Standard: To AS 3715.

Product: Designer nominated Glass and Aluminium systems Type: 100 x Shop frame for all Doors and windows.

Colour: Powder Coated.

Anodised

Standard: To AS 1231.

Thickness: ≥ 15 microns to 20 microns. Colour: Powder Coated

  - ANCILLARY MATERIALS Trims

Timber: Solid H3.1 timber at least 19 mm thick, mitred at corners. Extruded gaskets and seals

General: Provide seals as documented in the Window and door seal schedule.

Location or function: Allow for top and bottom weather Materials: Non-cellular (solid) elastopressive seals as follows:

- Flexible polyvinyl chloride (PVC): To BS 2571, 100% solids with high consistency, ultraviolet stabilized.

- Rubber products (neoprene, ethylene propylene diene monomer (EPDM) or silicone rubber): To BS 4255-1.

Flashings

General: Corrosion resistant, compatible with the other materials in the installation, and coated with a non-staining compound where necessary.

Standard: To AS/NZS 2904. Nylon brush seals

General: Dense nylon bristles locked into galvanized steel strips and fixed in a groove in the edge of

the door or in purpose-made anodised aluminium holders fixed to the door with double sided PVC foam tape.

Pile weather strips Standard: To AAMA 701/702. Location: To all Door stiles

Materials: Polypropylene or equivalent pile and backing, low friction silicone treated, ultraviolet stabilised.

Finned type: A pile weather seal with a central polypropylene fin bonded into the centre of the backing rod and raised above the pile level.

Weather bars

General: Provide a weather bar under hinged external doors, locate under the centres of closed doors.

  - HARDWARE

Hardware documented generically

General: Provide hardware of sufficient strength and quality to perform its function, appropriate to the intended conditions of use, compatible with associated hardware, and fabricated with fixed parts firmly joined.

Locks and latches Standard: To AS 4145.3.

Window catches: Provide 2 catches per sash to manually latched awning or hopper sashes over 1000 mm wide.

Sash balances

Requirement: Match the spring strength of the balances to the sash weight they support. Sash operators

Requirement: As documented.

- EXECUTION

  - GLASS PROCESSING General

Processing: Perform required processes on glass, including cutting, obscuring, silvering and bending. Form necessary holes, including for fixings, equipment, access openings and speaking holes. Process exposed glass edges to a finish not inferior to ground arises.

  - INSTALLATION Glazing

General: Install the glass as follows:

- Permanently fix in place each piece of glass to withstand the normal loadings and ambient conditions at its location without distortion or damage to glass and glazing materials.

- No transfer of building movements to the glass.

- Watertight and airtight for external glass.

Temporary marking: Use a method which does not harm the glass. Remove marking on completion.

Toughened glass: Do not cut, work, or permanently mark after toughening. Use installation methods which prevent the glass making direct contact with metals or other non-resilient materials.

Heat absorbing glass: In locations exposed to direct sunlight, provide wheel cut edges free from damage or blemishes, with minimum feather.

Preglazing

Window assemblies and glazed doors: Supply inclusive of glazing, shop preglazed. Site glazing

Do not fit on site, all glazing must be installed within the workshop prior to transport to site.

External timber framed glazing: Glaze with putty. Windows and glazed doors

General: Install windows and glazed doors frames as follows:

- Plumb, level, straight and true within building tolerances.

- Fixed or anchored to the building structure in conformance with the wind action loading requirements.

- Isolated from any building loads, including loads caused by structural deflection or shortening.

- Allow for thermal movement. Weatherproofing

Flashing and weatherings: Install flashings, weather bars, drips, storm moulds, caulking and pointing

so that water is prevented from penetrating the building between the window frame and the building structure under the prevailing service conditions, including normal structural movement of the building.

Fixing

Fasteners and fastener spacing: Conform to the recommendations of the manufacturer. Fasteners: Conceal fasteners.

Packing: Pack behind fixing points with durable full width packing.

Prepared masonry openings: If fixing of timber windows to prepared anchorages needs fastening from the frame face, sink the fastener heads below the surface and fill the sinking flush with a material compatible with the surface finish.

Joints

General: Make accurately fitted tight joints so that neither fasteners nor fixing devices such as pins, screws, adhesives and pressure indentations are visible on exposed surfaces.

Sealants: If priming is recommended, prime surfaces in contact with jointing materials. If frames are powder coated, apply a neutral cure sealant.

Operation

General: Make sure moving parts operate freely and smoothly, without binding or sticking, at correct tensions or operating forces and are lubricated.

Protection

Removal: Remove temporary protection measures from the following:

- Contact mating surfaces before joining up.

- Exposed surfaces. Repair of finish

Polyester or fluoropolymer coatings: Contact supplier for approval to apply touch up products,

otherwise replace damaged material. Trim

General: Provide mouldings, architraves, reveal linings, and other internal trim using materials and

finishes matching the window frames. Install to make neat and clean junctions between frames and the adjoining building surfaces.

  - HARDWARE Fasteners

Materials: Use materials compatible with the item being fixed and of sufficient strength, size and quality to perform their function.

- Concealed fixings: Provide a corrosion-resistant finish.

- Exposed fixings: Match exposed fixings to the material being fixed.

Support: Provide appropriate back support (for example lock stiles, blocking, wall noggings and backing plates) for hardware fixings.

- Hollow metal sections: Provide backing plates drilled and tapped for screw fixing, or provide rivet nuts with machine thread screws. Do not use self-tapping screws or pop rivets.

Proprietary window systems

Requirement: Provide the standard hardware and internal fixing points for personnel safety harness attachment, if required by and conforming to the governing regulations.

Operation

General: Make sure working parts are accurately fitted to smooth close bearings, without binding or sticking, free from rattle or excessive play, lubricated where appropriate.

Supply

Delivery: Deliver window hardware items, ready for installation, in individual complete sets for each window set, as follows:

- Clearly labelled with the intended location.

- In a separate dust and moisture proof package.

- Including the necessary templates, fixings and fixing instructions.

  - COMPLETION Hardware

Adjustment: Leave the hardware with working parts in working order, and clean, undamaged, properly adjusted, and lubricated where appropriate.

Trade clean

Method: Clean with soft clean cloths and clean water, finishing with a clean squeegee. Do not use abrasive or alkaline materials.

Extent: All frames and glass surfaces inside and out. Warranties

Window and door assemblies: Submit the manufacturer’s published product warranties.

Hardware: Submit the manufacturer’s published product warranties.

- SELECTIONS

  - PERFORMANCE

External Door and Window Minimum Performance (Design Loads)

| Property | Crime Safe Screen and Frame and Window Frames |
| --- | --- |
| Ultimate limit state (ULS) wind pressure (Pa) | 3.65 kPa |
| Serviceability limit state (SLS) wind pressure (Pa) | 2.40 kPa |
| Water penetration resistance (Pa) | 300Pa |
| Impact Loads from Wind Borne Debris (Clause 2.5.8 AS 1170.2 – 2011) | Vr 80m/s |

| Property | A – Hinge Doors (Internal and External) | B - Hinge Double Door (External and Internal) | C – Internal Sliding Door (internal) | D-External Bi Folding Door (external) |
| --- | --- | --- | --- | --- |
| Produ | Glass and Aluminium or equally an approved product. | Glass and Aluminium or equally an approved product. | Glass and Aluminium or equally an approved product. | Glass and Aluminium or equally an approved product. |
| Suite description | A versatile 100 mm (nominal) x 45 mm 'centre pocket' glazed framing system suitable for shopfront, entry door & general framing applications. Hinged standard profiles - 45 mm deep, heavy duty profiles – 70 mm deep | A versatile 100 mm (nominal) x 45 mm 'centre pocket' glazed framing system suitable for shopfront, entry door & general framing applications. Hinged standard profiles - 45 mm deep, heavy duty profiles – 70 mm deep | Sliding Door system with one fixed panel and single sliding door and access compliant | Bi-Folding Door system with bottom and head track and Jamb. Recess the bottom track so it is flush with the external and internal finish floor level. |
| Frame material | Aluminium | Aluminium | Aluminium | Aluminium |
| Frame finish | Powder Coated | Powder Coated | Powder Coated | Powder Coated |
| Insect screen: Frame material and finish | Powder Coated | Powder Coated | Powder Coated | Powder Coated |
| Insect screen: Mesh type | Stainless Steel – Powder coated Black | Stainless Steel – Powder coated Black | Stainless Steel – Powder coated Black | Stainless Steel – Powder coated Black |
| Glazing | 6.38mm Laminated Glass | 6.38mm Laminated Glass | 6.38mm Laminated Glass | 6.38mm Laminated Glass |
| Safety markings | 75mm wide Opaque | 75mm wide Opaque | 75mm wide Opaque | 75mm wide Opaque |
| Hardware | Refer to Door Specification | Refer to Door Specification | Refer to Door Specification | Refer to Door Specification |
| Fixing | Allow for a minimum of 4- M10 fixings per jamb and 2-M10 per head with masonry screws/bolts as per manufacturers specification. |  |  |  |

  - GLAZED DOORS SCHEDULE Glazed Door schedule

Glazed Window schedule

| Property | A –Internal Fixed Window | B - Internal Sliding Window | C - External Glass Louvre Window. Refer to Louver section |
| --- | --- | --- | --- |
| Product name | Glass and Aluminium or equally an approved product. | Glass and Aluminium or equally an approved product. | Glass and Aluminium or equally an approved product. |
| Proprietary suite | 048 Series is a 47.5mm wide framed window system with both openable sash and fixed panel glazing capabilities. | 131 Series is a 47.5mm wide framed window system for sliding coupled with the fixed window system | 050 Series with breezeway Altair 153 blade Strong hold system fitted within. |
| Suite description | As noted above | As noted above | Louvre galleries accommodate 102, 130 or 152 mm glass, aluminium and/or timber blade options in single and multiple bay configurations. Allow to reinforce frame for design wind pressure |
| Frame material | Aluminium | Aluminium | Aluminium |
| Frame finish | Powder Coated | Powder Coated | Powder Coated |
| Insect screen: Frame and material and finish | N/A | N/A | Refer to Crimesafe System |
| Insect screen: Mesh type | N/A | N/A | Refer to Crimesafe System |
| Glazing | 6.38mm Clear Laminated glass. | 6.38mm Clear Laminated glass. | 6.00mm Annealed Grey Tint Glass Blades. Opaque blades for all Toilet/Shower areas. Allow to cut glass to suit dimensions in both directions |
| Safety markings | Nil - | Nil - | Nil - |
| Hardware | Allow for Handles and keyed locks to close and secure the window. | Allow for Handles and keyed locks to close and secure the window. | Allow for Handles and keyed locks to close and secure the louvre. |

| Property | Crimesafe external Screen and Frame |
| --- | --- |
| Product name | Crime Safe screen and Cyclone frame or equally an approved product. |
| Generic description | Cyclone Debris Screen for region C and D . This includes the Crimsafe 100mm Cyclone Build Out Frame fixed to block wall complete with Crimesafe 1.2M 304 Stainless steel powder coated screen |
| Frame material | Aluminium |
| Frame finish | Powder Coated |
| Mesh type | 1.2mm 304 Stainless steel screw clamped into a commercial frame. |
| Design Wind Speed (Impact Speed) | 44m/s |

SCREENS Screen schedule

0453B DOORS AND ACCESS PANELS

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide doors, frames, doorsets, security screen doors and fire-resisting doorsets, as documented.

Prison Holding Cells

Requirement: Refer to attached “Cell Security Ltd” Technical Catalogue for relevant detail information in relation to Drawing details.

  - GENERAL

Requirement: Conform to the following:

- 0171 General requirements.

- 0455 Door hardware.

  - INTERPRETATION Definitions

General: For the purposes of this worksection the following definitions apply:

- Balanced construction : Flush door construction where the facings on one side of the core are nominally equal in thickness, grain direction, properties and arrangement to those on the other side of the core, such that uniformly distributed changes in moisture content will not cause warpage.

- Cellular core door: Timber hollow core doors with cellulose mesh grid or honeycomb core encased by timber rails and stiles.

- Door frame: Includes jamb linings.

- Doorset: An assembly comprising a door or doors and supporting frame, guides and tracks including the hardware and accessories necessary for operation.

. Fire-resisting doorset: A doorset which retains its integrity, provides insulation and limits, if required, the transmittance of radiation in a fire.

. Smoke-doorset: A doorset which restricts the passage of smoke.

- Flush door: A door leaf with two plane faces which entirely cover and conceal its structure. It includes doors with intermediate rail, cellular, blockboard, medium density fibreboard (MDF) and particleboard cores.

. Solid core door: A flush door with a solid core continuous between stiles and rails or edge strips and fully bonded to the faces.

. Hollow core door: Flush door leaf with air spaces between the two outer facings, constructed with either a cellular core or skeleton core.

- Joinery door: A door leaf with either stiles and rails, or stiles, rails and muntins, framed together. A joinery door may also incorporate glazing bars.

. Louvred door: A joinery door with framed openings filled in with louvre blades.

. Panelled door: A joinery door with framed openings filled in with panels including glass.

  - SUBMISSIONS Products and materials

Type tests: Submit results, as follows:

- Fire-resisting and smoke doorsets: To AS 1905.1 and BCA Spec C3.4.

- Weighted sound reduction index (Rw): To AS/NZS ISO 717.1.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Door frames in place before building in to masonry.

- Door frames installed before fixing trim.

- PRODUCTS

  - FRAMES Aluminium frames

General: Assembled from aluminium sections, including accessories such as buffers, pile strips, strike plates, fixing ties or brackets and cavity flashing, with provision for fixing documented hardware.

Threshold: If the frame includes a threshold member, provide a self-draining section with anti-skid surface.

  - DOORS General

Doors: Proprietary products manufactured for interior or exterior applications and for the finish required.

Materials

Standards: Conform to the following:

- Decorative laminated sheets: To AS/NZS 2924.1.

- Wet processed fibreboard (including hardboard): To AS/NZS 1859.4.

- Dry processed fibreboard (including medium density fibreboard): To AS/NZS 1859.2.

- Particleboard: To AS/NZS 1859.1.

- Plywood and blockboard for interior use: To AS/NZS 2270.

- Plywood and blockboard for exterior use: To AS/NZS 2271.

- Seasoned cypress pine: To AS 1810.

- Timber – hardwood: To AS 2796.1.

- Timber – softwood: To AS 4785.1. Certification

Panel doors: Provide panels branded under the authority of a recognised certification program

applicable to the product. Locate the brand on faces or edges which will be concealed in the works. Joinery doors

General: Provide joinery doors, as documented.

Flush doors

General: Provide flush doors of balanced construction. Solid core: Solid flush doors as follows:

- Flush door with blockboard: Core plate of timber strips laid edge to edge, fully bonded to each other and to facings each side of no less than two sheets of timber veneer.

- Flush doors with particleboard: Core plate of particleboard fully bonded to facings each side of no less than two sheets of timber veneer.

- Allow for metal 1.50mm BMT metal facing where doors are required to have a face or both faces lined.

Smoke doors: Solid core not less than 35 mm thick. Construction

Adhesives:

- Internal: To AS/NZS 2270.

- External: To AS/NZS 2271. Door thickness:

- General: 35 mm.

- External doors and doors over 900 mm wide: 40 mm.

Cut-outs: If openings are required in flush doors (e.g. for louvres or glazing), do not make cut-outs closer than the width of the stiles at the edges of the doors.

Edge strips: Minimum thickness 10 mm. Increase overall thickness to greater than 15 mm to accommodate the full depth of the rebate in rebated doors. Apply to the external edges of door after the facings are bonded to the door framing/core and finish flush with outside surface of the facings.

Edge strip location: along top and bottom edge

Louvre grilles: Construct by inserting the louvre blades into a louvre frame, and fix the frame into the door.

Double doors

Square edged doors: Bevel as necessary to prevent binding between the leaves.

Rebated meeting stiles: If not double acting doors, provide rebated meeting stiles or fix equivalent metal T stop to one leaf. Form rebates to suit standard rebated hardware.

Tolerance

Squareness: The difference between the lengths of diagonals of a door: Maximum 3 mm.

Twist: The difference between perpendicular measurements taken from diagonal corners: Maximum 3 mm.

Nominal size (mm):

- Height: ± 2.

- Width: + 2, - 0.

  - DOORSETS Duct access panels

General: Proprietary products comprising metal-faced doors side hung to steel door frames, including hardware and accessories such as hinges and lock and installation lugs.

Floor access panels

Frame: Weld from 50 x 50 x 6 mm angle, with two 40 mm cogged fixing lugs each side and shop prime.

Covers: 6.5 mm checker floorplate, on 40 x 40 x 6 mm angle welded frame with 32 x 6 mm diagonal stiffening flats. Cut, radius and grind off 100 x 25 mm lifting slots in each end of covers.

  - ANCILLARY MATERIALS Trims

Timber: Solid timber at least 19 mm thick, mitred at corners. Extruded gaskets and seals

General: As documented in the Door seal schedule.

Materials: Non-cellular (solid) elastopressive seals as follows:

- Flexible polyvinyl chloride (PVC): To BS 2571, 100% solids with high consistency, ultraviolet stabilised.

- Rubber products (neoprene, ethylene propylene diene monomer (EPDM) or silicone rubber): To BS 4255-1.

Flashings

General: Corrosion resistant, compatible with the other materials in the installation, and coated with a non-staining compound where necessary.

Standard: To AS/NZS 2904. Jointing materials

General: Compatible with each other and with the contact surfaces and non-staining to finished

surfaces. Do not provide bituminous materials on absorbent surfaces. Nylon brush seals

General: Dense nylon bristles locked into galvanized steel strips and fixed in a groove in the edge of

the door or in purpose-made anodised aluminium holders fixed to the door with double sided PVC foam tape.

Pile weather strips

General: Polypropylene or equivalent pile and backing, low friction silicone treated, ultraviolet stabilised.

Standard: To AAMA 701/702. Weather bars

General: Provide a weather bar under hinged external doors, locate under the centres of closed doors.

- EXECUTION

  - FRAMES General

Frames: Install the frames as follows:

- Plumb, level, straight and true.

- Fixed or anchored to the building structure.

- Isolated from any building loads, including loads caused by structural deflection or shortening. Frame fixing

Brackets: Metallic-coated steel:

- Width: Minimum 25 mm.

- Thickness: Minimum 1.5 mm.

Depth of fixing for building into masonry:

- Brackets: Minimum 200 mm.

- Expansion anchors: Minimum 50 mm.

- Plugs: Minimum 50 mm.

- Rods: Minimum 60 mm.

Jamb fixing centres: Maximum 600 mm. Joints

General: Make accurately fitted joints where fasteners, pins, screws, adhesives and pressure

indentations are not visible on exposed surfaces. Aluminium frames

Building into masonry: Screw galvanized steel brackets twice to jambs and build in.

Fixing to masonry openings: Build in seasoned timber plugs to masonry joints or use proprietary expansion anchors and screw twice through jambs at each fixing.

Fixing to stud frame openings: Screw once to studs at each fixing. Finishing

Trim: Provide mouldings, architraves, reveal linings, and other internal trim using materials and

finishes matching the door frames to make neat and clean junctions between the frame and the adjoining building surfaces.

Seals

General: Provide the fixings, rebates, grooves, and clearances required for installation and operation of the seals. Allow seals unwound from coils to settle before use.

Weatherproofing

Flashings and weatherings: Install flashings, weather bars, drips, storm moulds, caulking and pointing to prevent water from penetrating the building between the door frame and the building structure under the prevailing service conditions, including normal structural movement of the building.

  - DOORS Priming

General: Prime timber door leaves on top and bottom edges before installation.

  - DOORSETS

Security screen doorsets Standard: To AS 5040.

  - COMPLETION Operation

General: Make sure moving parts operate freely and smoothly, without binding or sticking, at correct tensions or operating forces and that they are lubricated where appropriate.

Protection

Temporary coating: On or before the date for practical completion, or before joining up to other surfaces, remove all traces of temporary coatings used as a means of protection.

| Property | Typical Solid Core Door (Internal) | Typical Solid Core Door (External) |
| --- | --- | --- |
| Frame | Aluminium Shop frame 472 Series 100x 40 or similar. Ensure stops are installed within the frame to take the required seals | Aluminium Shop frame 100x 40 or similar. Ensure stops are installed within the frame to take the required seals |
| -Finish | Powder Coated | Powder Coated |
| -Fan Light | Louvre Panel Neptune by Lowline Group or Similar approved | Similar to internal where noted on the door schedule |

SELECTIONS

  - DOOR TYPES SCHEDULE Door Frames

| Property | Typical Solid Core Door (Internal) | Typical Solid Core Door (External) |
| --- | --- | --- |
| Door type | Solid Core | Solid Core |
| Thickness (mm) | 40mm | 40mm |
| Core material | Solid H3.2 Pine Core | Solid H3.2 Pine Core |
| Facing material | H3.1 Plywood both sides | H3.1 Plywood internal face and 1.5mm BMT Folded metal on the external face wrapped around the edges, glued and pressed. |
| Face veneers: Timber species or group | Hoop Pine | Hoop Pine |
| Face veneers: Veneer quality | A Grade | A Grade |
| Face veneers: Matching arrangement | Match Along the length | Match Along the length |
| Edge strip thickness (mm) | Solid timber edging approximately 50mm | Solid timber edging approximately 50mm |
| Panel: Clear opening size (mm) | Refer to Door Schedule | Refer to Door Schedule |
| Finish | Paint Finish. Prime all edges prior to installation. | Paint Finish. Prime all edges prior to installation. |
| Floor clearance | 10mm | 10mm |
| Threshold |  | Raven RP77 or similar to suit floor finish and transition at door location |
| Weather seals (All Around) | Allow for RP7 seals to all doors. | Fit Raven RP7 or similar Rubber Seals or Closed Cell Foam Seals |
| Glazed Panels (If Shown) | 6.38mm laminated clear glass for all office where indicated and | Similar as internal where noted |

Flush doors construction schedule

| Property | Typical Solid Core Door (Internal) | Typical Solid Core Door (External) |
| --- | --- | --- |
|  | Opaque glass for Toilets |  |
| Ventilation Grilles | Where noted on the door schedule allow for door grilles for air relief instead of undercuts. Allow to install Holyoak Clear Anodised Louvre complete with frame | Similar as internal where noted |

0455 DOOR HARDWARE

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide door hardware, as documented.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - INTERPRETATION Abbreviations

General: For the purposes of this worksection, the abbreviations given in AS 4145.1 Appendix D apply.

Definitions

General: For the purposes of this worksection, the general definitions given in AS 4145.1 Section 2 apply.

Lock function: For the purposes of this worksection, the general definitions given in AS 4145.1 Appendix E apply.

  - SUBMISSIONS Door-by-door schedule

General: Submit a door-by-door hardware schedule.

Information sources: This worksection and the contract drawings. Key control system

New works: Submit details of the proprietary key control security system proposed by the lock

manufacturer for locks required to accept a group key (master, grandmaster).

Alterations and additions: Submit details to extend the existing key control security system for locks required to accept a group key.

Maintenance manuals

Automatic door operators: Submit the installer’s proposal for continuing maintenance after completion on an annual renewal basis.

Manual: Submit the manufacturer’s published recommendations for use, care and maintenance of the hardware provided.

Records

Door hardware schedule: Submit an amended schedule, prepared by the door hardware supplier, showing changes to the contract door hardware schedule resulting from the following:

- Approval of a hardware sample.

- Acceptance of an equivalent to a specified proprietary item.

- A contract variation to a door hardware requirement. Execution details

Re-use of recovered hardware: Submit a proposal describing the standard of cleaning, repair and

testing of recovered items and the location where each is to be reused. Samples

Generic items: Submit samples of hardware items offered as meeting the description of items not

specified as proprietary items. Technical data

Key codes: Submit the lock manufacturer’s record of the key coding system showing each lock type,

number and type of key supplied, key number for re-ordering, and name of supplier.

Keys: For locks keyed to differ and locks keyed alike, verify quantities against key records, and deliver to the contract administrator at practical completion.

Warranties

Requirement: Submit the following:

All door hardware: Submit a warranty (or interlocking warranties) from the supplier and installer for the system and its installation, for a period of at least twelve months from the date of practical completion.

Minimum period: 12months from Practical completion

Form of warranty: Written and must be executable through a local Samoan based supplier.

- PRODUCTS

  - GENERAL Supply

Delivery: Deliver door hardware items, ready for installation, in individual complete sets for each door, as follows:

- Clearly labelled to show the intendedlocation.

- In a separate dust and moisture proof package.

- Including the necessary templates, fixings and fixing instructions.

Hardware specified generically: Provide hardware of sufficient strength and quality to perform its function, appropriate to the intended conditions of use, suitable for use with associated hardware, and fabricated with fixed parts firmly joined.

Replacement items

Door hardware: Match items being replaced with existing unless documented otherwise. Upgrade hinges as necessary to conform to Hinges for timber doors table and Hinges for aluminium doors table.

  - LOCKS AND LATCHES Standard

General: To AS 4145.2. Padlocks

Standard: To AS 4145.4.

Lock and latch classification

Rating systems: To AS 4145.1 Section 3. Performance requirements: To AS 4145.2 Section 3.

  - HINGES

Butt hinge materials

Timber doors in timber or steel frames:

- Material: Brushed Stainless steel

- Product: Refer to hardware schedule Aluminium framed doors in aluminium frames:

- Material: Brushed Stainless steel

- Product: Refer to hardware schedule

Doors fitted with closers: Provide low friction ball bearing hinges. Fire-resisting doors: To AS 1905.1.

Power transfer hinges: Do not load and install with other compatible hinges.

Lift-off doors: If toilet cubicles require lift-off doors, provide lift-off hinges and allow for door panel with sufficient clearance at the head to facilitate door removal.

Solid core doors

Number of hinges: Determine the number of hinges required based on the nominated door leaf size and weight only. For other door leaf sizes or for doors with applied finishes, use the weight of the door to determine the number of hinges required. For a door leaf over 80 kg, use pivot hinges.

Size of hinges: Determine the size of the hinge based on the door leaf thickness:

- 35 to 43 mm thick door: 100 x 75 mm butt hinges with a minimum thickness of 2.5 mm.

- 44 to 55 mm thick door: 100 x 100 mm butt hinges with a minimum thickness of 2.5 mm.

- > 55 mm thick door: Refer to the door by door hardware schedule.

Hinge pin: Supply fixed pins to hinges of doors opening out or designated as a security doors. For all other doors, provide loose pins.

Wide throw: If necessary, provide wide throw hinges to achieve the required door swings in the presence of obstacles such as nibs, deep reveals and architraves.

Hinges for timber doors table

| Nominal door leaf size (H x W x T) (mm) | Door leaf weight | Number of hinges |
| --- | --- | --- |
| 2040 x 400 x 35 | ≤ 19 | 2 |
| 2040 x 600 x 35 | ≤ 29 | 2 |
| 2040 x 720 x 35 | ≤ 35 | 3 |
| 2040 x 820 x 35 | ≤ 39 | 3 |
| 2040 x 920 x 35 | ≤ 44 | 3 |
| 2040 x 1020 x 35 | ≤ 49 | 4 |
| 2040 x 720 x 40 | ≤ 37 | 3 |
| 2040 x 820 x 40 | ≤ 42 | 3 |
| 2040 x 920 x 40 | ≤ 48 | 3 |
| 2040 x 1020 x 40 | ≤ 52 | 4 |
| 2040 x 720 x 50 | ≤ 45 | 3 |
| 2040 x 820 x 50 | ≤ 50 | 3 |
| 2040 x 920 x 50 | ≤ 57 | 3 |
| 2040 x 1020 x 50 | ≤ 68 | 4 |
| 2400 x 720 x 40 | ≤ 50 | 4 |
| 2400 x 820 x 40 | ≤ 52 | 4 |
| 2400 x 920 x 40 | ≤ 55 | 4 |
| 2400 x 1020 x 40 | ≤ 60 | 4 |
| 2400 x 1220 x 50 | ≤ 72 | 5 |
| 2040 x 920 x 70 | ≤ 88 | Pivot hinges |

Aluminium doors

Application: Provide aluminium hinges for aluminium doors, or for doors of other materials in aluminium frames of a weight of 40 kg or less.

Hinges for aluminium doors table

| Nominal hinge size (L x W x T) (mm) | Door leaf weight | Knuckles (minimum) | Screws/hinge leaf (minimum) |
| --- | --- | --- | --- |
| 100 x 70 x 3 | ≤ 30 | 3 | 3 |
| 100 x 80 x 3.5 | ≤ 50 | 5 | 4 |
| 130 x 50 x 3.4 | ≤ 75 | Interfold | 3 |
| Note: Length (l) is the dimension along the knuckles, not including hinge tips, if any, and width (w) is the dimension across both hinge leaves when opened flat. |  |  |  |

  - DOOR HANGING SYSTEMS General

Requirement: Provide sliding door tracks as documented in the Door hardware selection schedule.

  - ANCILLARIES Bolts

General: Provide bolts including barrel bolts, flush bolts and tower bolts with keepers, including lock plates, staples, ferrules or floor sockets.

Mortar guards

General: For steel door frame installations, provide mortar guards designed to allow the full extension of the lock tongue or similar devices and the correct operation of the locking mechanism.

Rebated doors

General: For mortice locks or latches to rebated doors, provide purpose-made rebated pattern items. Strike plates

General: Use strike plates provided with the locks or latches. Do not provide universal strike plates.

  - DOOR CONTROLLERS Standard

General: To AS 4145.5. Performance

Requirement: Provide door controllers, pivots, floor or overhead door closers, and automatic door

operators, which are suitable for the door type, size, weight and swings required and the operating conditions, including wind and air conditioning pressure.

Closers

Hinged and pivot doors:

- Fire-resisting doors: Provide closers tested and certified for use as components of fire-resisting door assemblies:

. Standard: To AS 1905.1.

  - ELECTRONIC CONTROL DEVICES General

Requirement: Provide electric strikes, electric locks, drop bolts, or similar devices to suit door construction and hardware.

Electromagnetic hold-open devices: To AS 1905.1 and AS 1670.1.

Fail-safe: Connect door control devices in a fail-safe mode to permit egress in the event of power failure.

Fail-secure: Connect door control devices in a fail-secure mode to prevent egress in the event of power failure.

Authorized products: Provide equipment listed in the ActivFire Register of Fire Protection Equipment Register of Fire Protection Equipment.

Glass doors: Provide tumbler, drop bolts or magnetic holders.

Double leaf doors (solid frame): Provide an electric strike or lock on the fixed leaf, connected to the door frame by concealed flexible wiring.

Activation

Activation device: Provide keypads, card readers or other activation devices, and locate next to entry points.

External: Provide weatherproof (IP56) hoods or housings for external units.

Mounting height: 900 to 1100 mm from floor level and not less than 500 mm from internal corners.

  - KEYING

Temporary construction keys and cylinders Requirement: Provide one of the following:

- Loan cylinder: Install for construction locks and replace at practical completion.

- Construction keyed master key cylinder: Keep up-to-date records of keys issued including recipient’s name, company and contact details, date issued and date returned.

Delivery of keys

Great grandmaster, grandmaster and master keys: Arrange for the manufacturer or supplier to deliver direct to the principal.

Group keying

Keying system: Provide a group keying system as documented in the Key codes schedule.

Existing system: Obtain the details of existing group or master key systems to which a new system is required to be an extension.

Future extensions: Provide master and grandmaster group keying systems which are capable of accommodating future extensions.

Keying control security system: If cylinder or pin-tumbler locks accept a group key (e.g. master key, maison key) provide to those locks a proprietary keying control security system.

Stamping: Stamp keys and lock cylinders to show the key codes and/or door number as scheduled. Identification

Labelling: Supply each key with a purpose-made plastic or stamped metal label legibly marked to

identify the key, attached to the key by a metal ring. Key material

Lever locks: Malleable cast iron or mild steel.

| Key code | Key type | Minimum number of keys |
| --- | --- | --- |
| GGMK | Great grandmaster keys | 2 |
| GMK | Grandmaster keys | 2 |
| MK | Master keys | 2 per code group |
| KD | Locks keyed to differ | 2 per lock |
| KA | Locks keyed alike: |  |
|  | - 2 locks in code group | 4 |
|  | - 3 to 10 locks in code group | 6 |
|  | - 11 to 40 locks in code group | 10 |
|  | - 41 and over locks in code group | 1 per 4 locks or part thereof |

Pin tumbler locks: Nickel alloy, not brass. Number of keys table

- EXECUTION

  - INSTALLATION General

Handing: Before supply, verify on site, the correct handing of hardware items.

Operation: Make sure working parts are accurately fitted to smooth close bearings, without binding or sticking, free from rattle or excessive play, lubricated where appropriate.

Mounting height

Locks and latches: Centreline of the door knob or lever spindle above finished floor: between 900mm and 1100mm.

Locks

Cylinders: Fix vertically and with consistent key alignment. Door stops

Fixing: Fix on the floor, skirting or wall, as appropriate, to prevent the door or door furniture striking the

wall or other surface. Fasteners

Materials: Provide materials compatible with the item being fixed, and of sufficient strength, size and

quality to perform their function.

- Concealed fixings: Provide a corrosion resistant finish to concealed fixings.

- Exposed fixings: Match exposed fixings to the material being fixed.

Security: Locate exposed fixings to lock furniture on the inside faces of external doors and on the inside faces of internal doors to lockable rooms.

Support: Provide appropriate back support (for example lock stiles, blocking, wall noggings and backing plates) for hardware fixings.

- Hollow metal sections: Provide backing plates drilled and tapped for screw fixing, or provide rivet nuts with machine thread screws. Do not use self-tapping screws or blind rivets.

Floor springs

General: Form a recess in the floor slab for the floor spring box and grout the box in place so that the cover plate is flush with the finished floor.

Hinges

Metal frames: Fix hinges using metal thread screws.

Timber doorsets: Install butt hinges in housings equal in depth to the thickness of the hinge leaf (except for hinges designed for mounting without housing), and fix with countersunk screws.

  - COMPLETION Adjustment

General: Leave the hardware properly adjusted with working parts in working order, and clean, undamaged, properly adjusted, and lubricated where appropriate.

Automatic door operators: Maintain and adjust the system throughout the defects liability period. Keys

Contractor’s keys: Immediately before practical completion, replace or reset cylinders to which the

contractor has had key access during construction to exclude the contractor’s keys. Product warranties

Warranty: Cover materials and workmanship in the form of interlocking warranties from the

manufacturer or distributor and the installer.

- SELECTIONS

Contractor is to provide door hardware schedule for review. The schedule shall clearly state hardware brands. Contractor is to check and coordinate keying with the client prior to ordering.

0454B OVERHEAD DOORS

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide overhead doors, as documented.

  - CROSS REFERENCES

  - GENERAL

Requirement: Conform to the following:

- 0171 General requirements.

  - STANDARDS General

Garage doors and other large access doors: To AS/NZS 4505.

  - SUBMISSIONS

Operation and maintenance manual

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Framing or structure to receive tracks andmotor.

- Tracks and guides installed before doors or shutters are hung.

- PRODUCTS

  - ROLLING CURTAIN AND ROLLING SHUTTER DOORS Types

Rolling curtain:

- Curtain material: Continuous roll formed profiled steel.

. Product: Metalbilt NZ Steel roller Shutter Door – with roll formed interlocking slats or equally an approved product.

. Base metal thickness (BMT): 0.8mm BMT

. Finish: Powder coated

. Allow for security panel across lintel Rolling shutter:

- Curtain material: G250 grade steel

- Slat thickness: 100mm pitch Bottom rail

Requirement: Provide a stiffening member as follows:

- Interlocking with the bottom edge or lowest part of the curtain.

- Extending between the inner faces of the vertical guides.

- Formed or adapted as required to follow the contour of a sloping floor or threshold.

- Adapted to house a locking device, if required.

- Extruded aluminium bottom bar with a Neoprene weather strip fitted to the lower edge of the curtain. Wind locks

General: Provide 95mm deep wind lock end clips and guides to retain the curtain in wide openings or

under extreme wind conditions.

Drum

Maximum drum deflection: 1/360th of the span.

Springs: Helical torsion springs housed in the drum and arranged to counterbalance the curtain weight without exceeding the safe working stress of the spring material.

Wickets

General: Provide doors with metal frame and facings to match the curtain, and manufacturer’s standard lockset and furniture.

Operation method

General: Method of opening and closing the door:

- Manual:

. Chain: By pulling on a chain passing over a sprocket on the drum, with reduction gears where necessary.

  - OPERATION Manual operation

General: Install so that the force required to operate the door manually does not exceed 220 N.

- EXECUTION

  - INSTALLATION General

Requirement: Install overhead doors in conformance with the manufacturer’s recommendations and as documented.

Preparation

Substrate: Before start of installation, check the alignment of substrates or framing and adjust if required.

Frames

Requirement: Install frames as follows:

- Plumb, level, straight, true, and within tolerances and clearances recommended by the manufacturer.

- Fixed or anchored to the building structure using mechanical fixings suitable for the substrate and the imposed loads.

- Isolated from any building loads, including loads caused by structural deflection or shortening.

  - COMPLETION Operation

General: Make sure moving parts operate freely and smoothly, without binding or sticking, at correct tensions or operating forces and that they are lubricated where appropriate.

Safety: Make sure all safety features are operating.

Remote control devices: Make sure devices are programmed and operating. Protection

Temporary coating: On or before the date for practical completion of the works, or before joining up to

other surfaces, remove all traces of temporary coatings used for protection. Warranties

General: Submit the manufacturer’s published product warranties.

0456 LOUVRE WINDOWS

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide louvre windows, as documented.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements. Manufacturer's documents

Manufacturer’s and supplier’s documents related to this work section: Breezeway Louvres and

Frame system or equally an approved product.

  - STANDARDS Glazing

Glass type and thickness: To AS 1288, where no glass type or thickness is given. Materials and installation: To AS 1288.

Quality requirements for cut-to-size and processed glass: To AS/NZS 4667. Terminology for work on glass: To AS/NZS 4668.

  - INTERPRETATION

Abbreviations

General: For the purposes of this work section the following abbreviations apply:

- AWA: Australian Window Association.

- WERS: Window Energy Rating Scheme. Definitions

General: For the purposes of this work section, the following definitions apply:

- Louvres - horizontal: Louvres that span between frames stiles, mullions or vertical supports.

- Total system SHGC: Solar heat gain coefficient as defined by BCA and tested in conformance with NFRC 200.

- Total system u-value: Thermal transmittance as defined by BCA and tested in conformance with NFRC 100.

  - SUBMISSIONS Certification

Conformance: Submit evidence that window assemblies meet documented design wind pressures and water penetration resistance.

Operation and maintenance manual

General: Submit the louvre window manufacturer’s published instructions for operation, care and maintenance.

Samples

General: Submit samples of louvre window system components as follows:

- Accessory and hardware items not selected as proprietary items including louvre holders and operators, locks, latches, handles, catches, anchor brackets and attachments, masonry anchors and weather seals (pile or extruded).

- Colour samples of prefinished production material (e.g. anodised or organic coated extrusions and sheet) showing the limits of the range of variation in the selected colour.

- Frame member profiles and louvre materials.

- Frame member joining techniques.

- Label each sample, giving the Series code reference and date of manufacture.

Glazing: Submit samples of glazing materials, each at least 200 x 200 mm, showing specified visual properties and the range of variation, if any, for each of the following types of glass:

- Tinted or coloured glass.

- Patterned or obscured glass or glazing plastics. Shop drawings

General: Submit shop drawings to a scale that best describes the detail, showing the following:

- Frame member profiles.

- Hardware, fittings and accessories including fixing details.

- Junctions and trim to adjoining surfaces.

- Layout (sectional plan and elevation) of the window assembly.

- Lubrication requirements.

- Methods of assembly.

- Methods of installation including fixing, joint sealing and flashing.

- Provision for vertical and horizontal expansion.

Certification: Submit an engineer's certificate verifying conformance to AS 2047. Tests

Type tests: Submit reports verifying conformance to AS 2047 and the performance requirements as

documented in the Louvre window performance schedule. Warranties

Requirement: Submit the following:

- Manufacturers minimum period

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Openings prepared to receive windows (if windows are to be installed in prepared openings).

- Fabricated window assemblies at the factory ready for delivery to the site.

- Fabricated window assemblies delivered to the site, before installation.

- Commencement of window installation.

- PRODUCTS

  - GENERAL Standards

Flashings: To AS/NZS 2904.

Aluminium extrusions: To AS/NZS 1866. Marking

Identification: Marked to show the following:

- Manufacturer’s identification.

- Product brand name.

- Product type.

- Quantity.

- Product reference code and batch number.

- Date of manufacture.

- Material composition and characteristics such as volatility, flash point, light fastness, colour and pattern.

  - GLASS

Glass and glazing materials

Glass: Free from defects which detract from appearance or interfere with performance under normal conditions of use.

Glazing plastics: Free from surface abrasions, and warranted by the manufacturer for 10 years against yellowing or other colour change, loss of strength and impact resistance, and general deterioration.

  - GLAZING MATERIALS General

Glazing materials (including putty, glazing compounds, sealants, gaskets, glazing tapes, spacing strips, spacing tapes, spacers, setting blocks and compression wedges): Appropriate for the conditions of application and the required performance.

Jointing materials

Compatibility: Provide recommended jointing and pointing materials which are compatible with each other and with the contact surfaces and non-staining to finished surfaces. Do not provide bituminous materials on absorbent surfaces.

Pile weather strips Standard: To AAMA 701/702.

Location: Along edge of the window louvre unit

Materials: Polypropylene or equivalent pile and backing, low friction silicone treated, ultra violet stabilised.

Finned type: A pile weather seal with a central polypropylene fin bonded into the centre of the backing rod and raised above the pile level.

Extruded gaskets and seals

Type: Non-cellular (solid) elastopressive seals. Location or function: Head and Sill seals Material:

- Rubber products (neoprene, ethylene propylene diene monomer (EPDM) or silicone rubber): To BS 4255-1.

- Flexible polyvinyl chloride (PVC): To BS 2571, E type compounds, colour fastness grade B. Priming

Compatibility: Apply the recommended primer to the surfaces in contact with sealant materials.

  - GLASS IDENTIFICATION Safety glazing materials

Identification: Identify each piece or panel, to AS 1288.

  - LOUVRE WINDOW ASSEMBLIES General

Description: Provide louvre blades mounted in a metal surround frame or subframe and able to withstand the permissible-stress-design wind pressure for that location without failure or permanent distortion of members, and without louvre blade flutter.

Adjustable louvres

Description: Provide louvre blades clipped into pivoting blade holders to stiles or coupling mullions, linked together in banks, each bank operated by an operating handle incorporating a latching device, or by a locking bar.

  - STAINLESS STEEL WOVEN INSECT/ SECURITY MESH WINDOW SCREENS General

General: Provide proprietary metal security grille screens, or operable screen and frame, fixed to the building structure with tamper resistant fastenings.

Installation: To external face of the Windows typical. Product: Crime-Safe screen within a proprietary frame

  - ALUMINIUM LOUVRE FRAME FINISHES Anodised

Standard: To AS 1231. Thickness: 20 microns. Colour: Clear

- EXECUTION

  - PRE-INSTALLATION General

Timber reveals: Prime all surfaces of timber reveals which are to be painted before fixing to aluminium frames.

  - INSTALLATION Glazing

General: Install the glass as follows:

- Permanently fix in place each piece of glass to withstand the normal loadings and ambient conditions at its location without distortion or damage to glass and glazing materials.

- No transfer of building movements to the glass.

- Watertight and airtight for external glazing.

Temporary marking: Use a method which does not damage the glass. Remove marking on completion.

Toughened glass: Do not cut, work, or permanently mark after toughening. Use installation methods which prevent the glass making direct contact with metals or other non-resilient materials.

Heat absorbing glass: Provide wheel cut edges without damage or blemishes and with minimum feather in locations exposed to direct sunlight.

Louvre windows

General: Install louvre windows frames, as follows:

- Plumb, level, straight and true within acceptable building tolerances.

- Fixed or anchored to the building structure in conformance with the wind action requirements.

- Isolated from any building loads, including loads caused by structural deflection or shortening.

- Allowing for thermal movement. Flashing and weatherings

General: Install flashings, weather bars, storm moulds, joint sealant and pointing to prevent the

ingress of water between the window frame and the building structure under the prevailing service conditions, including normal structural movement of the building.

Fixing

General: Do not penetrate metal flashings with fixings.

Fastener spacing (nominal): 600 mm and maximum 150 mm from reveal ends.

Fasteners: Conceal fasteners. Use size and type to suit window unit size and wind loading conditions. Packing: Pack behind fixing points with durable full width packing.

Joints

Requirement: Make accurately fitted tight joints so that fasteners and fixing devices such as pins, screws, adhesives and pressure indentations are not visible on exposed surfaces.

Sealants: If priming is recommended, prime surfaces in contact with jointing materials. Operation

General: Make sure moving parts operate freely and smoothly, without binding or sticking, at correct

tensions or operating forces and are lubricated. Protection

Removal: Remove temporary protection measures from the following:

- Contact mating surfaces before joining up.

- Exposed surfaces. Repair of finish

Polyester or fluoropolymer coatings: Contact supplier for approval to apply touch up products,

otherwise replace damaged material. Trim

General: Provide mouldings, architraves, reveal linings, and other internal trims using materials and

finishes matching the window frames. Make neat and clean junctions between frames and the adjoining building surfaces.

  - COMPLETION Trade clean

Method: Clean with soft clean cloths and clean water, finishing with a clean squeegee. Do not use abrasive or alkaline materials.

Extent: All frames and glass surfaces inside and out. Warranties

Louvre windows: Provide a warranty as follows:

- Form: Against failure of materials and operation under normal environment and use conditions.

- Period: As offered by the manufacturer.

- Powder coating: As offered by the manufacturer for the selected finish.

- Hardware: As offered by themanufacturer.

- SELECTIONS

  - PERFORMANCE

External Door and Window Minimum Performance (Design Loads)

| Property | Crime Safe Screen and Frame and Window Frames |
| --- | --- |
| Ultimate limit state (ULS) wind pressure (Pa) | 3.65 kPa |
| Serviceability limit state (SLS) wind pressure (Pa) | 2.40 kPa |
| Water penetration resistance (Pa) | 300Pa |
| Impact Loads from Wind Borne Debris (Clause 2.5.8 AS 1170.2 – 2011) | Vr 80m/s |

| Property | A – to all Windows |
| --- | --- |
| Product name | G James Glass and Aluminium and Breeway Altair Louvre system or equally an approved product. |
| Proprietary suite | 050 Series with breezeway Altair 153 blade Strong hold system fitted within.. |
| Frame material | Aluminium |
| Frame finish | Powder Coated, Silver Pearl |
| Louvre blade material | Annealed Glass |
| Louvre blade finish | Clear finish – Grey tint |
| Louvre blade height and width (mm) | 652mm wide blade x 6mm thick with 650 max span |
| Operator type | Manual with lockable handles with Keyed lock |
| Operator and clip colour | Altair Standard louvre handle and locking bar |

LOUVRE WINDOWS Louvre window schedule

| Property | Crimesafe external Screen and Frame |
| --- | --- |
| Product name | Crime Safe screen and Cyclone frame or equally an approved product. |
| Generic description | Cyclone Debris Screen for region C and D . This includes the Crimsafe 100mm Cyclone Build Out Frame fixed to block wall complete with Crimesafe 1.2M 304 Stainless steel powder coated screen |
| Frame material | Aluminium |
| Frame finish | Powder Coated, Silver Pearl |
| Mesh type | 1.2mm 304 Stainless steel screw clamped into a commercial frame. |
| Design Wind Speed (Impact Speed) | 44m/s |

SCREENS Screen schedule

| Property | A – to all Louvre Windows |
| --- | --- |
| Glass type | Vridian Clear Float or equally an approved product. |
| Glass thickness (mm) | 6mm |
| Body tint colour | Grey tint |
| Edge processing | Square arisesed edges |
| Number of edges processed | All leading edges |

GLAZING Glass schedule

0471 THERMAL INSULATION AND PLIABLE MEMBRANES

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide thermal insulation and pliable membrane systems, as documented. Performance

Requirements:

- Complete for their function.

- Conforming to the detail and location drawings.

- Firmly fixed in position.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - INTERPRETATION Definitions

General: For the purposes of this worksection the following definitions apply:

- Fbs-1 (fibre-bio-soluble) mineral wool: Insulation composed of bio-soluble glass or rock fibres.

- Fibre batts: Flexible insulation supplied as factory cut pieces and composed of mineral wool (glass and rock fibre) or polyester fibre.

- Fire hazard properties: To BCA A2.4.

- Pliable building membrane: To AS/NZS 4200.1 and equivalent to sarking-type materials as defined in the BCA.

- Thermal insulation terminology:To AS/NZS 4859.1.

- Vapour permeable (breathable) membrane: A flexible membrane material, normally used for secondary waterproofing that allows for the transmission of water vapour.

  - SUBMISSIONS Certification

Requirement: Submit evidence of conformance to PRODUCTS, INSULATION AND PLIABLE

MEMBRANE, Insulation.

Certification provider: An organisation accredited by the Joint Accreditation System of Australia and New Zealand (JAS-ANZ).

Fire hazard properties

General: Submit evidence of conformance to INSULATION AND PLIABLE MEMBRANE MATERIALS, Fire hazard properties.

Products and materials

Thermal insulation properties: Submit evidence of conformance to AS/NZS 4859.1. Warranties

Manufacturer's published product warranties: Submit on completion.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the pliable membrane and insulation before they are covered up or concealed.

- PRODUCTS

  - GENERAL Marking

Identification: Marked to show the following:

- Manufacturer’s identification.

- Product brand name.

- Product type.

- Quantity.

- Product reference code and batch number.

- Date of manufacture.

- Material composition and characteristics such as volatility, flash point, light fastness, colour and pattern.

  - INSULATION AND PLIABLE MEMBRANE MATERIALS Fire hazard properties

Insulation fire hazard indices: Conform to the following for all materials, tested to AS/NZS 1530.3:

- Spread-of-Flame Index: ≤ 9.

- Smoke-Developed Index: ≤ 8 if Spread-of-Flame Index > 5.

- Materials with reflective facing: Test to AS/NZS 1530.3 clause 4.12(b) and the recommendations of Appendix A6.

Pliable membranes Flammability Index tested to AS 1530.2: ≤ 5. Insulation

Mineral wool blankets and cut pieces: To AS/NZS 4859.1 Section 8.

Wool: To AS/NZS 4859.1 Section 6.

Reflective thermal insulation: To AS/NZS 4859.1 Section 9. Pliable membranes

Standard: To AS/NZS 4200.1.

Vapour barrier:

- Vapour barrier classification: High.

Sarking membrane (other than walls and gables):

- Water barrier classification: High. Fasteners and supports

General: Metallic-coated steel.

Mesh support to roof insulation

Metallic-coated steel wire netting: To AS 2423 Section 4.

- Size: 45 mm mesh x 1 mm diameter. Welded safety mesh: To AS/NZS 4389.

- EXECUTION

  - GENERAL Bulk insulation

Installation: To AS 3999 and BCA J1.2.

General: Firmly butt together fibre blankets or batts, with no gaps except as follows:

- Access openings and vents: Do not obstruct.

- Light fittings: To AS/NZS 3000 clause 4.5.

- Electrical cables: To AS 3999 clause 2.6.

Glass Wool and Rock Wool insulation: Conform to the

ICANZ Industry code of practice for the safe use of glass wool and rock wool insulation.

Marking: Deliver mineral wool products to site in packaging labelled FBS-1 BIO-SOLUBLE INSULATION.

  - ROOF INSULATION General

Location: The whole of the roof area including skylight shaft walls, except the following:

- Eaves, overhangs, skylights, vents and openings.

- Roofs to outbuildings, garages, and semi-enclosed spaces such as verandahs, porches and carports.

Mesh support to roof insulation Locations: Provide support to the following:

- Sarking, vapour barrier or reflective thermal insulation membranes laid over roof framing members which are spaced at more than 900 mm centres.

- Blanket type thermal insulation laid over roof framing members as sound insulation to metal roofing.

Wire safety mesh: Lay over the roof framing allowing only natural mesh sag between members to suit the application. Staple to timber frame, wire to steel frame.

Installing welded safety mesh: To AS/NZS 4389. Metal roofs – bulk insulation

Product: Fletcher Permastop Tropic R1.4 60mm thick double foil blanket insulation

Installation:

- Batts: Fit tightly between framing members.

- Blanket for sound insulation: Install over the roof framing, reflective thermal insulation (if any), and mesh support, so that the blanket is in continuous contact with the underside of the metal roofing sheets.

- Combined blanket and reflective insulation: Lay facing reflective insulation face downwards over safety mesh.

Ceiling insulation – bulk insulation Product type: Fletcher Pink Baits R1.5

- Framed ceilings: Fibre batts.

- Suspended ceiling: Fibre blanket. Application: Over ceiling lining. Installation:

- Batts: Fit tightly between framingmembers.

- Blankets: Butt joint and lay over ceiling panels or lining.

  - COMPLETION Warranties

Insulation and pliable membranes: Submit the manufacturer’s published product warranties.

- SELECTIONS

  - INSULATION

Roof insulation schedule

| Property | A | B |
| --- | --- | --- |
| Application | Laid between and over purlins | Laid between and over ceiling battens in Air condition areas only |
| Type/Product | Fletcher Permastop Tropic double foil blanket | Fletcher Pink Baits Insulation |

| Property | A | B |
| --- | --- | --- |
|  | insulation or equally an approved product. |  |
| Location | Between roof sheeting and roof mesh | To extent ceiling above the lining. Protect baits from light fixtures |
| R-Value | 1.4 | 1.5 |
| Fire Retardant | Yes | Yes |
| Thickness (mm) | 60mm | 65mm |

0511B LINING

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide internal lining systems, as documented.

Performance

Requirement: Provide lining system with a surface that is:

- Resistant to impacts expected in use.

- Resistant to moisture encountered under expected environmental conditions.

- Free of irregularities.

- A suitable substrate for the nominated final finish.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

- 0182 Fire-stopping

  - INTERPRETATION Definitions

General: For the purposes of this work section the definitions given in AS/NZS 4491 and the following apply:

- Decorative overlaid wood panels: Particleboard or fibreboard with a bonded decorative finishing surface such as thermosetting resin (low pressure melamine), PVC film, paper foils or woodveneer.

- Dry processed fibreboard (MDF): A panel manufactured by bonding lignocellulosic fibres (derived from wood or other materials) with a synthetic resin adhesive and curing under heat and/or pressure.

- The panels are manufactured with a forming moisture content of less than 20%.

- Fibre cement sheet linings: Treated cellulose fibre in a matrix of cement and sand autoclaved sheet, sealed on one side.

. Sheets consisting of a decorative face and layers of fibrous sheet material (e.g. paper) impregnated with thermosetting resins and bonded together under heat and pressure of at least 5 MPa.

  - TOLERANCES Permitted deviations

Gypsum lining: To AS/NZS 2589 clause 4.2.2. Other lining: 4 mm from a 1.8 m straightedge. Substrates

Requirement: Plumb, level, in true alignment and to the lining manufacturer’s recommendations.

Timber, steel framing and battened masonry: To AS/NZS 2589 clause 4.2.

  - SUBMISSIONS Fire hazard properties

Requirement: Submit evidence of conformance to PRODUCTS, GENERAL, Fire hazard properties. Warranties

Requirement: Submit the following:

- As per manufacturers specification

Lining materials: Submit the manufacturer's published product warranties.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Substrate or framing before installation of linings.

- Finished surface of installation before applying:

. Sealer.

. Finish coatings or decorative papers.

- PRODUCTS

  - GENERAL

Fire hazard properties

Group number: To BCA Spec C1.10 and AS 5637.1.

Storage and handling

Requirement: Dry and undamaged lining stacked in pallets horizontally on a smooth, level surface. Prevent distortion or moisture ingress.

Timber or fibreboard panels: Store off the ground in a well-ventilated area.

Handling: Do not drag sheets across each other or across other materials. Protect edges, corners and surface from damage.

Acclimatization

Timber panels: Store on-site in final interior conditions for 2 to 3 weeks before installing. Do not install until the air conditioning system of the installation area is operating.

Marking

Identification: Marked to show the following:

- Manufacturer’s identification.

- Product brand name.

- Product type.

- Quantity.

- Product reference code and batch number.

- Date of manufacture.

- Material composition and characteristics such as volatility, flash point, light fastness, colour and pattern.

Certification

General: Label panels under the authority of a recognised certification program applicable to the product. Locate the label on faces or edges which will be concealed in the works.

  - ADHESIVES, SEALANTS AND FASTENERS Adhesives

For wallboards: Gunnable synthetic rubber/resin based mastic contact adhesive formulated for bonding flooring and wallboards to a variety of substrates.

Sealants

Acoustic sealant: Non-hardening sealant compatible with the materials to be sealed.

Fasteners

Steel nails: Hot-dip galvanized.

- EXECUTION

  - CONSTRUCTION GENERALLY Conditions

Commencement: Do not commence lining work until the building or installation area is enclosed and weathertight, and all wet trades have been completed.

Substrates or framing

General: Before fixing linings, check and adjust the alignment of substrates or framing, if necessary.

Battens

General: Fix at each crossing with structural framing members, to solid walls or ceiling support. Provide wall plugs in solid substrates.

Ceiling linings

General: Do not install until the timber roof structure is fully loaded for at least 14 days.

Accessories and trim

General: Provide accessories and trim as necessary to complete the installation.

Adhesives

General: Provide adhesive types appropriate for the purpose, and apply them so they transmit the loads imposed without causing discolouration of the finished surfaces.

  - PLASTERBOARD LINING Installation

Gypsum plasterboard and fibre reinforced gypsum lining: To AS/NZS 2589. Level of finish and jointing: To AS/NZS 2589 clause 3.1.

Supports

General: Install timber battens or proprietary cold-formed galvanized steel furring channels as follows:

- Where framing member spacing exceed the recommended spacing.

- Where direct fixing of plasterboard is not possible, due to the arrangement or alignment of the framing or substrate.

- Where the lining is the substrate for tiled finishes.

- Where required for penetrations for services, including mechanical grilles and lighting fixtures.

- If required to support fixtures.

Multiple sheet layers

Application: Fire-resistance rated and acoustic rated walls.

Joints: Fill and flush up all joints and fasteners in each layer and caulk up perimeters and penetrations before installing subsequent layers. Stagger all sheet joints by minimum 200 mm.

Joints

Flush joints: Provide recessed edge sheets and finish flush using perforated paper reinforcing tape. Butt joints: Make joints over framing members or provide back blocking.

External corner joints: Make joints over metallic-coated steel corner beads.

Dry joints: Provide square edged sheet and finish with a PVC-U joining section.

Control joints: Provide purpose-made metallic-coated control joint beads at not more than 12 m centres in walls and ceilings and to coincide with structural control joints.

Wet areas: Install additional supports, flashings, trim and sealants as required.

Joints in tiled areas: Do not apply a topping coat after bedding perforated paper tape in bedding compound.

  - FIBRE CEMENT LINING Supports

General: Install timber battens or proprietary cold-formed galvanized steel furring channels as follows:

- Where framing member spacing exceed the recommended spacing.

- Where direct fixing of fibre cement is not possible, due to the arrangement or alignment of the framing or substrate.

- Where the lining is the substrate for tiled finishes.

- Where required for penetrations for services, including mechanical grilles and lighting fixtures.

- If required to support fixtures.

Installation

Joints and layout: Run sheets across the framing members. In flush jointed applications, stagger end joints in a brick pattern and locate them on framing members, away from the corners of large openings. Provide supports at edges and joints.

Fixing

Timber framed construction: Nail only or combine with adhesive. Steel framed construction: Screw only or combine with adhesive.

Wall framing: Conform to the following:

- Do not fix to top and bottom plates or noggings.

- In tiled areas: Provide an extra row of noggings immediately above wall-to-floor flashings. Fix sheet at 150 mm centres to each stud and around the perimeter of the sheet.

Masonry wall construction: Conform to the following:

- Direct fixing: Adhesive fix to the masonry except where lining forms a substrate for tiled finish.

- Furring channels: Fix using screw and/or adhesive.

Ceilings: Fix using screw and/or adhesive to ceiling furring members. Do not fix sheets directly to the bottom chords of trusses.

- Ceiling battens: Fix at 600 mm maximum centres. Wet areas: Do not use adhesive fixing alone. Multiple sheet layers

Application: Fire-resistance rated and acoustic rated walls.

Joints: Fill and flush up all joints and fasteners in each layer and caulk up perimeters and penetrations before installing subsequent layers. Stagger all sheet joints by minimum 200 mm.

Joints

Joint width:

- Butt joints: 1 to 2 mm.

- Expressed joints: 10 mm maximum.

Joint backing for expressed joints: Black self-adhesive polyurethane tape.

Flush joints: Provide recessed edge sheets and finish flush using perforated paper reinforcing tape. External corner joints: Make joints over metallic-coated steel corner beads.

Dry joints: Provide square edged sheet and finish with a PVC-U joining section.

Control joints: Provide control joints to coincide with structural control joints and as follows:

- Walls: ≤ 7.2 m centres.

- Ceilings: To divide into bays not larger than 10.8 x 7.2 m.

- Soffit linings: To divide into bays not larger than 4.2 x 4.2 m or 5.6 x 3.6 m.

- Control joint beads: Purpose-made metallic-coated.

- Support: Provide framing parallel to the joint on each side. Do not fix the lining to abutting building surfaces.

Wet areas: Provide additional supports, flashings, trim and sealants as required.

Joints in tiled areas: Bed perforated paper tape in bedding compound. Do not apply a topping coat.

- Control joints: Not more than 4.2 m centres and space to suit joints required in tiling.

- Internal corners: Reinforce with metallic-coated steel angles. In corners subject to continuous moisture, flash over the angle and under the sheeting with continuous bitumen coated aluminium flashing.

.

  - TRIM AND ACCESSORIES General

Requirement: Provide trim such as beads, mouldings and stops to make neat junctions between lining components, finishes and adjacent surfaces.

Proprietary items: Provide complete with installation accessories.

Timber and MDF trim: Fix using full length so that trim is secure and without movement. Where nail or screw fixings are used, make sure fastener finishes sufficiently below face of trim so that stopping piece finishes flush with the face.

  - COMPLETION General

Damaged or marked lining and components: Replace.

Exposed surfaces: Touch up shop applied finishes and restore damaged or marked areas. Timber panels: If appearance is not uniform, replace panels.

Cleaning: Clean completed surfaces to remove irregularities and leave panels smooth and clean, to the manufacturer’s recommendations. If required, sand with fine paper to remove irregularities and refinish panel surface.

- Debris and unused material: Remove from site.

Warranties

Requirement: At practical completion, submit warranties against defective materials and installation.

101 | P a g e

0525B CUBICLE SYSTEMS

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide cubicles, as documented.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - TOLERANCES General

Deviation (from true grid lines and planes): 1:1000 to a maximum of 3 mm. Misalignment (of adjoining surfaces at panel junctions): 1 mm.

Panel thickness: ± 0.5 mm.

Length and width: 0.1% of the dimension or 0.5 mm, whichever is the greater.

Flatness, twist, winding and bow: 1 mm deviation from a 2.4 m straightedge placed in any position. Maximum deviation of edges from the intended true line: ± 1 mm.

  - SUBMISSIONS

Samples

General: Submit samples as follows:

- Each selected panel and door finish, at least 300 x 300 mm, with associated selected edge strips and trims.

- All hardware and metal components in the selected finish. Warranties

Cubicle system assemblies: Submit the manufacturer’s published product warranties.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Set-out before installation.

- Completion of installation.

- PRODUCTS

  - GENERAL Marking

Identification: Marked to show the following:

- Manufacturer’s identification.

- Product brand name.

- Product type.

- Quantity.

- Product reference code and batch number.

- Date of manufacture.

- Material composition and characteristics such as volatility, flash point, light fastness, colour and pattern.

  - PRODUCT SYSTEMS

High pressure decorative laminate (HPDL) and panels and doors

Material: Compact high pressure decorative laminate panels with an integral surface finish and edges sealed by the manufacturer.

Edge: Factory prefinished square cut, ground smooth and arises, and oiled by the manufacturer.

  - ACCESSORIES Suspension beam

General: For suspended systems, provide a suspension beam consisting of a galvanized mild steel channel, located immediately above the ceiling framing along the line of the partition fronts. Build the ends into masonry structure or provide end fixings to the structure, as necessary, to transfer the load. Drill the bottom flange of the channel for the partition fixing bolts.

Hardware

Fixing hardware: Bolts, dowels, brackets, standards, cappings and stabilizing bars supplied to complete the cubicle assembly.

Door furniture: As documented in the Cubicle schedule.

- EXECUTION

  - PANELS Acclimatization

General: Condition wood-based product components in the anticipated environment for two weeks before assembly.

Manufactured cubicle system installation

Assembly: Attach divisions and nibs to walls and fronts with purpose-made proprietary fixings. Cut nibs and divisions that abut walls, as required, so that assembly is plumb. Seal edges as recommended by the manufacturer.

Floor mounted/overhead braced type: Fix fronts to the floor with proprietary fittings and at the top to a metal channel headrail, supplied as part of the system. Run headrail across the fronts and fix to the walls at each end. Form the channel into a box section over doorways by snapping in a mating channel insert.

Heads of openings: Fix stabilising head channels by screwing to the top of the partitions. Provide an infill strip to the channel across the opening.

Ceiling hung type: Hang the fronts from a suspension beam with attachments incorporating a means of height adjustment, supplied as part of the system.

Freestanding type: Fix fronts to the floor with proprietary fittings.

- SELECTIONS

| Property | Public Toilets and DMO Toilets |
| --- | --- |
| Manufacturer | Laminex Partitioning System or equally an approved product. |
| Product name | Laminex - FOAB |
| Cubicle function | Toilet Partition |
| Type | Foot Assembly with Overhead Brace and Privacy strip |
| Front base fixing | Foot Assembly - Concealed fixing anodised aluminium adjustable legs |
| Material and thickness: Division (mm) | Laminex Multipurpose Compact Laminate 13mm (with or without privacy strips) |
| Material and thickness: Front and nib (mm) | Laminex Multipurpose Compact Laminate 13mm (with or without privacy strips). Predrill holes to suit hardware. |

SCHEDULES Cubicle schedule

| Property | Public Toilets and Office Toilets |
| --- | --- |
| Panel Finish: Type | Protect PLUS with Polished edges. |
| Foot & Framing Finish | Foot: Clear Anodised foot with Rubber base. Channels and Brackets: Clear Anodised Overhead brace: Clear Anodised |
| Material | Aluminium |
| Hardware Finish: Type | Aluminium |
| Finish: Colour | Clear Anodise |
| Indicator bolts | Indicator bolt and turn bolt |
| Hinges | Gravity Hinge |
| Coat hook/door stop | Buffer and Rubber tipped coat hook |
| Not standard to the cubicle product | Note widths and depths may not be standard. |

0551B JOINERY

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide joinery, as documented but not limited to the listed items below. The contract shall supply and install the following items as listed and itemise each unit within its tender.

    - MJCA Office Reception desk c/w Drawers

    - MJCA Office Reception Counter

    - Public Bench Seat furniture

    - MJCA Office Built in desks and c/w Drawers

    - Courtroom Furniture

    - Strong room shelving

    - Kitchen Bench, shelving and ALL built in appliances including Fridge/Freezer

    - Kitchenettes

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - STANDARDS General

Tactile indicators: To AS/NZS 1428.4.1.

  - TOLERANCES General

Requirement: Fabricate and install joinery items to substrates undamaged, plumb, level, straight and free of distortion.

Tolerances table

| Property | Tolerance |
| --- | --- |
| Plumb and level | 1:800 |
| Offsets in flush adjoining surfaces | 0.5 mm |
| Offsets in revealed adjoining surfaces | 2 mm |
| Alignment of adjoining doors | 0.5 mm |
| Difference in scribe thickness for joinery items centred between walls | 2 mm |
| Doors centred in openings | 0 |
| Joints in finished surfaces | 0 |

  - SUBMISSIONS

Operation and maintenance manuals

General: Submit manufacturer’s published recommendations for service use. Products and materials

Manufacturer's data: Submit manufacturer's data.

Proprietary items: Submit the manufacturer’s standard drawings and details showing:

- Methods of construction.

- Assembly and fixing, with dimensions and tolerances.  Shop drawings

General: Submit shop drawings to a scale that best describes the detail, showing the following:

- Overall dimensions.

- Materials, thicknesses and finishes of elements including doors, divisions, shelves and benches.

- Type of construction including mitre joints and junctions of members.

- Hardware type and location.

- Temporary bracing, if required.

- Procedures for shop and site assembly and fixing.

- Locations of benchtop joints.

- Locations of sanitary fixtures, stoves, ovens, sinks, and other items to be installed in the units.

- Relationship of fixture to adjacent building elements.

- Details of fabrication involving other trades or components.

- Proposals for the break-up of large items as required for delivery to the site.

- Proposed method of joining the modules of large items. Timing: Before fabrication.

Subcontractors

General: Submit names and contact details of proposed suppliers and installers.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Shop fabricated or assembled items ready for delivery to the site.

- Openings prepared to receive assemblies.

- Site erected assemblies on completion of erection, before covering up by cladding and encasing.

- Surfaces prepared for, and immediately before, site applied finishes.

- Completion of installation.

- PRODUCTS

  - DELIVERY AND STORAGE General

Requirement: Deliver joinery units to site in unbroken wrapping or containers and store so that its moisture content is not adversely affected. Do not store in areas of wet plaster. Keep storage time to a minimum by delivering items only when required for installation.

  - JOINERY MATERIALS AND COMPONENTS Visible work

Clear finished timber and veneer: Make sure all visible surfaces are free of branding, crayon or chalk marks and of blemishes caused by handling.

Joinery  timber

Hardwood for trim: To AS 2796.1. Softwood for trim: To AS 4785.1.

Finished sizes of milled timbers: Not less than the documented dimensions unless qualified by a term such as nominal, out of or ex to which industry standards for furnished sizes apply.

Melamine overlaid particleboard: Particleboard overlaid on both sides with low pressure melamine. Dry-processed fibreboard (including medium density fibreboard)

Standard: To AS/NZS 1859.2.

Melamine overlaid medium density fibreboard: Medium density fibreboard (STD MDF) overlaid on both sides with low pressure melamine.

  - JOINERY ASSEMBLIES Plinths

Material: Select from the following:

- High moisture resistant medium density fibreboard. Thickness: 18 mm.

Fabrication: Form up with front and back members and full height cross members at not more than 900 mm centres.

Finish: Plywood H3.2

- Colour: Hoop Pine. Fasteners: Conceal with finish.

Installation: Scribe to floor and secure to wall to provide level platform for carcasses. Carcasses

Material: Select from the following:

- Overlaid high moisture resistant particleboard.

- Overlaid high moisture resistant medium density fibreboard. Thickness: 18 mm.

Joints: Select from the following:

- Screws and glue.

Adjustable shelves: Support on proprietary pins in holes bored at equal centres vertically.

- Spacing: 32 mm.

Finish: Plywood H3.2 with hoop Pine Veneer Fasteners: Conceal with finish.

Installation: Secure to walls at not more than 600 mm centres.

Drawer fronts and doors Material: Select from the following:

- Melamine overlaid high moisture resistant particleboard.

- Melamine overlaid high moisture resistant medium density fibreboard. Thickness: 16 mm.

Door size: Not exceeding 1.5 m2 on face, with 2400 mm maximum height and 900 mm maximum width.

Drawer fronts: Rout for drawer bottoms. Finish: Ply grade A H3.2

Colour: Hoop Pine veneer

Edging: 2mm ABS Drawer backs and sides Material: Plywood H3.2 Thickness: 12 mm.

Colour: Hoop Pine

Installation: Mitre corners leaving outer skin of foil intact, finish with butt joints, glue to form carcass and screw to drawer front. Rout for drawer bottoms.

Drawer bottoms

Material: 10mm Prefinished HMR board with 2mm ABS. Thickness: 10 mm.

Colour: White

Drawer and door hardware

Hinge types: Concealed metal hinges with the following features:

- Nickel plated.

- Adjustable for height, side and depth location of door.

- Self-closing action.

- Hold open function. Hold open angle: 110

Piano hinges: Chrome plated steel, extending full height of doors. Slides: Metal runners and plastic rollers with the following features:

- 30 kg loading capacity.

- Closure retention.

- White thermoset powder coating or nickel plated. Pulls: Taper Bow Satin Finish

  - WORKING SURFACES

Exposed edges: Extend laminate over shaped nosing, finishing more than 50 mm back on underside. Splay outside corners at 45°.

Balance underside: Extend laminate to the undersides of benchtops.

Installation: Scribe to walls. Fix to carcass at least twice per 600 mm length of benchtop.

Joint sealing: Fill joint with sealant matching finish and clamp with proprietary mechanical connectors.

- EXECUTION

  - JOINERY General

Joints: Provide materials in single lengths whenever possible. If joints are necessary, make them over supports.

Framing: Frame and trim where necessary for openings, including those required by other trades. Concealed surfaces: Prime surfaces concealed by substrates.

Deficiencies: Examine joinery units for completeness and remedy deficiencies.

Substrate: Damp clean and vacuum substrate surfaces that will be permanently concealed. Openings: Provide openings for the following: Under bench fridges

Acclimatization

General: Acclimatize the joinery items by stacking in the in-service conditions with air circulation to all surfaces after the following are complete:

- Air conditioning operational.

- Lighting operational.

- Site drainage and stormwater works are complete.

- Space fully enclosed and secure.

- Wet work complete and dry. Accessories and trim

General: Provide accessories and trim necessary to complete the installation.

Fasteners

Visibility: Do not provide visible fasteners except in the following locations:

- Inside cupboards and drawer units.

- Inside open units, in which case provide proprietary caps to conceal fixings.

Visible fasteners: Where fasteners are unavoidable on visible joinery faces, sink the heads below the surface and fill the sinking flush with a material compatible with the surface finish. In surfaces which are to have clear or tinted finish, provide matching wood plugs showing face (not end) grain. In surfaces which are to have melamine finish, provide proprietary screws and caps finished to match.

Fix joinery units to substrates as follows:

- Floor mounted units: 600 mm centres maximum.

- Wall mounted units: To each nogging and/or stud stiffener.

Fasteners: Screws with washers into timber or steel framing, or masonry anchors. Adhesives

General: Provide adhesives to transmit the loads imposed and for the rigidity of the assembly, without

causing discolouration of finished surfaces. Finishing

Junctions with structure: Scribe, plinths, benchtops, splashbacks, ends of cupboards, kickboards and

returns to follow the line of structure.

Joints: Scribe internal and mitre external joints.

Edge strips: Finish exposed edges of sheets with edge strips which match sheet faces.

Matching: For surfaces which are to have clear or tinted finish, arrange adjacent pieces to match the grain and colour.

Hygiene requirements: To all food handling areas and voids at the backs of units in all areas, seal all carcass and junctions wall/floor, and cable and pipe entries with silicone beads for vermin proofing. Apply water resistant sealants around all plumbing fixtures and make sure sealants are fit for purpose.

Benchtops

Installation: Fix to carcass at least twice per 600 mm length of benchtop.

Joint sealing: Fill joints with sealant matching the finish colour and clamp with proprietary mechanical connectors.

Edge sealing: Seal to walls and carcasses with a sealant, which matches the finish colour. Splashbacks

Refer to Tilings section for splashbacks.

Labelling

General: Permanently mark each unit of furniture with the manufacturer's name, on an interior surface.

.

  - COMPLETION Protection

Timber treads: Provide full timber or plywood casing. Cleaning

Temporary coatings: On or before completion of the works, or before joining up to other surfaces,

remove all traces of temporary protective coatings.

Requirement: Remove all dust, marks and rubbish from all surfaces and internal spaces. Clean and polish all self-finished surfaces such as anodised and powder coated metals, sanitary ware, glass, tiles and laminates.

- CONSIDERED SELECTIONS

  - LOCKER & BENCH SEAT SYSTEMS

| Properties |  |  |
| --- | --- | --- |
| Product | Compact Laminex Door | Compact Lamniex Bench system with 13mm compact laminate seats fixed to proprietary frames or equally an approved product. |
| Material | Laminex Multipurpose Compact Laminate 13mm | Laminex Multipurpose Compact Laminate 13mm |
| Locks | Key Locks - All key different | Feet and bracket system at 400 crs, Clear anodised aluminium. |
| Hinge | 180 deg hinge |  |
| Colour | Charcoal | An Equally Approved product may be substituted for the above bench seat system. |
| Fixing | Allow to fix to block wall with 6 M12 Dome head stainless steel bolts per Bank. Allow for 25mm thick trims around perimeter of lockers and scribe to wall to close off any gaps. |  |

  - FIXED FURNITURE

| Properties | Reception Counter |
| --- | --- |
| Product/Type | 40mm thk Solid Teak with bull Nose edge scribed to the wall face and continued into the depth of the wall. |
| Material | 40mm with a 150mm extend edge beyond the wall face and at ends. |
| Colour | Clear Polyurethane Finish -4 coats |
| Support | Allow for Triangular supports fixed to wall face with concealed screws. |
| Fixing | Allow for to countersink and Dyna-bolt counter into block 150mm from both ends and to the centre. |

| Properties | Reception Desk | Properties | Office Desk |
| --- | --- | --- | --- |
| Product/Type | Carter holt HarveyPlyH3 with A and B Hoop Pine facing or equally an approved product. | Product/Type | Carter holt Harvey Ply H3 with A and B Hoop Pine facing or equally an approved product. |
| Material | 800mm wide x 25mm thk | Material | 25mm thk fixed to steel frame with concealed screws from underside. |
| Colour | Clear finish to tops and Matt Black to Side, rear and backing panels including drawers | Colour | Clear Finish to tops |
| Support | Allow for end panels and backing panel. To be screw fixed together using Haffele fastening systems | Support | Low wall – 1000mm high walls with 100x50 @600crs stud framing with 12mm A/B plywood screw fixed to all sides. Set frames 50mm back from front edge of the tops. Tops fixed to - 50x50x3mm SHS welded frame. Allow for perimeter frame under bench welded to SHS legs with adjustable feet. Paint black and allow to screw fix back into wall framing. Silicone seal around the edges. |
| Accessories | Allow to build in drawer unit with 3 drawers using Blum runners, handles and locks. Allow for desk cut outs for service reticulation. | Accessories | Tops to have cut outs for data systems. Allow for additional cut outs to access power and data reticulated below bench. |
| Edging | Allow for 2mm black ABS edging | Edging | Allow for 2mm black ABS edging |

| Properties | MJCA Offices (Manager & Judges’ Closed Office) |  |  |
| --- | --- | --- | --- |
| Product/Type | Carter holt Harvey Ply with A and B Hoop Pine facing or equally an approved product. |  |  |
| Material | 800mm wide desk tops x 25mm thk with workspace and returns |  |  |
| Colour | Clear finish to tops and Matt Black to Side, rear and backing panels including drawers |  |  |
| Support | Allow for end panels and backing panel. To be screw fixed together using Haffele fastening systems |  |  |
| Accessories | Allow to build in drawer unit with 3 drawers using Blum runners, handles and locks. Allow for levelling feet to each unit. |  |  |
| Edging | Allow for 2mm black ABS edging |  |  |

| Properties | Kitchen (Kitchen Bench) |  |  |
| --- | --- | --- | --- |
| Top | 18mm Corian Glacier White Top or equally an approved product. |  |  |
| Carcase | 18mm Carter holt Harvey Ply with A and B Hoop Pine facing |  |  |
| Material | Refer Above |  |  |
| Colour | Clear finish to tops and Matt Black to Side, rear and backing panels including drawers |  |  |

| White Goods | Kitchens |
| --- | --- |
| Fridge/Freezer | Fisher and Pykle Active Smart Fridge –Top and Freezer 447L, EZKleen Stainless steel or equally an approved product. Left Hand hinge |
| Microwave | Panasonic Microwave NN-ST7855 Stainless Steel Finish or equally an approved product. |
|  | Further Clarification of the Architectural Specification: Joinery White Goods: Supply new Fridge / Freezer and Microwave for the Courthouse Kitchen (Tea Room) and also for the Judges Residence Kitchen; Hot Water Boiler System for: - Courthouse Kitchen (Tea Room) - Judges Residence Kitchen |

0552B METALWORK - FABRICATED

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide metal fixtures, as documented. Performance

Requirements:

- Undamaged, plumb, level and straight.

- Free of surface defects or distortions.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - STANDARDS General

Access for maintenance: To AS 1657. Tactile indicators: To AS/NZS 1428.4.1.

  - SUBMISSIONS

Operation and maintenance manuals

General: Submit manufacturer’s published recommendations for service use. Products and materials

Manufacturer’s data: Submit manufacturer's product data including standard drawings and details.

Proprietary items: Submit the manufacturer’s standard drawings and details showing:

- Methods of construction.

- Assembly and fixing, with dimensions and tolerances.

Stainless steel: For each batch of stainless steel supplied to the works, submit the certificate of conformance or test certificate to the applicable standard, as documented.

Stainless steel welding : Before fabrication commences, submit evidence of qualification of the welding procedure by testing to AS/NZS 1554.6 clause 4.7 or evidence of prequalification to AS/NZS 1554.6 clause 4.13.

Shop drawings

General: Submit shop drawings to a scale that best describes the detail, showing the following information:

- Overall dimensions.

- Details of fabrication and components.

- Details of fabrication involving other trades or components.

- Information necessary for site assembly.

- Proposals for the break-up of large items as required for delivery to the site.

- Proposed method of joining the modules of large items. Subcontractors

General: Submit names and contact details of proposed suppliers and installers.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Shop fabricated or assembled items ready for delivery to the site.

- Commencement of shop or site welding.

- Site erected assemblies on completion of erection, before covering up by cladding and encasing.

- Steel surfaces prepared for, and immediately before, site applied finishes.

- PRODUCTS

  - MATERIALS AND COMPONENTS Metals and components

Performance: Provide metals in sections of strength and stiffness suited to their required function, finish and method of fabrication.

- EXECUTION

  - CONSTRUCTION GENERALLY Metals

Performance: Provide metals capable of transmitting the loads imposed and sufficient for the rigidity of the assembly without causing deflection or distortion of finished surfaces.

Incompatible metals: Separate using concealed layers of suitable materials in appropriate thicknesses. Fasteners

Performance: Provide non-galvanic corrosion fasteners.

Materials: Provide fasteners in materials of mechanical strength and corrosion resistance at least equal to that of the lowest resistant metal joined.

To copper and copper alloys: Provide copper or copper-alloy fixing devices only.

To aluminium and aluminium alloys: Provide aluminium alloy or non-magnetic stainless steel fixing devices only.

To stainless steel: Provide appropriate stainless steel materials only. Fabrication

Workshop: Fabricate and pre-assemble items in the workshop wherever practicable.

Edges and surfaces: Keep clean, neat and free from burrs and indentations. Remove sharp edges without excessive radiusing.

Tube bends: Form bends in tube without visibly deforming the cross section. Colour finished work: Match colours of sheets, extrusions and heads of fasteners. Thermal movement: Accommodate thermal movement in joints and fastenings.

Tolerances: ± 2 mm from design dimensions. Joints

General: Fit joints to an accuracy appropriate to the class of work. Finish visible joints made by

welding, brazing or soldering using grinding, buffing or other methods appropriate to the class of work, before further treatment.

Self-finished metals: Free of surface colour variations, after jointing. Joints: Fit accurately to a fine hairline.

Marking

General: Provide suitable and sufficient marks or other means for identifying each member of site- erected assemblies, and for their correct setting out, location, erection and connection. Mark bolted connections to show the bolting category. Do not mark stainless steel by notching.

Splicing

General: Provide structural members in single lengths.

  - WELDING AND BRAZING Welding

Quality: Provide finished welds which are free of surface and internal cracks, slag inclusion, and porosity.

Site welds: Avoid site welding wherever possible. If required, locate site welds in positions for down hand welding.

Butt weld quality level: Not inferior to the appropriate level recommended in AS/NZS 1554.1 Section 6, AS/NZS 1554.6 Section 6 or AS 1665 Appendix A, as appropriate.

  - STAINLESS STEEL FABRICATION Welding stainless steel

Certification of welders: To AS 1796.

  - PIPE RAIL BALUSTRADES Fabrication

Method: Welding.

Joints: Produce smooth unbroken surfaces at joints. Scribe the joints between posts and rails. Make end-to-end joints over an internal sleeve.

Bends: Make changes of direction in rails by evenly curved pipe bends.

Free ends: Seal the free ends of pipes with fabricated or purpose-made end caps. Fixing to structure

General: Provide fabricated predrilled or purpose-made brackets or post bases, and attach the piping

to the building structure with fixings, including bolts into masonry anchors, and coach screws or bolts into timber, of metal compatible with the piping.

Galvanizing

General: If possible, complete fabrication before galvanizing; otherwise apply a zinc-rich primer to affected joint surfaces.

  - COMPLETION Cleaning

Temporary coatings: On or before completion of the works, or before joining up to other surfaces, remove all traces of temporary coatings used as a means of protection.

- SELECTIONS

Steel balustrade components schedule

| Member | Sizes (mm) | Spacing | Material/Finish (All Steel is grade 316 SS) |
| --- | --- | --- | --- |
| Posts | 40 mm x 2.6 SCHS | 1200crs max | Stainless steel- Satin Finish |
| Handrail | 40 mm x 2.6 CHS | Refer detail. Allow for 10mm rod stubs and weld Handrail to top rail | Stainless steel- Satin Finish |
| Top and Bottom rail | 25 mm rod | Refer detail – welded to post and balusters | Stainless steel- Satin Finish |
| Balusters | 20 diameter Rod | 100mm welded top and bottom rail | Stainless steel- Satin Finish |
| Base Plate | Allow for welded base plate with 2 M16 chemset into concrete. |  |  |
| Balustrade to Structural post | Allow for 6mm end plate to Top and Bottom rails at both ends. Fix rails to Steel veranda posts with 2 x M8 Hollo bolt 40mm apart. |  |  |

Stair handrails:

| Member | Sizes (mm) | Brackets | Material/Finish |
| --- | --- | --- | --- |
| Handrail c/w with extensions and turn downs as per AS1428.1 | 40 mm x 2.6 CHS 316 SS | Allow for Stainless steel metal brackets at 1200 crs bolted to block walls | Stainless steel- Satin Finish |

| Member | Sizes (mm) | Brackets | Material/Finish |
| --- | --- | --- | --- |
|  |  | with M12 Dome head bolts. |  |

External Safety Rail

| Member | Sizes (mm) | Brackets | Material/Finish |
| --- | --- | --- | --- |
| Handrail c/w with extensions and turn downs as per AS1428.1 | 40 mm x 2.6 CHS 316 SS | Allow for Stainless steel metal brackets at 1200 crs bolted to block walls with M12 Dome head bolts. | Stainless steel- Satin Finish |

| Member |  | Material/Finish |
| --- | --- | --- |
| Grab Rail 1 - AWC | Caroma VITRU comfort Grab rail H 960mm x V 600mm fixed to wall as per manufactures specification. Right hand transfer. Refer to plan for WC location and transfer position. 45 deg bend Rails acceptable. | Stainless steel- Satin Finish |
| Grab rail 2 - AWC | Caroma VITRU comfort Grab rail H 450mm fixed to wall as per manufactures specification. | Stainless steel- Satin Finish |

| Member |  | Material/Finish |
| --- | --- | --- |
| Grab rail 3 – Ambulant Toilet | Caroma VITRU comfort Grab rail H 450mm x 450 90 bend Rh and LH fixed to toilet partition per manufactures specification. | Stainless steel- Satin Finish |
| Shelve- AWC | Rynatt or similar B295 x 14 Shelf (Suits Australian Standards AS1428.1-2009) | Stainless steel- Satin Finish |

0611 RENDERING AND PLASTERING

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide plaster finishes as documented. Performance

Requirements:

- Resistant to impacts expected in use.

- Free of irregularities.

- Consistent in texture and finish.

- Firmly bonded to substrates for the expected life of the application.

- Without obvious shrinkage cracks.

- As a suitable substrate for the nominated final finish.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - INTERPRETATION Abbreviations

General: For the purposes of this work section the following abbreviations apply:

- CRF: Cement render – finish.

- CRM: Cement render – medium.

- CRS: Cement render – stronger.

- CRW: Cement render – weaker.

- GPF: Gypsum plaster – finish. Definitions

General: For the purposes of this work section the following definitions apply:

- Base coat: A plaster coat applied before the application of the finish coat.

- Bonding treatment: A treatment of a substrate which improves adhesion of a plaster system.

- Finish coat (rendering and plastering): The final coat of a coating system.

- Finishing treatment (plastering): The treatment applied to a finish coat which may include processes and results.

- Plaster: A mixture of binders, aggregate and water which is applied to substrates in a plastic state and dries and cures to a hard surface which may subsequently be decorated:

. Cement plaster: Contains Portland cement as the principal binder.

. Gypsum plaster: Contains hydrated or anhydrous calcium sulfate as the principal binder.

- Plastering: The process of coating the framing or solid surfaces of a building with a plastic material which hardens and then may be decorated or remain self-finished.

- Plastering system: One or more coats of plaster and associated treatments comprising some or all of the following in sequence:

. Base coat 1 or 2.

. Bonding treatment.

. Finish coat.

. Finishing treatment.

- Render, rendering: plaster, plastering, usually single coat and usually cement:lime:sand.

- Substrate: The surface to which a material or product is applied.

| Description | Alignment | Tolerance |
| --- | --- | --- |
| Walls and other vertical structures | Vertical | 6 mm in 2400 mm |
| Reveals sides | Vertical | 3 mm in 1800 mm |
| Reveals head up to 1800 mm | Horizontal | 3 mm in 1800 mm |
| Reveals head over 1800 mm | Horizontal | 5 mm max |
| Reveals, piers, beams, wall stop ends up to 300 mm | Square | 3 mm max |
| Reveals, piers, beams, wall stop ends over 300 mm | Square | 5 mm max |
| Radius of corners | Round | Should not vary by more than ± 10% over the length of the arris. |

TOLERANCES Tolerances table

  - SUBMISSIONS Samples

Plaster systems: Prepare prototypes of each plaster system complete with beads and other embedded items:

- Size: 1200 x 2400 mm.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Prototypes ready for inspection.

- Substrates immediately before applying base coats.

- Finish treatments before decoration.

- PRODUCTS

  - MATERIALS AND COMPONENTS Accessories

Beads: Provide metal proprietary sections manufactured for fixing to substrates and/or embedding in the plaster to form and protect plaster edges and junctions.

Metal lath: Provide a proprietary product manufactured from raised expanded metal for use with plaster:

- Mass/unit area: 1.84 kg/m2 or greater.

- Material thickness: 0.70 mm or greater.

- Mesh size: 9.5 x 28.6 mm.

Metallic-coatings to AS 1397: For beads or lath in cement plaster: To the Corrosion resistance and durability table.

Admixtures

Plasticisers or workability agents: Do not use in cement plasters. Aggregates

Sand: Fine, sharp, well-graded sand with a clay content between 1% and 5%, and free from

efflorescing salts.

Sand grading for base coat plaster table

| Sieve size | Percent passing |  |
| --- | --- | --- |
|  | Minimum | Maximum |
| 4.75 mm | 100 | 100 |

| Sieve size | Percent passing |  |
| --- | --- | --- |
|  | Minimum | Maximum |
| 2.36 mm | 90 | 100 |
| 1.18 mm | 60 | 90 |
| 600 µm | 35 | 70 |
| 300 µm | 10 | 30 |
| 150 µm | 0 | 5 |
| 75 µm | 0 | 3 |

Plaster for autoclaved aerated concrete

General: Provide a proprietary product manufactured for use with the wall system. Bonding products

General: Provide proprietary products manufactured for bonding cement-based plaster to solid

substrates. Cement

Standard: To AS 3972.

Type: GP.

Colouring products

General: Provide proprietary products manufactured for colouring cement plaster.

Integral pigment proportion: 5% maximum weight of cement. Cornice cement

General: Provide a proprietary product manufactured for use with the cornice.

Cornices

Cast plaster: Proprietary item. Corrosion resistance and durability

Compliance: To the Corrosion resistance and durability table or provide proprietary products with

metallic and/or organic coatings of equivalent corrosion resistance and as follows:

- Galvanize: To AS/NZS 4680.

Corrosion resistance and durability table

| Atmospheric corrosivity category to AS 4312 | Metal lath, beads and embedded items | Minimum cement content (mix type) above damp-proof course |
| --- | --- | --- |
| C1 and C2 | Galvanize after fabrication 300 g/m2 Stainless 316 | CRW |
|  | Powder coated aluminium | CRM |
| C3 | Stainless 316 Powder coated aluminium | CRM |
| C4 and T1 | Stainless 316 Powder coated aluminium | CRS |
| 1 Avoid organic coating in Category T zones. |  |  |

Curing products

General: Provide proprietary products manufactured for use with the plaster system. Gypsum plaster

General: Provide a proprietary product containing calcium sulfate hemihydrate with additives to modify

setting. Lime

Limes for building: To AS 1672.1.

Lime putty

General: Prepare lime putty as follows:

- Stand dry hydrate of lime to AS 1672.1 and water for 24 hours or more without drying out.

- Stand quicklime and water for 14 days or more without drying out. Mixes

General: Select a mix proportion to suit the conditions of application.

Measurement: Measure binders and sand by volume using buckets or boxes. Do not allow sand to bulk by absorption of water.

Plaster mixing: Machine mix for 3 to 6 minutes.

Strength of successive coats: Make sure successive coats are no richer in binder than the coat to which they are applied.

Mix proportion table – Cement render, by volume

| Mix type |  | Substrate | Upper and lower limits of proportions by volume |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cement | Lime | Sand |
| Single or multi- coat systems with integral finishing treatments Base coats in multi-coat systems with cement or gypsum finishes | CRS | Dense and smooth concrete and masonry | 1 1 | 0 0.5 | 3 4.5 |
|  | CRM | Regular clay or concrete masonry | 1 1 | 0.5 1 | 4.5 6 |
|  | CRW | Lightweight concrete masonry and other weak substrates | 1 1 | 1 2 | 6 9 |
| Second coat - Internal | CRF | Cement render base coats | 1 1 | 1 2 | 6 9 |
| Second coat - External | CRF | Cement render base coats | 1 1 | 1 2 | 5 6 |

Mix proportion table – Gypsum finish coat, by volume

| Mix type |  | Substrate | Upper and lower limits of proportions by volume |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Gypsum | Cement | Lime putty | Sand |
| Gypsum | GPF | Cement | 1 | - | 1.5 | - |
| finish coats |  | render base | 1 | - | 2 | - |
|  |  | coats |  |  |  |  |

Mix proportion table – Gypsum finish coat, by weight

| Gypsum plaster (kg) | Lime putty (kg) |
| --- | --- |
| 17 | 25 |
| 34 | 50 |
| 51 | 75 |

Control joint products

General: Provide proprietary products manufactured for use with the plastering system and to accommodate the anticipated movement of the substrates and/or the plaster.

Water

General: Clean and free from any deleterious matter.

- EXECUTION

  - PREPARATION Substrates

General: Provide substrates as follows:

- Clean and free from any deposit or finish which may impair adhesion of plaster.

- If framed or discontinuous, support members in full lengths without splicing.

- If solid or continuous, remove excessive projections and fill voids and hollows with plaster stronger than the first coat and not weaker than the substrate.

Absorbent substrates: If suction is excessive, control it by dampening without over-wetting, and do not plaster substrates showing surface moisture.

Dense concrete: If not sufficiently rough to provide a mechanical key, roughen by scabbling or the like to remove 2 mm of the laitance and expose the aggregate before applying a bonding treatment.

Painted surfaces: Remove paint and hack the surface at close intervals.

Untrue substrates: If the substrate is not sufficiently true for conformity with the thickness limits for the plaster system, or has excessively uneven suction resulting from variations in the composition of the substrate, apply additional coats without exceeding the thickness limits for the substrate or system.

Beads

Location: Fix beads as follows:

- Angle beads: At all external corners.

- Drip beads: At all lower terminations of external plaster.

- Beads for control of movement: At all control joints.

- Stop beads: At all terminations of plaster and junctions with other materials or plaster systems. Joints in beads: Provide dowels to maintain alignment.

Mechanical fixing to substrate: ≤ 300 mm centres. Bonding treatment

General: If bonding treatment is required, throw a wet mix onto the background. Mix proportions to the

following:

- Cement plaster (cement:sand): 1:2.

- Gypsum plaster (gypsum:sand): 1:2.

Curing: Keep continuously moist for 5 days or more and allow to dry before applying plaster coats. Thickness: ≥ 3 < 6 mm.

Embedded items

General: To the Corrosion resistance and durability table. If there are water pipes and other embedded items, sheath them to permit thermal movement.

Lath

Location: Provide lath as follows:

- Chases: If chases or recesses are 50 mm wide or greater, fix metal lath extending 75 mm or more beyond each side of the chase or recess.

- Fit metal lath to masonry and concrete substrates:

- Metal and other non-porous substrates: Fix metal lath to provide a key. Installation: Fix lath as follows:

- General: Run the long way of the mesh across supports with strands sloping downwards and inwards from the intended face of the plaster.

- Fixing: Mechanically fix at centres of 150 mm or less.

- Laps: Tie with 1.25 mm galvanized wire at centres of 150 mm or less. Do not stop edges of sheets at corners but bend around.

- On solid substrates: Space the lath 5 mm or more clear of the substrate.

- Support spacing: ≤ 400 mm.

  - APPLICATION Plastering

Base coats: Scratch-comb each base coat in two directions when it has stiffened.

Metal lath: Press the plaster through the apertures of expanded metal lath and wings of beads. Incidental work

General: Return plaster into reveals, beads, sills, recesses and niches. Plaster faces, ends, and soffits

of projections in the substrate, such as string courses, sills, pilasters and corbels. Run neatly finished

throating on soffits of external projections. Trim around openings. Plaster exposed internal surfaces of built-in cupboards.

Joining up

General: If joining up is required, make sure joints are imperceptible in the finished work after decoration.

Control  joints

General: Provide joints in the finish to coincide with control joints in the substrate. Make sure the joint in the substrate is not bridged during plastering.

Size:

- Depth: Extend the joint right through the plaster and reinforcement to the substrate.

- Width: 3 mm, or the same width as the substrate joint, whichever is greater. Damp-proof courses: Do not continue plaster across damp-proof courses.

Plastering on metal lath: Provide control joints to divide the plastering area into rectangular panels of 10 m2 or less.

V-joints: Provide V-joints, cut right through the plaster to the substrate, at the following locations:

- Abutments with metal door frames.

- Abutments with other finishes.

- Junctions between different substrates. Cornices

General: Accurately cut and mitre corners. Match and align ornament. Do not make butt joints in the

length of a cornice unless required, or if full lengths are not available.

Installation: Butter edges, mitres and joins for the full length of the cornice with adhesive.

Mechanical fixing: If cornice projects across a ceiling 400 mm or more, provide additional mechanical fixing as follows:

- Fixing centres: ≤ 600 mm.

Temperature

General: If the ambient temperature is less than 10°C or more than 30°C, make sure the temperature of mixes, substrates and reinforcement at the time of application are between 5°C and 35°C.

  - FINISHES Finishing treatments

Plain even surfaces: Work the hardening plaster as follows:

- Bag: Rub the finish coat when set firm with a hessian bag or similar.

- Carborundum stone: Rub the finish coat when set hard with a carborundum stone to achieve a finish free from sand.

- Foam float: Float finish coat on application with a wood or plastic float to an even surface and finish with a foam float to achieve a fine sand textured finish.

- Steel trowel: Steel trowel finish coat to a smooth dense surface which is not glass-like and is free from shrinkage cracks and crazing.

- Wood or plastic float: Float the finish coat on application to an even surface with a wood or plastic float.

Ornamental patterned surfaces: Work the hardening plaster with a trowel or other tool. Sprayed textured surfaces: Spray plaster onto a substrate using a purpose-designed machine. Stippled textured surfaces: Work the hardening plaster with a stiff brush.

Rough thrown surfaces: Throw plaster onto a substrate or pebbles onto a plastic plaster base. Specialist plaster finishes

Polymer modified render:

- Basecoat render: Proprietary polymer modified cementitious render supplied as a complete plastering system.

- Finish coats: Proprietary trowelled on coloured and textured polymer modified finish coats.

Polished plaster: In situ applied plaster system incorporating selected stone dust in a proprietary matrix producing a smooth polished surface with visual patterning.

Glass bead coatings: Glass beads bound in a proprietary matrix.

  - COMPLETION Curing

General: Prevent premature or uneven drying out and protect from the sun and wind. Keeping moist: If a proprietary curing agent is not used, keep the plaster moist as follows:

- Base coats and single coat systems: Keep continuously moist for 2 days and allow to dry for 5 days before applying further plaster coats.

- Finish coats: Keep continuously moist for 2 days.

- SELECTIONS

  - SCHEDULES

Plastering construction schedule – Internal Surfaces only

| Property | A |
| --- | --- |
| Product | Unitex Uni-Dry Cote® HiLite Render™ (High build, lightweight) or equally an approved product. |
| Substrate material | Block Walls Internal |
| Substrate Moisture Leve | Must b less than 12% WME |

0621 WATERPROOFING – WET AREAS

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide wet area waterproofing systems as documented. Performance

Requirements:

- Grade to floor wastes, to dispose of water without ponding.

- Prevent moisture entering the substrate or adjacent areas.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - STANDARDS Wet areas

Standard: To AS 3740.

  - INTERPRETATION Definitions

General: For the purposes of this work section the definitions given in AS 3740 and the following apply:

- Bond breaker: A system preventing a membrane bonding to the substrate, bedding or lining.

- Membranes (waterproof): Impervious barriers to liquid water which may be:

. Installed below floor finishes.

. Installed behind the wall sheeting or render and termed External.

. Installed to the face of the wall sheeting or render and termed Internal.

. Applied in liquid or gel form and air cured to form a seamless film.

. Applied in sheet form with joints lapped and sealed.

- Preformed shower base: A preformed, prefinished vessel (including integral upstands) installed as the finished floor of a shower compartment, and provided with a connection point to a sanitary drainage system.

- Shower tray: An internal or external liquid or sheet membrane system used to waterproof the floor and the wall/floor junctions of a shower area.

- Substrate: The surface to which a material or product is applied.

- Waterproof (WP): The property of a material that does not allow moisture to penetrate through it.

- Waterproofing systems: Combinations of membranes, flashings, drainage and accessories which form waterproof barriers and which may be:

. Loose-laid.

. Bonded to substrates.

- Water resistant (WR): The property of material that restricts moisture movement and will not degrade under conditions of moisture.

- Wet area: An area within a building supplied with a floor waste.

  - SUBMISSIONS Products

Documentation: Submit copies of product manufacturer's:

- Product technical data sheets.

- Safety data sheets (SDS).

- Type tests certificates verifying conformance to AS/NZS 4858. Records

Placing records: Photographically record the application of membranes and information as follows:

- Date.

- Portion of work.

- Substrate preparation.

- Protection provided from traffic. Samples

General: Submit 300 x 300 mm samples of each type of membrane.

Shop drawings

Submit shop drawings showing:

- Junctions with vertical surfaces and upstands.

- Junctions at perimeters.

- Drainage details.

- Control joints.

- Flashings.

- Penetrations.

- Corners.

- Terminations and connections. Warranties

General: Submit details of the proposed warranty terms, form and period.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Substrate preparation completed.

- Secondary layers preparation completed.

- Before membranes are covered up or concealed.

- Flood testing, if applicable.

- PRODUCTS

  - GENERAL Membranes

Standard: To AS/NZS 4858. Membrane systems

Requirement: Provide a proprietary membrane systems certified as suitable for the intended wet area

waterproofing. Water stop angles

Material: Rigid, corrosion resistant angles compatible with the waterproof membrane system.

Bond breakers

Requirement: Compatible with the flexibility class of the membrane to be used.

Material: Purpose made bond breaker tapes and closed cell foam backing rods or fillets of sealant. Flashings

Requirement: Flexible waterproof flashings compatible with the waterproof membrane system.

Liquid membrane reinforcement

Requirement: Flexible fabric compatible with the waterproof membrane system. Sealants

Requirement: Waterproof, flexible, mould-resistant and compatible with host materials.

Adhesives

Requirement: Waterproof and compatible with host materials.

- EXECUTION

  - PREPARATION Substrates

General: Make sure substrates are as follows:

- Clean and free of any deposit or finish which may impair adhesion of membranes.

- If walls are plastered, remove loose sand.

- If walls or floors are framed or discontinuous, support members are in full lengths without splicing.

- If floors are solid or continuous:

. Excessive projections are removed.

. Voids and hollows greater than 10 mm with abrupt edges are filled with a cement:sand mix not stronger than the substrate nor weaker than the bedding.

. Depressions less than 10 mm are filled with a latex modified cementitious product with feathering eliminated by scabbling the edges.

. Cracks in substrates wider than 1.5 mm are filled with a filler compatible with the membrane system.

External corners: Round or arris edges. Moisture content

Concrete substrates: Cure for at least 21 days.

Moisture content: Verify that the moisture content of the substrate is compatible with the water vapour transmission rate of the membrane system by testing to AS 1884 Appendix A.

Falls

Substrate: If the membrane is directly under the floor finish, make sure the fall in the substrate conforms to the fall documented for the finish.

Sheet substrate fastening

Requirement: Fasten or adequately fix to the supporting structure. Control joints

Finishes: Align control joints in finishes and bedding with control joints or changes in materials in the

substrate.

Water stop angles

Requirement: Provide water stop angles at door thresholds and shower enclosures to support the waterproof membrane at junctions between waterproofed and non-waterproofed areas.

Sizing: Size the vertical leg of the water stop angle to conform to the requirements of AS 3740.

Corners: Cut the horizontal leg and bend the vertical leg at corners instead of forming vertical joints between separate lengths of angle.

Fixing: Fix water stop angles to the substrate with compatible sealant or adhesive and corrosion- resistant countersunk or wafer head screws.

Priming

General: If required by the membrane manufacturer, prime the substrates with a primer compatible with the membrane system.

Bond breakers

Requirement: After the priming of surfaces, provide bond breakers at all wall/floor, hob/wall junctions and at control joints where the membrane is bonded to the substrate.

Sealant fillet bond breakers:

- Application: Form a triangular fillet or cove of sealant to internal corners within the period recommended by the membrane manufacturer after the application of the primer.

- Widths: 5 mm x 5 mm to vertical corners. 6 mm x 6 mm to 9 mm x 9 mm to horizontal corners.

Backing rod bond breakers: Retain in position with continuous length of tape pressed firmly in place against the surfaces on each side of the rod.

  - APPLICATION

Protection

General: Protect membrane from damage during installation and for the period after installation until the membrane achieves its service characteristics that resist damage.

Extent of waterproofing

Waterproof or water resistant surfaces: To the requirements of BCA F1.7. Vertical membrane terminations

Upstands: At least 150 mm above the finished tile level of the floor or 25 mm above the maximum

retained water level, whichever is the greater.

Anchoring: Secure sheet membranes along the top edge. Edge protection: Protect edges of the membrane.

Flashings

Junctions between waterproof surfaces: Provide a bond breaker at internal corners behind flashings.

Junctions between waterproof surfaces and other surfaces: Provide a bead of sealant at the following junctions:

- Waterproof and water-resistant surfaces.

- Water-resistant and water-resistant surfaces.

- Water-resistant and non water-resistant surfaces.

Perimeter flashings: Provide continuous flashings to the full perimeter of waterproof areas at wall/floor junctions and to water stop angles.

Vertical flashings: Provide vertical corner flashings continuous across wall/wall junctions to at least 1800 mm above finished floor level.

Vertical liquid applied flashings:

- Return legs at least 40 mm on each wall.

- Overlap the vertical termination of the floor waterproofing membrane at least 20 mm. Vertical sheet flashings:

- Return legs at least 50 mm on each wall.

- Overlap shower tray upstands at least 50 mm.

- Do not penetrate flashing with wall lining fasteners.

Reinforcement: At coves, corners and wall/floor junctions with gaps greater than 3 mm reinforce liquid applied membranes with reinforcement fabric tape recommended by the membrane manufacturer.

Fold the tape in half lengthways and imbed it in the first flashing coat of membrane with one half of the tape on each side of the corner or joint. Apply a second coat of liquid membrane to seal the fabric.

Drainage connections

Floor wastes: Provide floor wastes of sufficient height to accommodate the thickness of floor finishes and bedding at the outlet position. Position drainage flange to drain at membrane level. Turn membrane down 50 mm minimum into the floor waste drainage flanges, and adhere to form a waterproof connection.

Floor wastes in shower trays: Provide drainage of the tile bed and a waterproof connection between the tray and the drain.

Preformed drainage channels:

- With continuous drainage flanges: Provide a continuous waterproof connection between the membrane and the channel.

- Without drainage flanges: Provide continuous waterproofing under the channel and terminate the membrane at a floor waste with a recessed drainage flange.

Enclosed hobless showers with trench drain located below screen

Framed or frameless shower screens: Install a water stop angle where the outer edge of the trench drain to the perimeter of the shower will be installed. Size the angle so that the vertical leg finishes at the underside of the tiles. Support and adhere the membrane over the water stop angle and terminate the membrane at floor wastes as described in Drainage connections. Install the trench drain with the shower screen located vertically above it.

Unenclosed showers

Requirement: Extend membrane at least 1500 mm into the room from the shower rose outlet on the wall.

Taps and spouts

Requirement: Waterproof penetrations for taps and spouts with proprietary flange systems or a sealant.

Provision for servicing: Install taps in a manner that allows tap washers or ceramic discs to be serviced without damaging the waterproofing seal.

Curing of liquid applied   systems General: To the manufacturer's instructions.

Curing: Allow membrane to fully cure before tiling.

  - FLOOD TEST General

Application: Perform a flood test before the installation of surface finishes. Moisture measurement method: Conform to AS 1884 Appendix A.

Set-up:

- Measure the wall/floor junction of adjacent spaces and the floor soffit below for dryness.

- Record the result for each area.

- Dam the doorway(s) and seal floor wastes and drainage outlets to allow 50 mm water level.

- Fill space with clean water and leave overnight. Evaluation:

- Make a visual inspection of the wall/floor junction of adjacent spaces and of the floor soffit below for obvious water or moisture.

- Test the same areas for dryness and compare the results to the measurements taken before flooding.

Compliance:

- Evidence of water from the visual test: Failure.

- No visual evidence of water: Proceed with moisture measurements.

- Test results indicating an increase in moisture before and after flooding: Failure. Records:

- Submit records of all flood tests.

  - COMPLETION Protection

General: Keep traffic off membrane surfaces until bonding has set or for 24 hours after laying, whichever period is the longer.

Reinstatement: Repair or replace faulty or damaged work. Warranties

Requirement: Cover materials and workmanship in the terms of the warranty in the form of interlocking

warranties from the supplier and the applicator.

- Form: Against failure of materials and execution under normal environment and use conditions.

- Period: As offered by the supplier.

- SELECTIONS

  - SYSTEMS

| Property | All Wet areas Floors and Showers |
| --- | --- |
| Proprietary system | Proprietary system Parchem Emer-Proof 680 or equally an approved product. |

| Property | All Wet areas Floors and Showers |
| --- | --- |
| Material type | Material type Water based, fast drying urethane modified waterproofing membrane |
| Priming | Priming In accordance with manufacturers current Emer-Proof 680 technical data sheet or equally an approved product. |
| Number of coats | Number of coats 3 |
| Reinforcement | Reinforcement In accordance with manufacturers current Emer- Proof 680 technical data sheet |
| Method of application | In accordance with manufacturers current Emer-Proof 680 technical data sheet |
| In accordance with manufacturers current Emer- Proof 680 technical data sheet | In accordance with manufacturers current Emer-Proof 680 technical data sheet |
| Dry film thickness | In accordance with manufacturers current Emer-Proof 680 technical data sheet |

0631B CERAMIC TILING

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide tiling systems to walls, floors and other substrates as documented. Performance

Requirements:

- Consistent in colour and finish.

- Firmly bonded to substrates for the expected life of the installation.

- Set out with joints accurately aligned in both directions and wall tiling joints level and plumb.

- Direct all water flowing from supply points to drainage outlets without leakage to the substrate or adjacent areas.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - STANDARDS Tiling

General: Conform to the documented recommendations of those parts of AS 3958.1 which are referenced in this work section.

Slip resistance Classification: To AS 4586.

  - TOLERANCES Completed tiling

Standard: To AS 3958.1 clause 5.4.6.

  - SUBMISSIONS Samples

General: Submit labelled samples of tiles, including fittings, accessories, grout and sealants, illustrating the range of variation in colour and finish.

Sample panels: Prepare a sample panel of each type of tiling system as follows:

- Size: > 2 m2.

- Include samples of junction details and trim.

- Preserve the panel until related work is complete. Tests

Type tests: Submit results, as follows:

- Slip resistance of tiles.

Site tests: Submit results, as follows:

- Slip resistance test of completed installation.

- Impact sound insulation.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Substrate immediately before tiling.

- Trial set-outs before execution.

- Control joints before sealing and grouting.

- Grout and sealant colours before application.

- PRODUCTS

  - GENERAL Marking

Identification: Marked to show the following:

- Manufacturer’s identification.

- Product brand name.

- Product type.

- Quantity.

- Product reference code and batch number.

- Date of manufacture.

- Material composition and characteristics such as volatility, flash point, light fastness, colour and pattern.

system.

  - TILES AND ACCESSORIES Tiles

Standard: To AS ISO 13006.

Tactile ground surface indicators: To AS/NZS 1428.4.1.

Coves, nosings and skirtings: Provide matching stop-end and internal and external angle tiles moulded for that purpose.

Exposed edges: Purpose-made border tiles with the exposed edge (whether round, square or cushion) glazed to match the tile face. If such tiles are not available, mitre tiles on external corners.

Accessories

General: Provide tile accessories which match the composition, colour and finish of the surrounding tiles.

  - ADHESIVES General

Standard: To AS ISO 13007.1. Type

General: Provide adhesives compatible with the materials and surfaces to be adhered, and as

documented in the Wall tiling schedule and to the Floor tiling schedule. Prohibited uses: Do not provide the following combinations:

- Cement-based adhesives on wood, metal, painted or glazed surfaces, gypsum-based plaster.

- Organic solvent-based adhesives on painted surfaces.

- Organic PVC-based adhesives and organic natural rubber latex adhesives in damp or wet conditions.

- PVA (polyvinyl acetate) based adhesives in wet areas or externally.

  - MORTAR Materials

Cement type to AS 3972: GP.

- White cement: Iron salts content not more than 1%.

- Off-white cement: Iron salts content not more than 2.5%. Lime: To AS 1672.1.

Sand: Fine aggregate with a low clay content selected for grading, sharp and free from efflorescing salts.

Measurement of volume: Measure binders and sand by volume using buckets or boxes. Do not allow sand to bulk by absorption of water.

Bedding mortar

Mix proportion (cement:sand), by volume: Select proportions from the range 1:3 to 1:4 for satisfactory adhesion. Provide minimum water.

Terracotta tiles: Use proprietary polymer modified mortar. Mixing: To AS 3958.1 clause 2.15.

Water

General: Clean and free from any deleterious matter.

  - GROUT Type

Cement based proprietary grout: Mix with water. Fine sand may be added as a filler in wider joints. Terracotta tiles: Use proprietary polymer modified grout.

General purpose cement based grout: Mix with fine sand. Provide minimum water consistent with workability.

Mix proportions (cement:sand), by volume:

- For joints < 3 mm: 1:2.

- For joints ≥ 3 mm: 1:3. Pigments

Pigments for coloured grout: Provide colourfast fillers compatible with the grout material. For cement-

based grouts, provide lime-proof natural or synthetic metallic oxides compatible with cement.

  - CONTROL JOINTS Control joint materials

General: As documented in the Control joint schedule.

Control joint strip: A proprietary expansion joint consisting of a neoprene filler sandwiched between plates with lugs or ribs for mechanical keying. Set flush with the finished surface.

Proprietary slide plate divider strip: An arrangement of interlocking metal plates grouted into pockets formed in the concrete joint edges.

Sealant: One-part self-levelling non-hardening mould resistant, silicone or polyurethane sealant applied over a backing rod. Finish flush with the finished surface.

- Floors: Trafficable, shore hardness greater than 35.

Backing rod: Compressible closed cell polyethylene foam with a bond-breaking surface.

- EXECUTION

  - SUBSTRATES Drying and shrinkage

General: Before tiling, allow at least the following times to elapse (for initial drying out and shrinkage) for these substrates:

- Concrete slabs: 42 days.

- Concrete blockwork: 28 days.

- Toppings on slabs and rendering on brick or blockwork: A further 21 days.

- Rendering on swimming pool shell: A further 28 days minimum.

  - PREPARATION Standard

Preparation: To AS 3958.1 Section 4. Ambient temperature

General: If the ambient temperature is less than 5°C or greater than 35°C, do not lay tiles.

Substrates without wet area membranes General: Conform to the following:

- Clean off of any deposit or finish which may impair adhesion or location of tiles.

- If framed or discontinuous, support members are in full lengths without splicing.

- If solid or continuous:

. Remove excessive projections.

. Fill voids and hollows greater than 10 mm with abrupt edges with a cement:sand mix not stronger than the substrate or weaker than the bedding.

. Fill depressions less than 10 mm with a latex modified cementitious product and eliminate feathering by scabbling the edges.

Absorbent substrates: If suction is excessive, control it by dampening but avoid over-wetting and do not apply mortar bedding to substrates showing surface moisture.

Dense concrete: If not sufficiently rough to provide a mechanical key, roughen by scabbling or the like to remove 3 mm of the surface and expose the aggregate; then apply a bonding treatment.

Substrates with wet area membranes General: Make sure substrates are as follows:

- Clean and free of any deposit or finish which may impair adhesion or location of tiles.

- Compatible with all components of the floor system.

  - TILING GENERALLY Sequence

General: Confirm alignment prior to proceeding with installation. Cutting and laying

Cutting: Cut tiles neatly to fit around fixtures and fittings and at margins where necessary. Drill holes

without damaging tile faces. Cut recesses for fittings such as soap holders. Rub edges smooth without chipping.

Laying: Return tiles into sills, reveals and openings. Butt up to returns, frames, fittings, and other finishes. Strike and point up beds where exposed. Remove tile spacers before grouting.

Variations

General: Distribute variations in hue, colour, or pattern uniformly, by mixing tiles or tile batches before laying.

Protection

Floor tiles: Keep traffic off floor tiles until the bedding has set and attained its working strength. Cleaning: Keep the work clean as it proceeds and protect finished work from damage.

Bath ventilation

General: Ventilate the space below fully enclosed baths with at least 2 vermin proof ventilating tiles.

  - SETTING OUT Tile joints

Joint widths: Set out tiles to give uniform joint widths within the following limits:

- Floors:

. Dry pressed tiles: 3 mm.

. Extruded tiles: 6 mm.

. Vitrified: 3 to 5 mm.

. Quarry tiles: 6 to 12 mm.

. Chemical resistant epoxy jointed tiling: 5 to 6 mm.

- Large and/or irregular floor tiles: 6 to 12 mm.

- Mounted mosaics: To match mounting pattern.

- Walls:

. Dry pressed tile: 1.5 mm.

. Extruded tile: 6 mm.

Joint alignment: Set out tiling with joints accurately aligned in both directions and wall tiling joints level and plumb.

Joint position: Set out tiles from the centre of the floor or wall to be tiled.

Margins

General: Provide whole or purpose-made tiles at margins where practicable, otherwise, set out to give equal margins of cut tiles. If margins less than half a tile width are unavoidable, locate the cut tiles where they are least conspicuous.

Fixtures

General: If possible position tiles so that holes for fixtures and other penetrations occur at the intersection of horizontal and vertical joints or on the centre lines of tiles. Continue tiling fully behind fixtures which are not built in to the tiling surface. Before tiling make sure that fixtures interrupting the tile surfaces are accurately positioned in their designed or optimum locations relative to the tile layout.

  - FALLS AND LEVELS Grading

General: Grade floor tiling to even and correct falls to floor wastes and elsewhere as required. Make level junctions with walls. Where falls are not required, lay level.

Fall, general: 1:100 minimum.

Fall, in shower areas: 1:60 minimum.

Change of finish: Maintain finished floor level across changes of floor finish including carpet.

  - BEDDING Standard

Cement mortar: To AS 3958.1 clause 5.5. Adhesive: To AS 3958.1 clause 5.6.

Preparation of tiles

Adhesive bedding: Fix tiles dry; do not soak.

Mortar bedding: Soak porous tiles in water for half an hour and then drain until the surface water has disappeared.

Terracotta tiles: Use pre-sealed tiles or apply a breathable sealer and lay dry. If a final sealed finish is selected, use a compatible laying sealer.

Bedding

General: Use bedding methods and materials which are appropriate to the tile, the substrate, the conditions of service, and which leave the tile firmly and solidly bedded in the bedding material and adhered to the substrate. Form falls integral with the substrate.

Thin adhesive beds

General: Provide only if the substrate deviation is less than 3 mm when tested with a 3 m straight edge. Cover the entire tile back with adhesive when the tile is bedded.

Thickness: 1.5 to 3 mm.

Thick adhesive beds

General: Provide on substrates with deviations up to 6 mm when tested with a 3 m straight edge, and

with tiles having deep keys or frogs. Nominal thickness: 6 mm.

Adhesive bedding application

General: Apply adhesive by notched trowel to walls and floors and direct to tiles if required, to provide evenly distributed coverage after laying as follows:

- Domestic internal walls: > 65%.

- Domestic internal floors: > 80%.

- Other wall and floors: > 90%.

- Wet areas and bench tops: 100%.

Pattern of distribution of adhesive: To the recommendations of AS 3958.1 clause 5.6.4.3. Verify by examining one tile in ten as work proceeds.

Wall tile spacers: Do not use spacer types that inhibit the distribution of adhesive.

Curing: Allow the adhesive to cure for the period nominated by the manufacturer before grouting or allowing foot traffic.

Mortar beds

For floor tiles: Either lightly dust the screeded bed surface with dry cement and trowel level until the cement is damp, or spread a thin slurry of neat cement, or cement-based thin bed adhesive, on to the tile back. Do not use mortar after initial set has occurred.

- Nominal thickness: 20 to 40 mm.

Thick reinforced beds: Place mortar bed in two layers, and incorporate the mesh reinforcement in the first layer.

  - CONTROL OF MOVEMENT General

Requirement: Provide control joints carried through the tile and the bedding to the recommendations of AS 3958.1 clause 5.4.5 and as follows:

- Floor location:

. Over structural control joints.

. To divide complex room plans into rectangles.

. Around the perimeter of the floor.

. At junctions between different substrates.

. To divide large tiled areas into bays.

. At abutments with the building structural frame and over supporting walls or beams where flexing of the substrate is anticipated.

- Wall location:

. Over structural control joints.

. At junctions with different substrate materials when the tiling is continuous.

. At vertical corners in shower compartments.

- Depth of joint: Right through to the substrate.

- Sealant width: 6 to 25 mm.

- Depth of elastomeric sealant: One half the joint width, or 6 mm, whichever is the greater.

  - GROUTED AND SEALANT JOINTS Grouted joints

General: Commence grouting as soon as practicable after bedding has set. Clean out joints as necessary before grouting.

Face grouting: Fill the joints solid and tool flush. Clean off surplus grout. Wash down when the grout has set. When grout is dry, polish the tiled surface with grout film remover and a clean cloth.

Edges of tiles: Grout exposed edge joints.

Epoxy grouted joints: Make sure tile edge surfaces are free of extraneous matter such as cement films or wax, before grouting.

Mosaic tiles

Grouting mosaics: If paper faced mosaics are to be bedded in cement mortar, pre-grout the sheeted mosaics from the back before fixing. After fixing, rub grout into the surface of the joints to fill any voids left from pre-grouting. Clean off surplus grout. When grout has set, wash down. If necessary, use a proprietary cement remover.

Sealant joints

General: Provide joints filled with sealant and finished flush with the tile surface as follows:

- Where tiling is cut around sanitaryfixtures.

- At internal corners of walls in showers.

- Around fixtures interrupting the tile surface, for example pipes, brackets, bolts and nibs.

- At junctions with elements such as window and door frames and built-in cupboards. Material: Anti-fungal modifiedsilicone.

Width: 5 mm.

Depth: Equal to the tile thickness.

  - JOINT ACCESSORIES

Floor finish dividers

General: Finish tiled floors at junctions with differing floor finishes with a corrosion-resistant metal dividing strip fixed to the substrate using mechanical fixings, with top edge flush with the finished floor. If changes of floor finish occur at doorways, make the junction directly below the closed door. Grout up underneath to provide continuous support.

Stepping: Less than 5 mm. Adjustments

Requirement: Check that the height of the floor finish divider is sufficient for the topping and tile

thickness. Adjust as required with a matching flat bar adhesive fixed to the divider angle. Weather bars

General: Provide a corrosion-resistant metal weather bar under hinged external doors. Locate under

the centres of closed doors.

  - COMPLETION Spare tiles

General: Supply spare matching tiles and accessories of each type for future replacement purposes. Store the spare materials on site.

Quantity: At least 1% of the quantity installed. Storage location: As directed by Client Cleaning

General: Clean tiled surfaces using an appropriate tile cleaning agent, and polish.

Operation and maintenance manuals

General: Submit a manual describing care and maintenance of the tiling, including procedures for maintaining the slip-resistance classification stating the expected life of the slip-resistance classification.

Tender for the MJCA’s New Courthouse & Judges’

Residence in Salelologa, Savaii Project.	0631B Ceramic tiling

- PREFERRED SELECTIONS (Contractor to nominate alternative selections)

  - SCHEDULES

| Designation | Wall Tiles (Kitchenette Splashback) | Wall Tiles (ALL Wet Areas) | Floor Tiles (All Areas) | Cut Skirting Tile | Floor Tiles (All Wet Areas) |
| --- | --- | --- | --- | --- | --- |
| Product | Metz tiles or equally an approved product. |  |  |  |  |
| Colour | White Gloss Porcelain | White Gloss Porcelain | Timeless – Light Grey Microgrip ® R12 or Similar Cushion Edge | Timeless – Light Grey Microgrip ® R12 or Similar Cushion Edge | Timeless – Light Grey Microgrip ® R12 or Similar Cushion Edge |
| Size | 300x 600 x 3mm | 300 x600 x 10mm | 600 x 600 x 8mm | 300 x 200 x 8mm cut tile | 300 x 300 x 8mm |
| Bond/Pattern | Stack | Stack | Stack | Stack | Stack |
| Grout (Noted as Metz or equally approved) |  |  |  |  |  |
| Type | Metz 5NF | Metz 5NF | Metz 5NF | Metz 5NF | Metz 5NF |
| Colour | White | White | Grey | Grey | Grey |
| Adhesive |  |  |  |  |  |
| Type | Use Metz HAS High Performance Ceramic Tile Adhesive, typical to all tiles or equally an approved product. |  |  |  |  |
| Trims |  |  |  |  |  |
| External corners | DTA Aluminium Square Edge or Similar, Matt Silver finish Height to suit selected tile. |  |  |  |  |
| Internal corners | DTA Aluminium Square Edge or Similar, Matt Silver finish Height to suit selected tile. |  |  |  |  |
| Top Trim | DTA Aluminium Round Edge or Similar, Matt Silver finish Height to suit selected tile. |  |  |  |  |
| Movement joints | DTA Neoprene Aluminium Movement Joint or Similar 6mm wide, colour grey Height to suit selected tile. |  |  |  |  |

145 | P a g e

0671P PAINTING

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide coating systems to substrates, as documented. Equally approved products to specified products will be consider as alternatives.

Performance

Requirement: Provide coating systems as follows:

- Consistent in colour, gloss level, texture and dry film thickness.

- Free of runs, sags, blisters, or other discontinuities.

- Paint systems which are fully opaque or at the documented level of opacity.

- Clear finishes at the level of transparency consistent with the product.

- Fully adhered.

- Resistant to environmental degradation within the manufacturer’s stated life span.

  - COMPANY CONTACTS

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - STANDARDS Painting

General: To the recommendations of those parts of AS/NZS 2311 referenced in this work section.

  - MANUFACTURER'S DOCUMENTS Technical manuals

Product Guide:

Duspec Product Data Sheets, SDS, paint system selection:

  - INTERPRETATION Abbreviations

General: For the purposes of this work section the following abbreviations apply:

- ASU: Acrylic sealer undercoat multipurpose combo product.

- DFT: Dry film thickness.

- OFC: Off form concrete.

- PDS: Product data sheet.

- PRN: Paint reference number.

- PSU: Primer sealer undercoat multipurpose combo product.

- WFT: Wet film thickness. Definitions

General: For the purposes of this work section the definitions in AS/NZS 2310 and the following apply:

- Gloss: The optical property of a surface, characterized by its ability to reflect light specularly.

- Gloss unit: Numerical value for the amount of specular reflection relative to that of a standard surface under the same geometric conditions.

- Levels of gloss finish: When the specular direction is 60 degrees, surfaces with the following specular gloss reading is defined as follows:

. Full gloss: Over 85 gloss units.

. Gloss: Between 50 and 85 gloss units.

. Semi-gloss: Between 20 and 50 gloss units.

. Low gloss (low sheen): Between 5 and 20 gloss units.

. Flat finish (matt): Up to 5 gloss units.

- Opacity: The ability of a paint or textured and membrane coating to obliterate the colour difference of a substrate.

- Paint or coating system: A product in liquid form, which when applied to a surface, forms a dry film having protective, decorative or other specific technical properties.

- Primer, prime coat: The first coat of a painting system that helps bind subsequent coats to the substrate and which may inhibit its deterioration.

- Sealer: A product used to seal substrates to prevent the following:

. Materials from bleeding through to the surface.

. Reaction of the substrate with incompatible top coats.

. Undue absorption of the following coat into the substrate.

- Substrate: The surface to which a material or product is applied.

- Undercoat: An intermediate coat formulated to prepare a primed surface or other prepared surface for the finishing coat.

  - SUBMISSIONS

Warranties

Requirement: Submit the coating manufacturer’s warranties as follows:

- [complete/delete]

Material warranty: Submit the manufacturer’s material warranty as follows:

- Extent: Paintwork generally.

- Terms: Paint systems are suitable for their intended use.

- Warranty period: As defined by the manufacturer.

Material performance warranty: Submit an alternative performance warranty as follows:

- Extent: [complete/delete]

- Terms: Submit the performance criteria as defined by the manufacturer.

- Measure: As defined by the manufacturer.

- Warranty period: As defined by the manufacturer. Timing: Before the application of the paint system.

  - INSPECTION

Notice

Inspection: Give notice so that inspection may be made of the following:

- Painting stages:

. Completion of surface preparation.

. After application of final coat.

- Clear finishing stages:

. Before surface preparation of timber.

. Completion of surface preparation.

. After application of final coat.

- PRODUCTS

  - GENERAL Product substitution

Other products: Conform to PRODUCTS, GENERAL, Substitutions in the 0171 General requirements worksection.

Storage and handling

General: Store materials not in use in tightly covered containers in well-ventilated areas with temperatures maintained at the manufacturer’s recommendations.

Delivery: Deliver paints to the site in the manufacturer’s labelled and unopened containers. Marking

Identification: Marked to show the following:

- Manufacturer’s identification.

- Product brand name.

- Product type.

- Quantity.

- Product reference code and batch number.

- Date of manufacture.

- Material composition and characteristics such as volatility, flash point, light fastness, colour and pattern.

  - PAINTING MATERIALS Combinations

General: Do not combine paints from different manufacturers in a paint system. Paint products and coating systems have been selected and specified for this project. Any unauthorised product substitution will void the warranties.

Clear timber finish systems: Provide only the combinations of putty, stain and sealer recommended by the manufacturer of the top coats.

Tinting

General: Provide only products which are colour tinted by the manufacturer or supplier. Toxic ingredients

General: To the Poisons Standard (including SUSMP 11) Part 2 Section 7.

Standards

Paint types: Conform to the Australian Standard referenced in the OCP paint type reference table. paint type reference table legend

Key:

ASU = Acrylic Sealer/Undercoat. NE = No Equivalent.

PSU = Primer/Sealer/Undercoat.

Low VOC products are noted in the Table.

^ Use is discouraged in favour of water based paints because of environmental concerns. # These paints have either limited availability or low requirement in the Building Industry. DuluxGroup/Dulux paint type reference table used as reference only.

| Paint type | DuluxGroup/Dulux material description | Dulux PDS No. | AS/NZS 2311PRN (Table 4.2) | Standard |
| --- | --- | --- | --- | --- |
| Low gloss latex: exterior | Dulux Weathershield Low Sheen Acrylic | DD0053 | B7b | AS 3730.8 |

| Paint type | DuluxGroup/Dulux material description | Dulux PDS No. | AS/NZS 2311PRN (Table 4.2) | Standard |
| --- | --- | --- | --- | --- |
| Low gloss latex: interior | Dulux Wash&Wear Low Sheen Acrylic (low VOC) | DD02070 | B7a | AS 3730.3 |
|  | Dulux Wash&Wear +Plus Kitchen & Bathroom Low Sheen (low VOC) | DD02074 |  |  |
| Metal primer for zinc-coated surfaces, latex | Dulux Professional Galvanised Iron Primer (water based, low VOC) | DD0156 | B12a | AS 3730.15 |
| Concrete and masonry sealer | Dulux Sealer Binder Dulux Acratex Acraprime 501/2 Berger Gold Label Acrylic Block Filler | DD0074 DA0442 DD0217 | B15 | AS 3730.22 |

- EXECUTION

  - PREPARATION Standards

General: To AS/NZS 2311 Sections 3. Order of work

Other trades: Before painting, complete the work of other trades as far as practicable within the area

to be painted, except for the installation of fittings, floor sanding and laying flooring materials.

Clear finishes: Complete clear timber finishes before commencing opaque paint finishes in the same area.

Protection

General: Before painting, clean the area and protect it from dust contamination. Use drop sheets and masking agents to protect surfaces, including finished surfaces and adjacent finishes, during painting.

Fixtures and furniture: Remove door furniture, switch plates, light fittings and other fixtures before painting, and conform to the following:

- Labelling and storage: Attach labels or mark fixtures using a non-permanent method, identifying location and refixing instructions, if required. Store and protect against damage.

Difficult to remove fixtures: Where removal is impractical or difficult, apply surface protection before substrate preparation and painting.

Wet paint warning

Notices: Place in a conspicuous location and do not remove until the paint is dry. Substrate preparation – generally

General: Prepare substrates to receive the painting systems in conformance with AS/NZS 2311 and

the paint manufacturer's recommendations.

Cleaning: Clean down the substrate surface. Do not cause damage to the substrate or the surroundings.

Filling: Fill cracks and holes with fillers, sealants, putties or grouting cements as appropriate for the finishing system and substrate, and sand smooth.

- Clear finish: Provide filler tinted to match the substrate.

Clear timber finish systems: Prepare the surface so that its attributes will show through the clear finish without blemishes, using methods including the following:

- Removal of bruises.

- Removal of discolorations, including staining by oil, grease and nail heads.

- Bleaching where necessary to match the timber colour sample.

- Puttying.

- Fine sanding, with the last abrasive no coarser than 220 grit, so that there are no scratches across the grain.

Treated surfaces: If surfaces have been treated with preservatives or fire retardants, make sure coating is compatible with the treatment and does not adversely affect its performance.

Iron and steel: Remove weld spatter, slag, burrs, or any other objectionable surface irregularities and radius all edges to a minimum of 2 mm. Degrease by solvent or alkaline cleaning.

Iron and steel blast cleaning: To AS 1627.9 and to the class specified in the specified protective treatment. Provide a surface roughness or profile appropriate for the specified treatment. Where steelwork to be abrasive cleaned includes irregular shapes allow for special equipment to achieve required abrasive cleaning.

Structural steel: All exposed fixings including bolts, screws and the like, are to be painted to match adjacent steelwork paint system.

Concrete and masonry: Before application to very smooth concrete, brick or masonry, either acid etch, mechanically grind or abrasive track blast the surface as appropriate to provide a suitable key for the subsequently applied coating and to remove laitance. Remove loose friable matter before filling surface discontinuities.

Set plaster surfaces: Do not apply solvent borne paint or other impervious coatings if the moisture content at the surface, tested with a moisture meter, exceeds 12%.

  - PAINTING Standard

General: To AS/NZS 2311 Section 6. Light levels

General: During preparation of surfaces, painting and inspection, maintain light levels such that the

luminance (photometric brightness) of the surface is equal to the specified permanent artificial illumination conditions or 400 lux, whichever is the greater.

Substrate moisture content

Requirement: Use a moisture meter to demonstrate that the moisture content of the substrate is at or below the recommended maximum level for the type of paint and the substrate material.

Paint application

General: Apply the first coat immediately after substrate preparation and before contamination of the substrate can occur. Apply subsequent coats after the manufacturer’s recommended drying period has elapsed.

Painting  conditions

General: Unless the paint is recommended for such conditions, do not paint under the following conditions:

- Dusty conditions.

- Relative humidity: > 85%.

- Surface temperature: < 10°C or > 35°C. Priming before fixing

General: Apply one coat of wood primer, and 2 coats to end grain, to the back of the following before

fixing in position:

- External fascia boards.

- Timber door and window frames.

- Bottoms of external doors.

- Associated trims and glazing beads.

- Timber board cladding.

Sanding

Clear finishes: Sand the sealer using abrasives no coarser than 320 grit without cutting through the colour. Take special care with round surfaces and edges.

Repair

Requirement: Clean off marks, paint spots and stains progressively and restore damaged surfaces to their original condition.

Maintenance painting: To AS/NZS 2311 Section 8. Repair of galvanizing

Cleaning: For galvanized surfaces which have been subsequently welded, power tool grind to remove

all surface contaminants, including rust and weld splatter. Prime affected area immediately after cleaning.

Primer: Type 2 organic zinc-rich coating for the protection of steel to AS/NZS 3750.9. Tinting

General: Tint each coat of an opaque coating system so that each has a noticeably different tint from

the preceding coat where possible, except for top coats in systems with more than one top coat. Services

General: Paint all new services and equipment, including those in plant rooms, if not embedded,

except chromium, anodized aluminium, GRP, PVC-U, stainless steel, non-metallic flexible materials and normally lubricated machined surfaces.

Proprietary items: Repaint only if damaged. Windows

Operation: Make sure opening windows function correctly before and after painting.

Doors

Drying: Maintain door leaf in the open position during drying. Do not allow door hardware or accessories to damage the door finish during the drying process.

Exclusions

Exclude the following surfaces from paint systems (unless specifically requested):

- Flexible duct connections, rubber hoses and mountings and other non metallic flexible fittings.

- Wire rope and machined surfaces.

- Metals plated or specially finished for appearance, bronze, brass, copper and stainless steel (except as specified in the Pipe identification clause of the Services work sections).

- Aluminium frames.

- Prefinished aluminium frames to windows and doors, and trim.

- Metal floor duct covers.

- Raised access floors.

- Floors.

- Fair faced brickwork, blockwork, stonework, artificial stone and exposed aggregates.

- Sprayed vermiculite.

- Floors, paving, roads unless otherwise specified.

- Timber roof structure.

- Concealed timber roof structure.

- Timber ceiling and eaves lining.

- Exterior timber sheeting.

- Exterior timber stairs and decking.

- Plastic finishes generally

- Inside of service ducts, heat exchangers, pipes and valves.

- Shower seats, store shelving, work benches.

- Those parts of timber fixtures, such as insides of cupboards, not visible when doors are closed, unless otherwise specified. Insides of bathroom cabinets are not excluded and shall be painted.

- Self finished surface such as glass and plastic laminates.

- Door hardware, including hinges.

  - COMPLETION General

Protection and masking: Remove masking and protection coverings before paint has dried.

Cleaning: On completion of painting, remove splatters by washing, scraping or other methods which do not scratch or damage adjacent finished surfaces.

Reinstatement: Repair, replace or refinish any damage, including works of other trades. Touch up new damaged decorative paintwork or misses only with the paint batch used in the original application.

Removed fixtures: Refix undamaged fixture in the original location, make sure they are properly fitted and in proper working order.

Disposal of paint and waste materials.

Requirement: Conform to requirements of the local government authority.

- SELECTIONS

  - PAINTING SCHEDULES GENERALLY Paint system schedules

Requirement: Apply paint systems as documented in the Interior painting schedule and the Exterior painting schedule.

General: Apply the paint system nominated for each substrate to the referenced manufacturer’s Product Data Sheets (PDS) and Spec Sheets and include:

- The number and order of coats.

- The paint type for each coat. Additional coats: Apply if necessary to:

- prepare porous or reactive substrates with prime or seal coats consistent with the manufacturer's recommendations;

- achieve the total film thickness or texture specified; or

- achieve a satisfactory opacity, in the specified or required colour. Painting systems

Standards: The scheduled DuluxGroup/Dulux paint systems override AS/NZS 2311 as follows:

- New unpainted interior surfaces: To AS/NZS 2311 Table 5.1.

- New unpainted exterior surfaces: To AS/NZS 2311 Table 5.2.

Paint Reference Number (PRN): The number in brackets against the individual product refers to the Paint Ref. No. (PRN) listed in the DuluxGroup/Dulux paint type reference table (See PRODUCTS) and AS/NZS 2311 Appendix D.

  - INTERIOR PAINTING SCHEDULE

Colour selection to be confirmed prior to Construction. Allow for Complete range of Dulux standard colours or equally approved. The paint system below can be substituted for an equally approved product.

Contractor must submit specifications and demonstrate how the alternative paint system matches or exceeds the specification below.

Flat and matt latex: Interior

| Substrate | 1st coat | 2nd coat | 3rd coat | Manufacturer’s Spec Sheet Ref |
| --- | --- | --- | --- | --- |
| Internal Plaster Board ceilings: | Dulux Acrylic Sealer Undercoat | Dulux Wash&Wear Matt | Dulux Wash&Wear Matt | SD05662 |
| Plasterboard (low VOC system) |  |  |  |  |
| Internal Plaster Board ceilings: | Dulux Acrylic Sealer Undercoat | Dulux Wash&Wear Matt | Dulux Wash&Wear Matt | SD06059 |

| Substrate | 1st coat | 2nd coat | 3rd coat | Manufacturer’s Spec Sheet Ref |
| --- | --- | --- | --- | --- |
| Fibre cement products (low VOC system) |  |  |  |  |
| Timber Doors, Door and window trims. | Dulux Professional Acrylic Primer | Dulux Weathershield Low Sheen | Dulux Weathershield Low Sheen | SD 9246 |

Low gloss latex (mould resistant) – Interior

| Substrate | 1st coat | 2nd coat | 3rd coat | Manufacturer’s Spec Sheet Ref |
| --- | --- | --- | --- | --- |
| Concrete blockwork (low VOC system) | Berger Gold Label Acrylic Block Filler | Dulux Wash&Wear +Plus Kitchen & Bathroom Low Sheen | Dulux Wash&Wear +Plus Kitchen & Bathroom Low Sheen | SD 2741 |

| Substrate | 1st coat | 2nd coat | 3rd coat | Manufacturer’s Spec Sheet Ref |
| --- | --- | --- | --- | --- |
| Fibre cement products | Dulux Weathershield Low Sheen | Dulux Weathershield Low Sheen | Dulux Weathershield Low Sheen | SD 1333 |
| Timber and veneers (incl. exposed verandah beams) | Dulux Professional Acrylic Primer | Dulux Weathershield Low Sheen | Dulux Weathershield Low Sheen | SD 9246 |
| Concrete blockwork | Berger Gold Label Acrylic Blockfiller | Dulux Weathershield Low Sheen Acrylic | Dulux Weathershield Low Sheen Acrylic | SD 1555 |
| HD Galvanized steel or zinc- primed steel | Dulux Professional Galvanised Iron Primer | Dulux Weathershield Low Sheen | Dulux Weathershield Low Sheen | SD 07811 |

EXTERIOR PAINTING SCHEDULES Low gloss latex – Exterior

0811S SANITARY FIXTURES

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide sanitary fixtures, as documented.

  - STANDARDS General

Design for access and mobility: AS 1428.1 and AS 1428.2. Authorised products

Standard: Listed in the WaterMark Product Database, unless otherwise required by the network utility

operator.

Labelling

Water efficiency labelling: Provide only products conforming to and labelled to the Water Efficiency

Labelling Scheme (WELS).

- PRODUCTS

Products have been listed. Equally approved product will be considered, please refer to the substitution section within the generals section of the specification.

- EXECUTION

Refer to the services drawings for

- SELECTIONS

Basins schedule

| Property | A – All Toilets except for Prison Cell Toilets | B – AWC |
| --- | --- | --- |
| Brand | Caroma Concorde 500 or equally an approved product or equally an approved product. | Caroma Concorde 500 or equally an approved product or equally an approved product. |
| Material | Vitreous China Wall | Vitreous China Wall |
| Colour | White | White |
| Bowl size (nominal) (mm) | 495 | 495 |
| Soap recesses | Yes | Yes |
| Sides | Tile Splash Back | Tile Splash Back |
| Supports | Concealed fixing | Concealed fixing |
| Overflow | Yes required | Yes required |
| Number of tap holes | Centre | Centre |
| Trap | To suit installation. Concealed behind Full height Pedestal | To suit installation. Concealed behind Full height Pedestal |
| Location | Centre of Pedestal | Centre of Pedestal |

| Property | A – All Toilets except for Prison Cell Toilets | B – AWC |
| --- | --- | --- |
| Rim height from floor (mm) | 865mm min – refer to drawing | 865mm min – refer to drawing |

WC and Cistern pans schedule

| Property | A – Except for Prison Cell Toilets | B – AWC |
| --- | --- | --- |
| Brand | Caroma Regal 2 or equally an approved product. | Caroma Care 300 Concorde Suite. Allow for Colour Seat and Backrest (Blue Colour) and Raised Push Buttons or equally an approved product. |
| Material | Vitreous China | Vitreous China |
| Colour | White | White |
| Nominal flush (litres) | 4.5/3 Litres | 4.5/3 Litres |
| Water star rating to AS/NZS 6400 | 4.5/3 litres | 4.5/3 litres |

Urinal assemblies schedule

| Property | A – Except for Prison Cell Toilets |
| --- | --- |
| Type | Carmoa Interga Urinal or equally an approved product. |
| Flushing system | Mains Pressure - Refer to Hydraulics |
| Material | Vitreous China |
| Supports | Concealed fixing to bracket |
| Water star rating to AS/NZS 6400 | 3 Litres |

Kitchen Sink

| Property | A – All Kitchen & Kitchenettes |
| --- | --- |
| Type | Clark Advance 1.75 end Bowl or equally an approved product. |
| Material | Stainless steel - 316 |
| Accessories | Steel Bask Waste, Advanced Powder Coated Draining Basket and Advanced board and Colander Set |
| Supports | Concealed fixing to bench top from underneath |

0812S TAPWARE

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide tapware, as documented.

  - STANDARDS General

Design for access and mobility: To AS 1428.1 and AS 1428.2 Authorized products

Standard: Listed in the Watermark Product Database, unless otherwise required by the network utility

operator.

Labelling

Water efficiency labelling: Provide only products conforming to and labelled to the Water Efficiency

Labelling Scheme (WELS).

- PRODUCTS

Specified product or equally an approved product to suit purposes and installation in conjunction with the approved sanitary fixture. .

- EXECUTION

Install all tapware in accordance with manufacturer’s instructions and allow to seal and test all fittings prior to commissioning. Provide warrantees on all fixtures as required by the generals section of the specification.

- PREFERRED SELECTIONS

  - TAPWARE

Basin and Sink tapware schedule

| Property | A – Bathroom Basin | B- AWC Basin | C- Kitchen Sink | C- External Taps or Cleaners taps |
| --- | --- | --- | --- | --- |
| Brand | Enware Delabie Temp Stop Time flow Basin Pillar tap or equally an approved product. | Enware Oras Vega Basin Mixer with Accessible Extended Lever or equally an approved product. | Enware Oras Vega Basin Mixer with Accessible Extended Lever or equally an approved product. | Enware Plaza Bib Tap FI 50mm Extended or equally an approved product. |
| Catalogue number or model | TFC745 - 15 Second Delay | SLM606D-6 – 200mm long lever | SLM607D-6 – 200mm long lever | PZQ333F – 200mm long lever |
| Finish | Chrome Polished | Chrome Polished | Chrome Polished | Chrome Polished |
| Outlet type | Aerated | Aerated | Aerated | Aerated |
| Plug | Not required | Not required | Not required | Not required |
| Vandal resistant construction | YES | YES | YES | YES |
| Miniature stop valves | Yes to be located below basin | Yes to be located below basin | Yes to be located below basin | Yes to be located below basin |
| Water star rating | 3 | 6 | 6 | 6 |

Shower tapware schedule

| Property | A – Change rooms only |
| --- | --- |
| Brand | Enware Plaza Recessed Set With SP273C Chrome Hand Held Shower and Heavy Duty 900mm Wall Bar or equally an approved product. |
| Catalogue number or model | PZQ364 |
| Finish | Chrome Polished |
| Hand shower | Yes with 1.5m hose |
| Shower head | SP273C Chrome Hand Held Shower |
| Vandal resistant construction | YES |
| Water star rating | 3 |

0815P BOILING, CHILLED AND FILTERED WATER DISPENSERS

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide chilled, boiling, warm and filtered water unit dispensers from Billi complete with options and accessories, as documented. An equally approved product will be considered, please refer to the substitution section within the generals section of the specification.

  - COMPANY CONTACTS

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

- 0801 Hydraulic systems.

  - STANDARDS General

Water supply: To AS/NZS 3500.1. Heated water supply: To AS/NZS 3500.4.

Testing of products for use with potable water: ToAS/NZS 4020. Product authorization requirements: To AS 3498.

Design for access and mobility: AS 1428.1 and AS 1428.2. products

Quality management for manufacture: To AS/NZS ISO 9002. Certificate Number QEC11627.

Beverage dispensers: To ATS 5200.105.

Electrical safety certificate: Test to Victorian Electrical Safety Act 1998 Approval Number CS100600V. Watermark certificate:

Other authorized products

Standard: For non-BILLI products, provide products listed in the WaterMark Product Database, unless otherwise required by the network utility operator.

Labelling

water systems: Provide water systems rated to the Water Efficiency Labelling Scheme (WELS):

- Quadra Plus: 4 star rated.

- Sahara Plus: 4 star rated.

- KwikWash: 6 star rated.

- Sensor tap: 6 star rated.

Other tapware: Provide only products conforming to and labelled to WELS.

  - MANUFACTURER’S DOCUMENTS Technical manuals

Brochures:

Install and owners’ manuals:

  - INTERPRETATION Definitions

For the purpose of this work section, the following definitions apply:

- Ambient water: Potable water at a temperature provided by the cold water piping system, without further heating or cooling.

- Boiling water: Potable water at a temperature at or close to 100°C.

- Chilled water: Potable water at a temperature between 6°C and 15°C.

  - SUBMISSIONS Certification

General: Submit evidence that proposed products are listed in the WaterMark Product Database.

Warranties

General: Submit evidence of warranties for all proposed tapware which clearly defines the warranty period and any conditions.

Requirement: Submit the following:

- PRODUCTS

  - GENERAL Product substitution

Other products: Conform to PRODUCTS, GENERAL, Substitutions in the 0171 General requirements work section.

  - COMBINED BOILING AND CHILLED FILTERED WATER DISPENSING SYSTEMS General

Requirement: Provide the boiling and chilled under-bench drinking water unit

dispensers, including disabled compliant levers and a concealed safety switch, as documented:

BILLI Eco XL Chrome or equally an approved product.

Filters: Provide filters, as documented. Options:

Provide the options, as documented.

Accessories: Provide the accessories, as documented, to suit the documented system and the documented finish.

  - OTHER PRODUCTS Sealants

General: Use only sealants that do not support microbial growth. Colour: To match fixture.

- EXECUTION

  - INSTALLATION General

Requirement: Install level, plumb and true to line in the required location. Make sure moving parts function freely and without obstruction. Do not modify supplied units.

Manufacturer's recommendations: Install and use tools and templates as recommended by the manufacturer.

Seal: Provide resilient seals between fixtures and back nuts.

Fixing: Provide rigid fixing for dispenser base so that it does not move in normal operation. products: Conform to the Install and owners’ manuals.

Location

General: Locate to dimensions, as documented. Cutting and fitting

General: If it is necessary to cut and/or fit substrate to install an item, carry out before the surface is

finished or painted. Remove items when required for painting and protect until reinstalled. Cap or plug the open ends of pipes. Reinstall items when painting and finishing is complete.

  - COMPLETION Adjustment

General: Inspect and adjust dispensers for correct and smooth operation. If adjustment does not rectify incorrect or defective operation, replace units.

Damage

General: Inspect all work and replace or repair, to factory condition, damaged or marked fixtures and components.

Foreign matter: Inspect for presence of foreign matter. Remove if found. Replace damaged seals. Protective coatings

General: On completion of the dispenser installation remove all protective coatings and stickers and

clean surfaces. Check and clean debris from traps.

  - WARRANTIES water systems

Period: Provide warranty periods for the dispensing systems (excluding filters) as follows:

- 24 months.

  - MAINTENANCE water systems

Service: Provide maintenance recommendations for the duration of the contract maintenance period. Provide all consumables including, but not limited to, replacement filter cartridges.

- SELECTIONS

  - BILLI SYSTEMS

BILLI water dispenser systems schedule

| Property | A – Both Courthouse & Judges Residence’ Kitchens |
| --- | --- |
| BILLI model | Eco XL Chrome or equally an approved product. |
| Options |  |
| Finish | Stainless Steel |
| BILLI font | Required |
| Dispenser riser | Required |
| Sink mixer disabled lever | Not Required |
| Ventilation kit for air cooled systems | Required |
| Air cooled model | Required |
| Mixer tap | Required |
| Dispenser style | Rounded |
| Filter | 5 Micron and Sub Micron Filters |
| Tundish | Not Required |

| Property | A – Both Kitchens |
| --- | --- |
| Mounting | Bench Mounted, allow for cut-out to Clark sink between 0.75 b |

0581B SIGNAGE

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide signage systems, as documented or equally an approved product. Performance

Requirements: Provide signage as follows:

- Appropriately secured.

- Located within a clear line of vision.

- To contrast with the background.

- With clean, well defined edges or arises, and free from blemishes.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - STANDARDS Signs

Safety signs - design and use: To AS 1319.

Signs and graphics for disability access: AS 1428.1 and AS 1428.2.

- PRODUCTS

  - MATERIALS

Materials standards Aluminium:

- Plate for engraving: Alloy and temper designation 6063-0.

- For casting: To AS 1874.

Stainless steel: Surface finish designation 4 (general purpose polished). Plastics:

- PVC-U sheet: Semi-rigid sheet.

- Rigid cellular polystyrene: To AS 1366.3, class VH for cut-out shapes. Photoluminescent exit signs: To BCA E4.8(b).

- EXECUTION

  - WORKMANSHIP Production

General: Form signage and graphic items accurately with clean, well defined edges or arises, free from blemishes.

Engraving to two layer plastic laminate: Engrave lettering to expose the lower laminate.

Engraved and filled: Lettering precision cut and filled colouring material. Clean faces of all filling material.

Casting: Produce shapes free of pits, scale, blow holes or other defects, hand or machine finished if necessary.

Laser cut lettering: Individual vinyl letters with self-adhesive backing. Printed lettering: Lettering and graphic images screen/digitally printed on:

- Film with self-adhesive backing.

- Acrylic sheet.

- Aluminium plate.

- Stainless steel plate.

Large format digital printing: Lettering and graphic images screen printed film with self-adhesive backing.

Signwriting: Lettering and graphic images hand painted direct to the background by a tradesman with recognized qualifications and demonstrated experience.

Fabricated: Three dimensional, formed as follows:

- Laser cutting from solid material and hand finished as necessary.

- Moulding: Individual plastic hollow three dimensional characters and shapes formed by:

. Injection moulding.

. Vacuum forming.

- Built-up individual shapes by fabricating the faces and edges from separate pieces neatly and securely joined.

Installation

General: Install signage and graphic items level and plumb, securely mounted, with concealed theft- resistant fixings.

Self-adhesive signs: Fix free of bubbles and creases.

- SELECTIONS

  - STATUTORY SIGNS

Portable fire extinguishers – location signs

| Position | As nominated in AS 2444 clause 3.2 at every installed extinguisher nominated BCA Table E1.6 |
| --- | --- |
| Message | Prescribed graphic |
| Letter height (minimum) | 16 mm |
| Sign type | Computer generated adhesive backed vinyl graphic |
| Compliance | BCA E1.6 AS 2444 clause 3.3 Fire Brigade |

Fire blankets

| Position | As nominated in AS 2444 clause 6.4 at every blanket location |
| --- | --- |
| Message | Prescribed graphic |
| Letter height (minimum) |  |
| Sign type | Computer generated adhesive backed vinyl graphic |
| Compliance | BCA E1.6 AS 2444 clauses 6.3, 6.4 and Fig 6.1 Fire Brigade |

Non-accessible pedestrian entrance

| Position | At each non-accessible pedestrian building entrance. |
| --- | --- |
| Message | Signage incorporating the international symbol of access to direct a person to the location of the nearest accessible pedestrian entrance |

| Letter height | AS 1428.2 clause 17, Table 2. |
| --- | --- |
| Symbol size | AS 1428.2 clause 16, Table 1. |
| Sign type |  |
| Compliance | AS 1428.1 BCA D3.6 |

| Position | Sign Message | Type | Size | Image | Manufacture |
| --- | --- | --- | --- | --- | --- |
| Entry Doors | To the Left hand side of the main entry door | Silver- Acrylic with encapsulated background colour | 235mm x 180mm x 3mm |  | Braille Supplies www.braillesignsupplies.com.au/ or equally an approved product. |
| Outside reception |  | Silver- Acrylic with encapsulated background colour | 235mm x 180mm x 3mm |  | Braille Supplies www.braillesignsupplies.com.au/ or equally an approved product. |

| Outside reception (left and side of window) | Indicating location of Accessible Toilets | Silver- Acrylic with encapsulated background colour | 235mm x 180mm x 3mm |  | Braille Supplies www.braillesignsupplies.com.au/ or equally an approved product. |
| --- | --- | --- | --- | --- | --- |
| Outside AWC Public Toilets | Male Toilets with Ambulant Compartment | Acrylic with encapsulated background colour – Right hand transfer. | 235mm x 180mm x 3mm |  | Braille Supplies www.braillesignsupplies.com.au/ or equally an approved product. |
| Outside Male Public Toilets | Male Toilets with Ambulant Compartment | Acrylic with encapsulated background colour | 235mm x 180mm x 3mm |  | Braille Supplies www.braillesignsupplies.com.au/ or equally an approved product. |

| On Cubical Door of the Male Ambulant Compartment | Male Ambulant Compartment | Acrylic with encapsulated background colour | 235mm x 180mm x 3mm |  | Braille Supplies www.braillesignsupplies.com.au/ or equally an approved product. |
| --- | --- | --- | --- | --- | --- |
| Outside Female Public Toilets | Female Toilets with Ambulant Compartment | Acrylic with encapsulated background colour | 235mm x 180mm x 3mm |  | Braille Supplies www.braillesignsupplies.com.au/ or equally an approved product. |
| On Cubical Door of the Female Ambulant Compartment | Female Ambulant Compartment | Acrylic with encapsulated background colour | 235mm x 180mm x 3mm |  | Braille Supplies www.braillesignsupplies.com.au/ or equally an approved product. |
| Non staff Office entry | Staff Only + Do Not Obstruct or Keep Open | Acrylic with encapsulated background colour | 235mm x 180mm x 3mm |  | Braille Supplies www.braillesignsupplies.com.au/ or equally an approved product. |

| Custom Braille Signs to each of the offices | - Meeting Room - Office - Briefing Room | Acrylic with encapsulated background colour | 235mm x 180mm x 3mm |  | Braille Supplies www.braillesignsupplies.com.au/ or equally an approved product. |
| --- | --- | --- | --- | --- | --- |
|  | - Training room |  |  |  |  |

| Where First AID kit is kept. | MJCA to advise. | Acrylic with encapsulated background colour | 235mm x 180mm x 3mm |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Cleaners Cupboard | Acrylic with encapsulated background colour | 235mm x 180mm x 3mm |  | Braille Supplies www.braillesignsupplies.com.au/ or equally an approved product. |

Building Signage:

Contractor to allow for 500mm High text x 50mm deep 3D laser Cut Corten steel with mounting frame to be fixed to the main entry of the Building. Text: MJCA to provide

Building Plaque:

Contractor to allow for 300high x 200mm Metalex Laser Engraved Plaque glued to a Natural timber backing and screwed to the blockwall.

0195P TACTILE INDICATORS AND STAIR NOSINGS

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide tactile indicators and edging, as documented at existing floor step locations at the main entry door locations. Whilst the DTAC product has been specified, substitution of an equally approved product will be considered in accordance with the substitution criteria within the generals section of the specification.

  - COMPANY CONTACTS

DTAC technical   contacts Website: www.dtac.com.au/contact

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - STANDARDS General

Tactile indicators: To AS/NZS 1428.4.1. Stair nosing: To AS 1428.1.

  - MANUFACTURER'S DOCUMENTS

Technical manuals

Technical data sheets and laying guides: www.dtac.com.au/resources DTAC Stair & Tread Edging: www.dtac.com.au/product/edging

  - SUBMISSIONS

Slip resistance

Tactile indicators and edging: Submit evidence of conformance to AS 4586. Testing

Luminance contrast Testing: Submit evidence of conformance to AS/NZS 1428.4.1 Appendix E and

AS 1428.1 Appendix B.

- 30% contrast between the tread strip and adjoining surface. Warranties

Tactile indicators and edging: Submit the of DTAC product and installation warranties.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the completed substrate ready for tactile indicators and edging installation.

- PRODUCTS

  - GENERAL Product substitution

Other products: Conform to PRODUCTS, GENERAL, Substitutions in the 0171 General requirements work section.

  - DTAC PRODUCTS

Edging products

Material: Extruded aluminium. Finish: Hard anodized.

- EXECUTION

  - EDGING PRODUCTS Fixing

Substrate: Concrete

Fixing: 3 per tread strip - M4.5 x 50mm long Countersunk Ramset Zinc Plated Dyna bolt

  - COMPLETION Warranties

Conditions: Installation by DTAC or DTAC approved by installer. Warranty period: 2 years.

- SELECTIONS

.

  - DTAC EDGING

Stair Treads and slab edging Application: At all floor step locations

Size: 75mm x 3mm thick with 10mm face edging Colour: Black hard Infill

Code: DTAC Urban Edge Mill finished Aluminium profile– DTAC Product code DE075MF or equally an approved product.

http://www.dtac.com.au/

Ensure the selected tread insert achieves 30% contrast with the background

0457 EXTERNAL SCREENS

- GENERAL

  - RESPONSIBILITIES General

Requirement: Provide external screens, as documented. Performance

Requirements: Conform to the following:

- Plumb, level, straight and true within the building tolerances of the structural system.

- Undamaged and free of surface defects or distortions.

- Fixed or fastened to the building structure.

- Able to resist wind and other actions without vibration or permanent distortion.

  - CROSS REFERENCES General

Requirement: Conform to the following:

- 0171 General requirements.

  - STANDARDS General

Aluminium framed sunscreens, awnings and shutters:

- Stress analysis of members: To AS/NZS 1664.1 or AS/NZS 1664.2. Horizontal screen loadings: To AS/NZS 1170.1 Table 3.2.

Electrically operated external louvres and blinds:

- Drive motors: To AS/NZS 60335.2.97. Access for maintenance: To AS 1657.

  - INTERPRETATION

Abbreviations

General: For the purposes of this work section the following abbreviations apply:

- BMS: Building Management Systems.

- PVC-U: Unplasticized Polyvinylchloride. Definitions

General: For the purposes of this work section, the following definitions apply:

- Louvres - continuous: Louvres that run continuously past, and are supported by, concealed framing or brackets.

- Louvres - horizontal: Louvres that span between frames stiles, mullions or vertical supports.

- Louvres - vertical: Louvres that span between frame heads and sills, or horizontal supports.

- Membrane (tensioned membrane structures and external screens): A thin and flexible sheet of fabric material.

- Screen: Includes sunscreens, trafficable sunscreens, external louvres and blinds, shutters, awnings and pergolas fixed to building facades or openings to control sunlight and/or provide privacy, to screen plant and equipment, or to provide an architectural feature. It applies to fixed, adjustable, operable and automatically controlled types.

- Tensioned membrane: A thin cloth or sheet that is held in a predetermined 2- or 3-dimensional shape under permanent tension. The shape and the tension are interrelated and designed to safely carry the permanent and imposed loads (such as those resulting from wind actions) in a predictable manner.

  - SUBMISSIONS

Operation and maintenance manuals

Requirement: At completion, submit the screen manufacturer’s recommendations for operation, care and maintenance.

Samples

General: Submit samples of the following:

- Sections proposed for frame members, louvres, accessories, cover panels and trim.

- Joints made, using proposed techniques.

- Colour samples of prefinished production material (e.g. anodized or organic coated extrusions or sheet, glazing, infill panel material or fabric), each at least 200 x 200 mm, showing the limits of the range of variation in the selected colour, if any, for each component of the screens specified.

- Accessory and hardware items documented descriptively or by performance (i.e. not proprietary items). Include handles, operators, controls, switches, sensors, motors, fixing clips, anchor brackets and attachments, fixings, gaskets and weather seals.

Labelling: Label each sample, giving the brand and product name, manufacturer’s code reference, date of manufacture and intended building location.

Sample installations: Install the designated typical screens in their final position incorporating at least one example of each component in the system, including attachments to the structure, flashing, caulking, sealing, infill materials, operating hardware and controls.

Sealant compatibility

Compatibility statements: Submit statements from all parties to the installation that certify the compatibility of sealants with screen components, finishes and all substrates.

Shop drawings

General: Submit shop drawings to a scale that best describes the detail, calculations and specifications conveying the following information:

- Layout of the screen assembly (sectional plans, vertical sections, and elevations of each building face where screens are to be installed).

- Full size sections of typical members including mullions, transoms, subheads, sills, subsills, louvres, infill panel material or fabric, beads, bearings, linkages, exposed fixings, sealant beads, glazing gaskets, splice plates, trays and cover strips, with notes specifying the proposed materials.

- Lubrication requirements for adjustable or operable screens.

- Method of assembly, including isometric or axonometric and exploded views of typical framing junctions, showing panel to panel joints (for modular systems).

- Method of installation, including the following:

. Location and magnitude of reactions to be accommodated by the support structure.

. Type and location of fasteners and other attachments to be cast or otherwise built into the building structure.

. Erection tolerances.

. Accurate locations and full size details of machined slots, keyholes and other penetrations in frame extrusions for lifting and installing the units.

. Junctions and trim to adjoining surfaces.

. Caulking and flashing.

. Locations of visible heads of fasteners.

- Provision for differential vertical or horizontal movements, including the following:

. Thermal expansion and contraction.

. Frame deflections.

- Details of motor and operating mechanism enclosures.

- Method of draining the assembly, including details showing the following:

. Pressure equalised drained joints.

. Location, number and size of weepholes.

- Connection points to rainwater or stormwater systems.

- Hardware, fittings and accessories including window cleaning restraints and visible heads of fasteners.

- Infill panel stiffening.

- Location and power requirements of motors, sensors and controls.

- Wiring diagrams of control systems and how they connect to BMS.

- Scale drawings and descriptions of prototype external screens. Subcontractors

General: Submit names and contact details of the proposed manufacturers and, if the manufacturer is

not the installer, the installers recommended by the manufacturers. Warranties

Screens: At completion, submit the manufacturer’s published product warranties.

  - INSPECTION Notice

Inspection: Give notice so that inspection may be made of the following:

- Sample assembly.

- Fabricated screen assemblies at the factory ready for delivery to the site.

- Fabricated screen assemblies delivered to the site, before installation.

- Commencement of installation of screen assemblies.

- Completion of installation.

- PRODUCTS

  - MATERIALS GENERALLY Structural steel

Design and materials: To AS 4100. Welding: To the AS/NZS 1554 series. Galvanizing: To AS/NZS 4680.

Cables

Requirement: Preload cables by cyclic loading to achieve a uniform modulus of elasticity and a linear stress/strain relationship within the working range. Use a swaging system to achieve a breaking strength of terminals not lower than the minimum design strength of the cable system.

Materials: Stainless steel type 316 or galvanized steel. Fabric

Supply: Supply fabric by a single manufacturer as part of a single batch.

Inspection: Check each roll of material for flatness, faults in the woven fabric and the coatings, where present, by visual inspection in directional sunlight at a distance of 4 m and by passing the membrane over a uniformly illuminated surface.

Perimeter reinforcing: Reinforce the perimeter of each with UV stabilised polyester, coated with PVC and incorporating pockets for the tension cables.

  - METAL FINISHES

Surface preparation Standard: To AS 1627 series. Anodised

Standard: To AS 1231.

Thickness: ≥ 15 microns to 20 microns. Hot-dip galvanizing

Coating mass/thickness minima: To AS/NZS 4680.

  - PROPRIETARY SCREENS OR EQUALLY AN APPROVED PRODUCT

General

Requirement: Provide UV Resistant Kaynmaile panel materials mounted using a 12.5mm Stainless steel tube with proprietary stainless steel hooks at 400 crs sub frame. Allow for turn tensioning turnbuckles as required to:

- To withstand imposed actions and wind actions for the location without failure or permanent distortion, and without panel flutter.

- To shed water without pooling. Framing materials

Requirement: Provide frames fabricated from solid or hollow metal sections. Fix to fastener brackets or

arms mounted on the face of the building, and brace as necessary with stays, including tensile elements such as wire cables and turnbuckles.

Intermediate fixing:

Allow to run hidden supports continuously through the mesh horizontally every 3–5 metres and fix back to the building.

  - TIMBER SCREENS (VERTICAL AND HORIZONTAL) General

Requirement: Provide 200x 50 minimum horizontal and vertical timber screens fixed to sub frames with 8mm x 75mm wide Mild Steel cleat for each bolt fixing. Allow to fix sub frame to primary frame. The frame and

- To withstand imposed actions and wind actions for the location without failure or permanent distortion, and without panel flutter.

- To shed water without pooling.

- EXECUTION

  - FABRICATION

Aluminium fabrication and construction Standard: To AS/NZS 1664.1 or AS/NZS 1664.2. Fasteners

Requirement: Provide fasteners of sufficient strength and quality to perform their required function.

Joints

Requirement: Make accurately fitted tight joints so that neither fasteners nor fixing devices create pressure indentations that are visible on exposed faces. Where heads of fasteners are unavoidably visible, finish them to match the adjacent finished surface.

Protection

Corrosion protection: Provide protection against corrosion which may be caused in metals by products or processes normally employed on a building site or by normal atmospheric or other ambient conditions and by-products including rainwater, potable and non-potable water, airborne salt and airborne pollution.

Durability: Provide materials resistant to exposure to weather and UV radiation so that their colour, surface finish, flexibility and water resistance are maintained.

Temporary measures: Do not use adhesive tape, film or paper, or applied coatings liable to bond to the substrate, when exposed to sunlight or weather, as temporary measures to protect screen components during the course of the works. If temporary measures are used, remove all traces, particularly from contact mating surfaces before joining up.

Operation

Requirement: Provide moving parts which operate freely and smoothly, without vibration, rattling, binding or sticking, and at correct tensions or operating forces. Lubricate if appropriate.

  - WELDING

General

Quality: Provide finished welds descaled and free of surface and internal cracks, slag inclusion and porosity. Provide continuous welding unless permanently concealed.

Restrictions: Do not weld as follows:

- On site.

- On finished surfaces.

- Next to a finished surface or glass, unless the adjacent surface is protected from damage.

  - INSTALLATION

Installation tolerance Alignment:

- Maximum deviation of any member from its true alignment (plumb, level, or line of slope): 1:1000, up to a maximum of 10 mm in a continuous run of members in one direction.

- Maximum misalignment between adjoining members: 1 mm. Position:

- Maximum deviation of any part from its true position: 10 mm Marking

Requirement: Before the separate parts of the screens are delivered to the site, provide suitable and

sufficient marks or other means for identifying each part, and for showing its correct location and orientation, when installed.

Reference lines and marks

Requirement: Provide on each floor, in agreed locations, accurate perimeter offset reference lines, plumb with corresponding lines on other floors, and height benchmarks.

Cleaning

Requirement: During erection, promptly remove foreign matter from the screens without damage to finishes. Do not use abrasive cleaners or acid.

  - COMPLETION Cleaning

Method: Clean all visible surfaces with soft clean cloths and clean water or approved cleanser, finishing with a clean cloth. Do not use abrasive or alkaline material.

APPENDICES

APPENDICES : CONTAIN EXTRA INFORMATION ON RECOMMENDED SPECILISED BUILDING MATERIALS & EQUIPMENTS (MANUFACTURER’S):-

APPENDIX A – RECOMMENDED CEILING SYSTEM

APPENDIX B – WON-DOOR FOLDING PARTITION APPENDIX C – COURTROOM FURNITURE & EQUIPMENT

APPENDIX D - HOLDING CELL TECHNICAL INFORMATION (CELL SECURITY LTD CATALOGUE)

ACCORDION FOLDING PARTITIONS PART 1 - GENERAL

  - SUMMARY OF WORK

    - Furnish and install all accordion folding partitions shown on the drawings and specified herein.

    - All headers, support structures, surrounding insulations, jambs, pocket doors, blocking, architraves and trim shall be furnished and installed by others.

    - Peripheral leakage of sound through ceiling, floor, other walls, ducts or vents is to be made good by others.

  - QUALITY ASSURANCE

    - Installation shall be carried out by factory trained personal.

    - DuraSound model shall have the laboratory sound rating indicated (STC43), when tested in accordance with the requirements of ASTM E-90.

  - SUBMITTALS

    - Specification listed as either Single Parting, Bi-Parting or Rolling Lead Post.

    - Indicate Clear Opening Width & Clear Opening Height of partitions (height is from underside of header to top of flooring coverings).

    - Indicate required stack depth & pocket width (if applicable).

    - Show installation details, layout and any optional electrical requirements.

  - DELIVERY, STORAGE & HANDLING

    - Deliver to job site in manufacturer’s original and unopened packaging.

  - COORDINATION BY GENERAL CONTRACTOR

    - Coordinate the efforts of the various trades affected by the work of this section. Assure accurate construction of header, jambs and trims. Provide “As-Built” Final Fabrication dimensions of each opening on forms provided by manufacturer.

    - Supervise unloading and handling. Store boxes flat (no more than three high) in a dry area and protect from elements that may damage the materials. Replace any damaged materials at no additional cost to the owner.

    - OPTION: If section 2.02 O is specified, the following paragraph should be included: Permanent power shall be in place for final connection when partitions are installed. Insure access to and proper clearance for motor operators.

  - WARRANTY

    - Materials shall be warranted against defects and workmanship for a period of three (3) years from the date of installation. DuraSound to be installed by factory trained personal or warranty is void.

  - MANUFACTURER

    - Folding partitions shall be DuraSound model as manufactured by Won-Door New Zealand Limited and Won-Door Australia Pty Ltd.

  - MATERIALS

    - Operation: Shall be top hung and manually operated (no floor track)

    - Construction: Shall consist of two parallel accordion-type folding partition walls of panels independently suspended from top track. With no pantograph or interconnections except for the Lead Post.

    - Panels: Shall be formed of 115mm wide x 1.6mm thick aluminium, V-grooved and powder coated for added strength. Panels shall be connected by full height extruded vinyl hinges.

    - Insulation: Interior surfaces of both walls of partition panels shall be completely covered with a continuous acoustic blanket of high-density foil-backed fibreglass insulation fastened to each panel with patented stainless-steel spring-loaded clips.

    - Suspension & Track System: Shall consist of two extruded aluminium tracks spaced 152mm apart (or 203mm for motorized operation) on centre of header. Tracks to be attached to the overhead structural support.

    - Trolleys: Each panel shall be suspended by a hardened-steel hanger pin and a 25mm diameter nylon-tyred ball bearing roller. Each Lead Post shall be suspended by a 12-wheel ball bearing aluminium trolley.

    - Lead Posts: Shall be of 3.15mm extruded aluminium and shall be connected to the partition by specially designed aluminium Wing Panels.

    - Striker (receiver): Shall be of 1.9mm extruded aluminium and shall be secured to wall with fasteners by Won-Door. Striker face to be fitted with full height acoustic seal.

    - Perimeter Seals: Shall consist of continuous extruded vinyl sweep strips attached to the top & bottom of the partition. Leading edges of Lead Posts and Striker receiver posts shall be acoustically sealed by extruded vinyl interlocking seals.

    - Handle: Lead Post Handles shall include standard grip-type aluminium handles and sliding latch to affect closure.

Note: LOCK AND KEY mechanism can also be specified as OPTON (refer 2.02 M).

    - Hanging Weight: shall be 18kg per m2.

    - Fixings: All necessary track and fixing screws are supplied

    - OPTION: LOCK AND KEY: DuraFlex can be specified with a LOCK AND KEY mechanism for added security. Standard specification is for a Latch closure mechanism only.

    - OPTION: DURASOUND WITH STABALIZER BAR: For DuraSound models over 3.4m tall. Shall consist of a top supported, internally mounted diagonal brace connected to the Lead

Post for proper alignment during operation and latching. Requires a minimum 368mm wide header (refer manufacturers Technical Details on DuraSound with Stabilizer Bar).

    - OPTION: MOTOR OPERATION:

      - Operation: Motor operated folding partition shall be driven by means of a roller chain attached to the stabilizer bar trolley. An internally mounted stabilizer bar shall keep Lead Posts plumb and in proper alignment during operation and insure a tight-fitting closure without the use of mechanical latches.

      - Assembly: Shall consist of a 115/230 volt single-phase A.C. motor or 208 volt 3-phase

A.C. gear motor, reversing electromagnetic starter and limit switches. Size of each motor shall be determined by manufacturer to insure proper operation.

  - ACOUSTICAL PERFORMACE

    - Sound transmission class (STC) shall be STC43 when tested in accordance with requirements of ASTM E-90.

  - COLOURS

    - Powder coat finish colours to be selected from manufacturer’s standard colours.

PART 3 – EXECUTION

  - PREPERATION BY GENERAL CONTRACTOR

    - Openings shall be constructed to the dimensions specified & approved.

    - Openings to be straight, plumb and level.

    - Headers to be parallel with the finished floor to within +/- 5mm over the entire length of the opening.

  - INSTALLATION

    - Install partitions in accordance with manufacturer’s instructions.

    - Upon completion of the installation, the General Contractor shall protect partitions from damage and replace or repair subsequent damage so that the partitions are acceptable to the architect, at no additional cost to the owner.

PART 4 – FURTHER SPECIFIERS INFORMATION

  - ELEVATION DESIGN SPEC

    - Single Parting – single partition which stacks entirely to one side when in the open position and latches closed at the other side.

    - Bi-Parting – two partitions which stack to each side when in the open position and latch together in the middle.

    - Rolling Lead Post – single partition which can latch closed at both sides and stack open at both sides.

  - PLAN VIEW DESIGN SPEC

    - Straight door only – all track types can be specified.

  - TRACK DESIGN SPEC

    - T-11 Standard Track with Soffit (310mm wide).

    - T-11 Standard Track without soffit (305mm wide).

    - T-02 Suspended Ceiling Track (282mm wide).

    - If 2.02 Option N or O is specified, then T-09 Track system with Soffit (364mm wide).

Tracks generally supplied as standard to match colour of partition. Arctic White, or other colours can be specified for the Track system to tie in with surrounding ceilings/bulkheads. Refer manufacturers Technical Details for further information on track types and designs.

  - STACK DIMENSIONS

    - Stack Depth for DuraSound is 146mm per lineal meter of opening plus 152mm for Lead Post. For Bi-Parting partitions use half the width of the opening.

    - Thickness of the door when Stacked Open is 280mm.

  - MAXIMUM DIMENSIONS

    - 3.4m high Standard model DuraSound

    - Over 3.4m high DuraSound with Stabilizer Bar

    - Over 4.0m high Motorized Operation required (max 5m high) Note: almost no limitations in width.

  - REQUIRED DIMENSIONS FOR MANUFACTURE

    - Fully detailed and authorised measurements of the opening are required on Fabrication Measurements Forms provided by Won-Door.

    - Typical opening measurements required:

      - Width at top of opening.

      - Height at 2m intervals across length of opening.

      - Floor Covering thickness.

Copyright © 2019 by Won-Door New Zealand & Won-Door Australia, all rights reserved. END OF SECTION

### Why DuraSound & DuraFlex Folding Partitions?

The most cost effective partition on the market.

DuraSound (acoustic) & DuraFlex (non-acoustic) aluminium accordion folding partitions increase building utility & flexibility by offering a range of sight and sound barrier solutions for your space division. Also known as room dividers, accordion walls and concertina doors. These versatile accordion partitions are very competitively priced, cost less to install and maintain, and last years longer than any other folding partition on the market.

A great solution for dividing and separating spaces in many applications including

| Educational Facilities | Churches | Community Centres |
| --- | --- | --- |
| Early Childcare Centres | Sports Clubs & Gyms | Offices |
| Conference & Meeting Rooms | Elderly Care Facilities | Multi-Purpose Areas |

Won-Door is the world’s largest manufacturer of Accordion Type Folding Partitions and is recognized worldwide as the industry leader in quality & design.

### DuraSound

BROCHURE TECHNICAL DETAILS SPECIFICATIONS

Available in heights up to 6m and virtually no limitations for width (for doors over 4m tall Motorization is recommended).

PHOTO GALLERY ‣

### DuraFlex

BROCHURE TECHNICAL DETAILS SPECIFICATIONS

Available in heights up to 3.4m and virtually no limitations for width (Manual operation only).

Features of DuraSound & DuraFlex Wall Dividers

##### DURABILITY & LIFE SPAN

30+ year life span! With no internal parts to get damaged and patented v-grooved aluminium panels, Won-Door is designed to last as long as the buildings in which they are installed.

##### SMOOTH, EFFORTLESS OPERATION

Lightweight aluminium partitions are easy to operate (18kg m2 for DuraSound & 9kg m2 for DuraFlex).

##### EASY INSTALLATION

DuraSound to be installed by qualified personal with minimum inconvenience, while DuraFlex can be installed by customer or Won-Door personal if required.

##### IN-PLACE REPAIRABILITY

With simple tools, in-house maintenance staff can repair any damaged parts by simply removing and replacing panels, hinges or rollers.

##### NO FLOOR TRACK

Won-Door’s design eliminates the need for troublesome floor tracks and exposed hardware.

##### COLOURS & FINISHES

Attractive and modern powder coat finishes available.

4

### DuraSound & DuraFlex Room Dividing Configurations

Won-Door’s unique design provides numerous possibilities for dividing up spaces. From traditional Single Parting & Bi-Parting options to multiple doors intersecting.

### DuraSound construction & acoustic performance

Won-Door DuraSound has an STC43 acoustic rating and is ideal for separating and dividing off areas where a sound barrier is required. DuraSound is commonly specified in Classrooms & Meeting Rooms, Conference Centres and many other applications where larger areas are divided into smaller multi-use spaces. DuraSound provides superior acoustic insulation and durability at a very cost effective price.

DuraSound acoustic partitions operate on a patented twin track system. Unlike other pantograph style partitions, DuraSound is engineered with no internal parts to get damaged or fail under operation. Won-Door DuraSound is a double walled aluminium partition with an internal air cavity designed to absorb a tremendous amount of abuse without showing a dent or scratch on the door. This air space between the two aluminium walls creates an effective cushion against even the most severe blows.

Available in heights up to 6m and virtually no limitations for width (for doors over 4m tall Motorization is recommended).

### Acoustic Performance

DuraSound has an STC43 rating.

What is STC? Sound Transmission Class (STC) is an integer rating of how well a building partition attenuates airborne sound. Numerical values for STC are comparable to Rw values (Weighted Sound Reduction). As a guideline it is commonly accepted that an Rw33 (or STC33) rating is suitable for primary schools and small meeting rooms. While an Rw38 (or STC38) is appropriate for meeting rooms, church halls and classrooms.

The performance of an acoustic partition also depends on the building in which it is installed. Peripheral leakage can occur through the ceiling, floor or even through air conditioning vents and can limit the effectiveness of an acoustic partition.

### Take a closer look

##### DuraSound

NEXT GENERATION GYM

SEE PROJECT

##### DuraFlex

0800 688 555

##### 09 634 2242

info@won-door.co.nz

FLEX FITNESS PUKEKOHE

SEE PROJECT

Unit 1 29B Alfred Street PO Box 13-381 Onehunga Auckland, New Zealand

Follow Won-Door



Home | Back to top | Contact Us

© 2010 Won-Door. All rights reserved. For Won-Door by Studio7

2021

Bespoke Courtroom Furniture | Specialist Joinery

courts

purpose-built workshops across 4 units

established in 2003

in-house design, manufacture & installation

employing over 90 staff

2 | jcareydesigncourts

125+ years collective court-specific experience

constructionline, safecontractor & FSC accredited

2021 edition

From our inception in 2003 the jcareydesign brand has become synonymous with the design, manufacture and installation of courtroom furniture.

In this our 2021 publication, we have categorised our portfolio in relation

to specific areas within a court building. Our in-house team of designers have unparalleled knowledge within this sector, and work in sync with architects, interior designers, and main contractors to accomplish all aspects of the brief, provide best possible value, and fully compliant solutions. Providing functional, durable and aesthetically pleasing furniture for modern-day courtrooms and ancillary areas, to last for many years to come.

For any enquiries, no matter how large or small please call 01535 637968

Jonathan Jacob-Carey	Steve Draper

CONTENTS

04 courtroom furniture

24 docks | public galleries

- cable management

- audio visual

- acoustic treatments

- crests

- courtroom seating

34 signage

36 front of house

42 front of house seating

44 retiring rooms

48 staff offices

52 teapoints

- auxiliary areas

- demountable dais

SPECIFICATION

We offer a vast range of products that we can alter design styles or specifications to meet any budget. Over the years to help simplify project pricing we have developed a bronze, silver and gold pricing guide.

GOLD SILVER

BRONZE

This key is used throughout our brochure.

jcareydesigncourts | 3

## courtroom

furniture

4 | jcareydesigncourts

GOLD

The four crown courts shown formed part of our full Civil Justice Centre furniture fit-out which also included a further three magistrates’ and two civil courtrooms, six retiring rooms and the judicial dining area.

GOLD

These images reflect one of our most distinguished and intricate styles, which can be adapted to suit a variety of finishes and modified

to suit each bespoke project’s requirements and budget.

jcareydesigncourts | 5

courtroom

furniture

6 | jcareydesigncourts

These three large magistrates’ courtrooms were finished in solid maple and matching crown cut veneer. Consistent quality applies throughout our ranges, with the level of detail varying to suit the project budget.

SILVER

jcareydesigncourts | 7

courtroom

furniture

# 320

number of people that can be seated at any one time.

# 20

SILVER

quantity of individual demountable units which together form linked benches.

8 | jcareydesigncourts

A fully demountable public enquiry environment, suitable for relocation and re-use in future cases.

This high-profile enquiry centre is just one example of the options available within our silver range, combining both oak and walnut veneers. This building was fitted

SILVER

out within a very tight timescale. We also furnished a huge public and press annex where proceedings were closely followed.

jcareydesigncourts | 9

courtroom

furniture

This was part of a large scheme for 12 hearing rooms and ancillary areas. Dark grey marmoleum is inset into the magistrates’ benches

BRONZE

providing a comfortable but durable writing surface.

For any queries, no matter how large or small please call 01535 637968

10 | jcareydesigncourts

BRONZE

Furniture within the court well can be reconfigured to suit different case scenarios, fitted with hidden sturdy roller castors for ease of movement.

jcareydesigncourts | 11

courtroom

furniture

12 | jcareydesigncourts

SILVER

Veneer selection played an important part within these courtrooms with a combination of fixed and loose furniture.

SILVER

Inlaid marmoleum dictates the hierarchy within the courtroom, from Judges and clerks through to advocates and witnesses.

jcareydesigncourts | 13

courtroom

furniture

GOLD

These Scottish Sheriffs’ Courts feature traditional raised and fielded panels on every fascia.

14 | jcareydesigncourts

GOLD

Although the furniture layouts in Scottish High Courts differ slightly to those in England, we always ensure compliance with Court and Tribunals Design Guide rules and regulations.

jcareydesigncourts | 15

courtroom

furniture

For this scheme we provided all specialist fitted furniture for three large Crown Courts with secure docks,

SILVER

the largest of which seated up to 20 defendants. Raised public galleries were set above, allowing each case to be safely observed from this viewing point.

16 | jcareydesigncourts

For any queries, no matter how large or small please call 01535 637968

SILVER

Feature walnut horizontal banding contrasts with oak veneer and ties in with the large backboard behind the Judges bench, adding to the grandeur of the room.

jcareydesigncourts | 17

courtroom

furniture

SILVER

White-washed oak is used throughout this contemporary courtroom, within Scotland’s first

purpose-built Justice Centre.

18 | jcareydesigncourts

SILVER

A large advocates table sits within the main well of the room, in one of six large Sheriffs’ Courts.

For any queries, no matter how large or small please call 01535 637968

jcareydesigncourts | 19

courtroom

furniture

Part of a large scheme over 4 floors and

BRONZE

4 blocks, 26 courtrooms were designed, manufactured and installed.

20 | jcareydesigncourts

BRONZE

Items provided included acoustic wall panelling, retiring rooms, counters, kitchenettes, office spaces including desking and lockers, meeting rooms, public circulation spaces and seating.

For any queries, no matter how large or small please call 01535 637968

jcareydesigncourts | 21

courtroom

furniture

SILVER

These minimalist courtrooms were installed across multiple floors of a new-build Civil Justice Centre, the first major court complex built in Britain for over 100 years.

22 | jcareydesigncourts

# 3

pilot schemes completed

in England and Wales for the Courts of the Future.

SILVER

We were honoured to be involved in providing these modern courtroom environments, with the emphasis being placed on a more sustainable and adaptable solution; this fully demountable courtroom can be relocated if required.

jcareydesigncourts | 23

docks and public galleries

SILVER

Strict guidelines must be adhered to when designing for these highly secure environments, utilising specialised high security glazing to

provide a compliant, robust solution.

24 | jcareydesigncourts

GOLD

We design and manufacture docks in a wide range of shapes and sizes to suit the specific room layouts

and stair wells.

jcareydesigncourts | 25

cable

management

26 | jcareydesigncourts

Fully bespoke timber cable flaps or flush aluminium power

GOLD

and data units cater for plugging straight into the bench top.

SILVER

Integrated power and data units sit proud of the

worktop, often accompanied by desktop microphones.

BRONZE

Solid timber cable ports keep trailing wires tidy, sockets are then situated below the desktop or within a nearby floor box.

audio

visual

Discrete AV cupboards securely house digital sound recording equipment for within the courtroom, alongside video link equipment to connect remotely with an external witness or defendant.

jcareydesigncourts | 27

## acoustic

treatments

28 | jcareydesigncourts

###### Acoustic sound absorbing wall and ceiling panels are often a requirement within a new courtroom fit-out, with styles including traditional raised

& fielded, or more contemporary slotted, perforated, slatted, grooved or coloured acoustic panels.

## crests

###### Crests display the official stamp of HMCTS depicting the royal coat of arms within each finished courtroom, and are available in a variety of sizes, styles and finishes.

Traditionally cast resin

GOLD

crests are then hand-painted in full colour, with detailed shadows and outlining.

Pewter or stainless steel projects off the

SILVER

backboard creating a more contemporary   appearance to newly formed courtrooms.

BRONZE

Standard colour cast resin or slimline stainless steel crests provide a more subtle solution.

jcareydesigncourts | 29

## courtroom

seating

30 | jcareydesigncourts

GOLD

From Judges to clerks, jury, witness and advocates alongside fitted solutions for docks and public galleries, we offer numerous courtroom seating solutions.

jcareydesigncourts | 31

courtroom

seating

# 300+

different seating ranges available.

# 2,000+

largest quantity of chairs supplied for one project.

SILVER

Chairs can be provided in a variety of materials including timber, chrome or aluminium frames, with fabric, leather or vinyl upholstery. We can also offer extremely durable non-upholstered seating.

32 | jcareydesigncourts

BRONZE

Seat frames can be cantilevered, cross-based, four-legged or beam-mounted, and can be fitted with castors, feet or glides.

jcareydesigncourts | 33

signage

34 | jcareydesigncourts

Our largest external crest installation at this flagship Civil Justice Centre was almost 4 metres in height with LED lighting included.

S14 Location and crest front door

S15 Building name, crest, directional signage	S11 Welcome sign foyer/reception

We worked closely with the Court of the Future team to develop a system of easy to understand signage for use across all court buildings, colour coded by floor.

S7 Listings directory

S2 Main floor level directory

S3 Floor level directory public area

S9 Suspended directional

S5 Information coloured panel

S6 Door sign

S8 Wall mounted directional

S10 Door sign public areas S16 Door sign private

S1 Main reception

S4 Wayfinding coloured

S12 Security notice

S13 Hearing room

S17 Toilet signage

jcareydesigncourts | 35

front

of house

American white oak veneer, grey marmoleum and Corian® are used on this counter with feature stainless steel details.

36 | jcareydesigncourts

#### High volumes of visitors pass through these areas daily, providing a vital point of contact between staff and the general public.

jcareydesigncourts | 37

front

of house

A series of counters fulfilling different purposes were provided within this Civic Centre and Civil and Family Court Building.

Public counters

From a large main reception to secure payment counters, rates, births and deaths, youth offenders and ushers, we catered accordingly for each specific use.

38 | jcareydesigncourts

jcareydesigncourts | 39

front

of house

Anti-bandit glass provides an extra level of security between court personnel and members of the public.

40 | jcareydesigncourts

Counters are divided into bays set between partition walls, to provide privacy for the users.

Purpose made security search tables are available at varying heights with inset slide bars and lockable storage below.

jcareydesigncourts | 41

## front of house seating

42 | jcareydesigncourts

Waiting areas can vary in style, from private acoustic sofas through to open seating spaces.

jcareydesigncourts | 43

## retiring

rooms

44 | jcareydesigncourts

For any queries, no matter how large or small please call 01535 637968

GOLD

Retiring rooms are an integral part within a courthouse building and can be tailored to meet individual Judges’ requirements/needs.

jcareydesigncourts | 45

retiring

rooms

SILVER

Executive desks, storage, meeting tables, coat hooks, pedestals and bookcases were all designed, manufactured and installed to suit the available space.

46 | jcareydesigncourts

For any queries, no matter how large or small please call 01535 637968

BRONZE

Feature storage units can accommodate space for robes, integrated electrical appliances and essential reference law books.

jcareydesigncourts | 47

## staff

offices

Key to the running of a court building, offices require intelligent, flexible space planning to maximise

BRONZE

productivity and organisation.

48 | jcareydesigncourts

BRONZE

Desking, storage, shelving, lockers, seating, teapoints, breakout areas and dining tables were all provided for the staff working within these Civic, Family Court and Tribunal Centre offices.

jcareydesigncourts | 49

staff

offices

50 | jcareydesigncourts

SILVER

A combination of oak, white and coloured melamine creates a simple yet functional and bright working environment.

GOLD

White desking and storage solutions are combined with real wood veneers for a crisp modern finish.

jcareydesigncourts | 51

## teapoints

#### Staff welfare facilities can be adjusted in size and style to suit budget constraints.

52 | jcareydesigncourts

Alongside storage cupboards and drawers, teapoints are individually designed to incorporate integrated appliances, sinks and specialist taps as required.

jcareydesigncourts | 53

auxiliary

areas

54 | jcareydesigncourts

Other areas we furnish include advocates rooms, vulnerable witness, VC rooms, jury deliberation areas, dining, breakout, consultation and interview rooms.

demountable

## dais

These demountable platform areas can be installed rapidly to immediately transform the function of a space.

jcareydesigncourts | 55

courthouse overview

fixed courtroom furniture

We provide fully fitted bespoke solutions for anything from wall panelling, Judges benches, barriers and tamper proof/secure wicket gates, to Clerks benches, jury benches, exhibits tables, raised dais, press/probation benches, witness boxes and advocates benches.

We design, manufacture and install a range of furniture styles which can be supplied to a bronze, silver or gold specification to suit the project budget and requirements. Alternatively, fully bespoke solutions can also be manufactured in-keeping with existing rooms, by closely matching the timber species, stain and design style/requirements.

loose courtroom furniture

Generally utilised in more informal or flexible courtroom settings, our typically supplied items include mobile Judges, Clerks and advocates benches, ushers desks, witness boxes, demountable witness screens, demountable raised dais and demountable Judges wall panelling, along with a wide range of seating. Again, these can be supplied to a bronze, silver or gold specification, or fully bespoke options can be selected to blend into existing rooms to suit their environment.

crests

We offer numerous coats of arms or crests for both internal and external walls. Regularly supplied styles include the Royal Coat of Arms of the United Kingdom and the Royal Arms of Scotland in a wide range of sizes and styles. Materials such as stainless steel or cast pewter/resin are used, in either a

monochrome, standard full colour or special full colour palette.

seating (courtroom, retiring rooms, offices, waiting areas)

We supply a wide variety of seating options within the courtroom, surrounding rooms and circulation spaces, including Judges chairs, high back and mid-back fully adjustable seating, DSE compliant chairs, timber framed, cantilevered seats, bariatric chairs, fixed or tip-up beam seating and secure seats which can be floor-fixed or freestanding.

Numerous styles and colours are available in timber, powder coated steel, upholstery or leather.

refurbish and re-use

Dependant on budget and the condition of existing installed items, rather than providing new, we can also

assist with removal and relocation, as well as refinishing and re-upholstery. Our services extend to altering and adding to existing joinery to suit changing requirements.

secure docks and public galleries

JCD are regularly appointed to the design, manufacture, and installation of secure docks to meet HMCTS design guide specification. Detailed site surveys are carried out and careful co-ordination with other trades is essential re: electrics, lighting, affray alarms and sound enhancement. Disruption is minimised by works being carried out across night shifts and weekends, enabling the courtroom to be fully functional at all times.

video conference centre (vcc) link furniture and digital hearing rooms

As courts move more towards the digital era, we provide various solutions for both courts and prisons, to enable VC links and digital hearing rooms to successfully function.

security enhancements

Within existing Judges chambers, unused offices and spare spaces, additional fixed barriers are often installed to provide dual usage capabilities, allowing for more hearings to take place. Courtrooms have been upgraded, involving the modification of non-secure docks to become secure, upgrades to the security of public galleries, and the addition of vulnerable witness screens or secure screens. Additional wicket gates have been added to existing courtrooms, these works are ongoing, to bring all rooms across the estate up to correct security guidelines.

front of house and counters

Matching furniture is also supplied for public areas, usually including security arch search tables, public counters

with pass through facilities, glazed/secure counters and security/CCTV desks, reception desks, coffee tables, soft seating, interview pods or booths, consultation rooms and children’s rooms.

signage

Having worked on all Court of the Future Pilot Schemes,

we worked closely with HMCTS to design a completely new signage solution for entire Court buildings. This wayfinding signage is Court and Tribunal Design Guide (CTDG) compliant, we provide full building solutions as well as installing additional signage to existing buildings. Signage wording is discussed on a site walk round, and detailed artwork for approval is signed off prior to manufacture.

retiring rooms

Alongside our courtrooms, we regularly supply matching furniture elsewhere within the building, this includes a mixture of Judges retiring room desks, lockable pedestals, lockable combination storage units comprising wardrobes, bookshelves and storage cupboards, heavy duty bookshelves, meeting room tables, credenzas and seating. Breakout spaces are also populated with items such as library shelving, dining tables, soft seating and kitchen or tea-point facilities.

offices and auxiliary areas

Staff areas within court, council or other civil service buildings are furnished with our standard or height adjustable desks, power and data units, DSE compliant chairs, staff lockers, filing or pedestal storage, upholstered screens, first aid rooms, breakout furniture and tea-points.

design, service and support

J Carey Design provide a comprehensive service from initial enquiry through to finished installation. We attend site

meetings, take detailed surveys, produce samples and CTDG compliant space planning layouts. Various layout options are provided and discussed, alongside existing and proposed drawings and a detailed specification of our works. We

have a dedicated team of project managers who oversee the in-house manufacture in our purpose-built workshops and co-ordinate all details with other trades. Items are then installed by our DBS checked in-house site team and finally handed back over to the court building with operations and maintenance guidance to suit.

56 | jcareydesigncourts

600+

courtroom fit-outs completed

dedicated project managers guide all works through every stage

court and tribunal design guide compliant solutions to suit any budget

bespoke layouts, visuals and production drawings

projects from

£1,000 to

£1,500,000

125+ years collective court-specific experience

jcareydesigncourts | 57

J Carey Design

No 1- 4 Skelton Business Park, Skelton Road Crosshills, West Yorkshire BD20 7BY

T: 01535 637968 E: info@jcareydesign.com

www.jcareydesign.com

Jonathan Carey Design Ltd, Registered in England, No. 4804059

Approved Supplier*

Introduction

CellSecurity.co.uk

INTRODUCTION

custodial and detention solutions

INTRODUCTION

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

2	Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Index

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### Introduction

CellSecurity.co.uk

custodial and detention solutions

INTRODUCTION

Welcome to the new 8th edition Cell Security Limited product catalogue.

The new 8th edition catalogue is filled with information and over 540 PRODUCTS detailing the latest developments in custodial and detention equipment. During the past 25 years, since the formation of Cell Security in 1988, a portfolio of products have been developed to truly deliver A SINGLE, INTEGRATED SOLUTION for the custodial environment for all our clients both in the UK and internationally. Cell Security Ltd offer high security UK MANUFACTURED PRODUCTS of the highest quality and to the latest standards for new construction, extension and refurbishment, combined with planned preventative maintenance, spares and repairs.

Prisons Police Courts Immigration Customs

Armed Forces

Secure Hospitals

What’s new in the 8th edition …..

European specification cell door New observation cell door Ligature resisting gate

Secure hospital seclusion room door ASTM specification cell door

55dB Acoustic door

Expanded range of detention furniture New prison window

New police cell window

Prison gable end grilles and curtain walling

Cell roof light and sun pipe Expanded range of sanitary ware Washroom accessories

New custodial LED lighting Expanded range of mirrors Security management system New cell call & alarm systems

Expanded range of locks & components New range of door hardware

Expanded range of viewers & hatches

New accessories range

Expanded range of spares

Cell Security Limited has continued to reinforce its position as the UK’s leading manufacturer of custodial & detention solutions by continuing to develop and deliver new innovations to improve the safety, quality and cost effectiveness of its products.

Cell Security Limited is approved in the UK by the HOME OFFICE (responsible for police establishments) and the MINISTRY OF JUSTICE (NOMS) (responsible for prison, court and immigration establishments). This extensive knowledge of the highly regarded UK standards for custodial products and facilities can offer great benefits to our international clients. Cell Security Ltd not only manufactures and installs products, but we assist our international clients with best practice design advice.

Also new for 2013 is our specially developed PRISON WORKS DIVISION BROCHURE. The brochure outlines the range of products and services we can provide to the UK prison services works departments to help maintain, refurbish and replace a wide range of prison door components, furniture, fixtures and fittings.

The new 8th edition catalogue incorporates many new products for our international clients. Cell Security   Limited   is   represented   internationally by a NETWORK OF DISTRIBUTORS. Our products are installed in establishments in over 20 nations worldwide. Please visit our website for our list of distributors.

UAE • Bahrain • France • Cyprus • Gibraltar • Iraq • Ireland • Czech Republic

- Caribbean • Sudan • Afghanistan • Hong Kong • Mauritius • Somalia • Saudi Arabia

INTRODUCTION

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

4	Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Introduction

INTRODUCTION

Product drawings are prepared by Cell Security Limited’s team of draughtsmen for each contract using the latest 3D CAD software. Our team of draughtsmen ensure our products are designed, drawn and presented for approval in accordance with the project programme. Our team of surveyors will carry out the necessary site surveys and pre-installation check prior to site installation. Many products are held in stock for immediate despatch with fabricated products such as doors & windows manufactured to order by our experienced team of production engineers. Bespoke design and manufacture to suit specific requirements is also offered.

Cell Security   Limited’s   technical and manufacturing capabilities are supplemented with dedicated teams of installation engineers offering full UK coverage and internationally. All our engineers are full time employees with the appropriate security clearances for working in secure establishments.

The publication of the new 8th edition catalogue coincides with the 10 year anniversary of Cell Security Ltd joining the BOWMER + KIRKLAND GROUP. With the continued support of the group, Cell Security has continued to grow and invest in product development. Cell Security remains in the privileged position of being the BENCHMARK FOR CUSTODIAL PRODUCTS which is why our products continue to be specified by design professionals who require surety of product performance in demanding environments.

In 2006 our parent company, BOWMER + KIRKLAND, formed a Homeland Security Division, SONCELL INTERNATIONAL, of which CELL SECURITY were the founding member. Since then SONIC COMMUNICATIONS (INT) LTD and PANOPTECH have joined the group and combined we help the police and national agencies meet the threat of, and manage the impact of, international terrorism and crime. 2011 saw the addition of SONCELL MIDDLE EAST to the group, providing a direct sales channel into the region.

Sonic Communications (Int) Ltd has 30+ years’ experience in the security industry and specialises in the design and manufacture of a large number of communications accessories for both overt and covert digital and analogue radios, the supply of body worn, fixed and vehicle covert video surveillance equipment, and carrying out specialist vehicle installations for law enforcement agencies, government departments and military units, worldwide.

Panoptech are security solutions providers offering CCTV system design and installation, high performance wireless transmission links, integrated command and control software, evidential audio/video recording and streaming solutions together with secure networks and hardware.

INTRODUCTION

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	5

#### 1	Cell Doors

SECTION 1

#### CellSecurity.co.uk

custodial and detention solutions

SECTION 1

Cell Security Ltd provide a comprehensive range of cell doors for custody officers to deal with detainees safely. Built to a high specification they comply with the relevant standards for Police Stations, Magistrates Courts, Crown Courts, Prisons, HM Armed Forces, Custom and Excise, or any buildings that require a detention facility. Door units are also available that meet a Worldwide or European Standards.

All doors are of a welded steel construction, hung on a continuous or pivot hinge, and supplied in a substantial steel frame. Door panels are insulated with mineral wool and where applicable fitted with internal reinforcing kick plates on the cell side to resist abuse.

Please note:       Where required we can also provide door panels to be set within existing door frames.

Each door will either be supplied with, or prepared for, a secure approved detention lock.

CSL0101

Cell Door (CEL)

Police Station

CSL0102

Cell Door Electric Locking (CEL)

Police Station

CSL0103

Cell Door

Police Station / Armed Forces

CSL0104

Cell Door

Magistrates Court Existing & Replacement

CSL0105

Cell Door

Magistrates Court & Crown Court

Type A New Build

CSL0106

Cell Door

Custom & Excise

CSL0108

Cell Door

English Prison

CSL0109

Double Cell Door

English Prison

CSL0110

Double Cell Door

English Prison

CELL DOORS

CSL0111

Cell Door

Scottish Prison

CSL0112

Cell Door

Irish Prison

CSL0113

Cell Door

Prison Integrated Drugs

Treatment system

SECTION

6

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Cell Doors	1

SECTION 1

SECTION 1

CSL0114

Cell Door

Prison Hospital

CSL0117

Cell Door

Observation (Home Office Approved)

CSL0115

Cell Door

Prison Dirty Protest

CSL0118

Cell Door

Observation

CSL0116

Observation Door / Gate / Frame

Prison

CSL0119

Holding Room Door (HOL)

Police Station

CSL0120

Holding Room Screen

Police Station

CSL0121

Cubicle Cell

Police Station

CSL0122

International Cell Gate

Police Station / Jail / Detention

CSL0123

Cell Gate Ligature Resisting

Police Station / Jail / Detention

CSL0126

ASTM Sliding Cell Door

Police Station / Jail / Detention

CSL0124

Cell Door European

Prison

CSL0127

ASTM Sliding Cell Gate

Police Station / Jail / Detention

CSL0125

ASTM Hinged Cell Door

Police Station / Jail / Detention

CSL0128

Timber Service Duct Door (DUC)

Police Station / Jail / Detention

CSL0129

Service Duct Door inc Locker

Police Station / Jail / Detention

CSL0130

CELL DOORS

Service Duct Door

Police Station / Jail / Detention

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	7

#### 1	Cell Door

SECTION 1

POLICE STATION (CEL)

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Door panel constructed from 40mm wide rolled hollow steel sections with 3mm thick steel door skins.

- Cell side of door clad with an additional 0.8mm thick bright annealed rigidised stainless steel skin.

- Internally reinforced with 3mm thick plates up to the underside of lock.

- Mineral wool insulation to all internal voids.

- 125mm x 75mm x 8mm rolled steel angle frame.

- 50mm x 25mm stainless steel rebates. Welded and monobolted to door frame.

- Frame fixed into the structural opening with concealed 12mm dia sleeve anchors.

- Door panel mounted within the door frame and hung on a substantial continuous hinge.

- Home Office Type 1 slam action lock with adjustable keep.

- Gasket free vertical louvred vision panel.

- Full View 3 position service hatch.

- Tolerance between door panel and rebate conforms to current Home Office specification not to exceed 2mm.

Finish

  - One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

  - Rigidised stainless steel bright annealed to cell side of door panel. (Pattern ref: 12LG)

CELL DOORS

Options

  - Gasket free circular spy hole with swivel cover.

  - Frame profile and fixings to suit a variety of opening constructions.

  - Rubber door stops

SECTION

8

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Cell Door Electronic Locking	1

SECTION 1

POLICE STATION (CEL)

Specification

- Door panel constructed from 40mm wide rolled hollow steel sections with 3mm thick steel door skins.

- Cell side of door clad with an additional 0.8mm thick bright annealed rigidised stainless steel skin.

- Internally reinforced with 3mm thick plates up to the underside of lock.

- Mineral wool insulation to all internal voids.

- 125mm x 75mm x 8mm rolled steel angle frame.

- 50mm x 25mm stainless steel rebates. Welded and monobolted to door frame.

- Frame fixed into the structural opening with concealed 12mm dia sleeve anchors.

- Door panel mounted within the door frame and hung on a substantial continuous hinge.

- Home Office Type 1 slam action electric lock with adjustable keep.

- Gasket free vertical louvred vision panel.

- Full View 3 position service hatch.

- Tolerance between door panel and rebate conforms to current Home Office specification not to exceed 2mm.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

- Rigidised stainless steel bright annealed to cell side of door panel. (Pattern ref: 12LG)

CELL DOORS

Options

- Gasket free circular spy hole with swivel cover.

- Frame profile and fixings to suit a variety of opening constructions.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	9

#### 1	Cell Door

SECTION 1

POLICE STATION / ARMED FORCES

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Door panel constructed from 40mm wide rolled hollow steel sections with 3mm thick steel door skins.

- Mineral wool insulation to all internal voids.

- 125mm x 75mm x 8mm rolled steel angle frame.

- 50mm x 25mm RHS mild steel rebates welded to door frame.

- Frame fixed into the structural opening with concealed 12mm dia sleeve anchors.

- Door panel mounted within the door frame and hung on a substantial continuous hinge.

- Reinforcing plates to inside of door panel on cell side up to the height of the lock.

- Home Office Type 1 slam action lock with adjustable keep.

- Single hand operation 2 position service hatch.

- Gasket free circular spyhole with swivel cover.

Finish

  - One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

CELL DOORS

0.8mm thick rigidised stainless steel bright annealed skin to cell side of door panel. (Pattern ref:

12LG)

  - Vertical louvred vision panel.

  - Full view 3 position service hatch.

  - Stainless Steel door frame rebates.

SECTION

10

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Cell Door	1

SECTION 1

MAGISTRATES COURT EXISTING & REPLACEMENT

Specification

- Door panel constructed from 40mm wide rolled hollow steel sections with 3mm thick steel door skins.

- Mineral wool insulation to all internal voids.

- 125mm x 75mm x 8mm rolled steel angle frame with 50mm x 25mm steel rebates.

- Frame fixed into the structural opening with concealed 12mm dia sleeve anchors.

- Door panel mounted within door frame and hung on a substantial continuous hinge.

- Reinforcing plates to inside of door panel on cell side up to the height of the lock.

- Home Office Type 1 slam action lock with adjustable keep.

- 150mm x 150mm x 21mm thick physical attack laminated glass vision panel with hinged steel cover.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

- Gasket free circular spy hole with swivel cover.

CELL DOORS

Service hatch.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	11

#### 1	Cell Door

SECTION 1

MAGISTRATES COURT & CROWN COURT TYPE A NEW BUILD

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Door panels constructed from 38mm wide rolled hollow steel sections with 2mm thick steel door skins seam welded all round and manufactured in accordance with all current MoJ standards.

- Rolled / Pressed steel hollow section door frame with steel rebates.

- Mineral wool insulation to all internal voids.

- Frame fixed by chemical resin anchors.

- Door panel mounted within door frame and hung on a full height tubular steel pivot hinge.

- Reinforcing plates to inside of door panel on cell side up to the height of the lock.

- Vertical laminated vision panel with hinged steel cover.

- Preparation Only for Home Office Type 3 mortice locking latch.

Finish

  - One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

  - Door panel to open out into the corridor.

CELL DOORS

Alternative fixing methods to suit site conditions for retro fit scenarios.

SECTION

12

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Cell Door	1

SECTION 1

CUSTOM & EXCISE

Specification

- Door panel constructed from 40mm wide rolled hollow steel sections with 3mm thick steel door skins.

- Mineral wool insulation to all internal voids.

- 125mm x 75mm x 8mm rolled steel angle frame.

- 50mm x 25mm RHS mild steel rebates welded to door frame.

- Frame fixed into the structural opening with concealed 12mm dia sleeve anchors.

- Door panel mounted within the door frame and hung on a substantial continuous hinge.

- Reinforcing plates to inside of door panel on cell side up to the height of the lock.

- Home Office Type 1 slam action lock with adjustable keep.

- Single hand operation 2 position service hatch.

- 500mm x 500mm x 22.3mm thick physical attack laminated/toughened glass vision panel with hinged steel cover.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

CELL DOORS

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	13

#### 1	Cell Door

SECTION 1

ENGLISH PRISON

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Door panel constructed from 38mm wide rolled hollow steel sections with 2mm thick steel door skins seam welded along all edges and manufactured in accordance with all current MoJ standards.

- Mineral wool insulation to all internal voids.

- Steel hollow section door frame with anti-barricade device.

- Frame fixed by chemical resin anchors.

- Door panel mounted within door frame and hung on a full height tubular steel pivot hinge.

- Inundation unit.

- Vertical laminated glass vision panel with hinged steel cover.

- Preparation for MoJ Class 1 lock supplied and fitted by HMPS or Private Prison Operator.

- Anti-ligature strips to front face of door, along top and down leading edge terminating at lock.

Finish

  - One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

CELL DOORS

IDTS hatch.

  - Cast-in type frame.

  - Heavy duty barrel bolt.

SECTION

14

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Double Cell Door	1

SECTION 1

ENGLISH PRISON

Specification

- Door panels constructed from 38mm wide rolled hollow steel sections with 2mm thick steel door skins seam welded along all edges and manufactured in accordance with all current MoJ standards.

- Mineral wool insulation to all internal voids.

- Cast-in double frame type.

- Steel hollow section door frame with anti-barricade device and central pressed steel closure panel.

- Door panels mounted within door frame and hung on a full height tubular steel pivot hinge.

- Inundation unit.

- Vertical laminated glass vision panel with hinged steel cover.

- Preparation for MoJ Class 1 lock supplied and fitted by HMPS or Private Prison Operator.

- Anti-ligature strips to front face of door, along top and down leading edge terminating at lock.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

CELL DOORS

IDTS hatch.

- Heavy duty barrel bolt.

- Re-bar casting hoops in lieu of rods.

- Standard frame type.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	15

#### 1	Double Cell Door

SECTION 1

ENGLISH PRISON

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Door panels constructed from 38mm wide rolled hollow steel sections with 2mm thick steel door skins seam welded along all edges and manufactured in accordance with all current MoJ standards.

- Mineral wool insulation to all internal voids.

- Cast-in double frame type.

- Steel hollow section door frame with anti-barricade device and central pressed steel closure panel.

- Door panels mounted within door frame and hung on a full height tubular steel pivot hinge.

- Inundation unit.

- Vertical laminated glass vision panel with hinged steel cover.

- Preparation for MoJ Class 1 lock supplied and fitted by HMPS or Private Prison Operator.

- Anti-ligature strips to front face of door, along top and down leading edge terminating at lock.

Finish

  - One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

CELL DOORS

IDTS hatch.

  - Heavy duty barrel bolt.

  - Re-bar casting hoops in lieu of rods.

  - Standard frame type.

SECTION

16

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Cell Door	1

SECTION 1

SCOTTISH PRISON

Specification

- Door panel constructed from 38mm wide rolled hollow steel sections with 2mm thick steel door skins seam welded along all edges and manufactured in accordance with all current SPS standards.

- Mineral wool insulation to all internal voids.

- Steel hollow section door frame with anti-barricade device.

- Frame fixed by chemical resin anchors.

- Door panel mounted within door frame and hung on a full height tubular steel pivot hinge.

- Inundation unit.

- Vertical laminated glass vision panel with sliding stainless steel cover.

- Preparation for SPS Class 1 lock supplied and fitted by SPS or Private Prison Operator.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

CELL DOORS

Cast-in type frame.

- Heavy duty barrel bolt.

- Double arrangement.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	17

#### 1	Cell Door

SECTION 1

IRISH PRISON

Specification

#### CellSecurity.co.uk

custodial and detention solutions

- Door panel constructed from 50mm wide rolled hollow steel sections with 3mm thick steel door skins seam welded and manufactured in accordance with all current IPS standards.

- Mineral wool insulation to all internal voids.

- Steel hollow section door frame.

- Frame fixings to suit a variety of opening construction.

- Door panel mounted within door frame hung on a full height tubular steel pivot hinge.

- The frame header is provided with removable panel to allow access to electrical wiring. (All removable panels are secured with security screws, tamper proof switches can be supplied and fitted and wired by others to monitor plate removal).

- Stainless steel card holder.

- Inundation unit.

- Horizontal laminated glass vision panel with stainless steel hinged privacy cover.

- The door panel is prepared for a Manual / Electric IPS Class 1 lock which is mounted on a 10mm thick plate, lock supplied and fitted by IPS.

Finish

CELL DOORS

One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

  - Cast-in type frame.

  - Double arrangement.

SECTION

18

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Cell Door	1

SECTION 1

ENGLISH PRISON INTEGRATED DRUGS TREATMENT SYSTEM

Specification

- Door panel constructed from 38mm wide rolled hollow steel sections with 2mm thick steel door skins seam welded along all edges and manufactured in accordance with all current MoJ standards.

- Mineral wool insulation to all internal voids.

- Steel hollow section door frame with anti-barricade device.

- Frame fixed by chemical resin anchors.

- Door panel mounted within door frame and hung on a full height tubular steel pivot hinge.

- Inundation unit.

- IDTS Hatch with sliding cover and glazing c/w locking facility.

- Preparation for MoJ Class 1 lock supplied and fitted by HMPS or Private Operator.

- Anti-ligature strip to front face of door, along top and down leading edge terminating at lock.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

CELL DOORS

Cast in type frame.

- Heavy duty barrel bolt.

- Double arrangement

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	19

#### 1	Cell Door

SECTION 1

ENGLISH PRISON HOSPITAL

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Door panel constructed from 38mm wide rolled hollow steel sections with 2mm thick steel door skins seam welded along all edges and manufactured in accordance with all current MoJ standards.

- Mineral wool insulation to all internal voids.

- Steel hollow section door frame with anti-barricade device.

- Frame fixed by chemical resin anchors.

- Door panel mounted within door frame and hung on a full height tubular steel pivot hinge.

- Inundation unit.

- IDTS Hatch with sliding cover and glazing c/w locking facility.

- Preparation for MoJ Class 1 lock supplied and fitted by HMPS or Private Prison Operator.

- Anti-ligature strips to front face of door, along top and down leading edge terminating at lock.

Finish

  - One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

CELL DOORS

Cast-in type frame.

  - Heavy duty barrel bolt.

  - Double arrangement.

SECTION

20

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Cell Door	1

SECTION 1

PRISON DIRTY PROTEST

Specification

- Door panel constructed from 38mm wide rolled hollow steel section with 2mm thick steel door skins seam welded and manufactured in accordance with all current MoJ standards..

- Mineral wool insulation to all internal voids.

- Steel hollow section door frame with anti-barricade device and preparation for illuminated header.

- Frame fixed by chemical resin anchors.

- Door panel mounted within door frame and hung on a full height tubular steel pivot hinge.

- Inundation unit fitted in door panel.

- Preparation for a slam action MoJ Class 1 cell lock supplied and fitted by HMPS or Private Prison Operator.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Special Features

CELL DOORS

Special service hatch with ratchet mechanism which allows hatch to be raised only when food box is attached to the door.

- Removable food box.

- Double sliding observation window with cover.

- Stainless Steel canister to collect soiled vision panel lenses.

- Double arrangement.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	21

#### 1	Observation Door / Gate / Frame Arrangement

SECTION 1

PRISON

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Door panel constructed as CSL0108 and gate constructed as CSL0406.

- Mineral wool insulation to all internal voids to door panel

- Steel hollow section door frame with anti-barricade device.

- Frame fixed by chemical resin anchors with welded heads.

- Door panel mounted within door frame and hung on a full height tubular steel pivot hinge.

- Inundation unit.

- Vertical laminated glass vision panel with hinged steel cover.

- Preparation for MoJ Class 1 lock supplied and fitted by HMPS or Private Prison Operator.

- Anti-ligature strips to front face of door, along top and down leading edge terminating at lock.

Finish

  - One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

  - IDTS hatch.

CELL DOORS

Cast-in type frame.

  - Heavy duty barrel bolt.

SECTION

22

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Cell Door	1

SECTION 1

OBSERVATION

Specifications

- Door panel constructed from 40mm wide rolled hollow steel sections with 3mm thick door skins, grade 304 stainless

steel to cell side.

- 125mm x 75mm x 8mm rolled steel angle frame.

- 50mm x 25mm stainless steel rebates welded an monobolted to door frame.

- Frame fixed with heavy duty expansion bolts.

- Home Office type 1 slam action lock with adjustable keep.

- Continuous hinge.

- Home Office approved two position hatch.

- Laminated toughened glass / polycarbonate vision panel with sliding privacy cover incorporating a louvred vertical viewer.

- Rubber door stops.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

CELL DOORS

Cell side door skin and rebates, satin polished stainless steel.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	23

#### 1	Cell Door

SECTION 1

OBSERVATION

#### CellSecurity.co.uk

custodial and detention solutions

Intelli-glass privacy glazing

Specification

- Door panel constructed from 40mm wide rolled hollow steel section with 3mm thick steel door skins.

- 125mm x 75mm x 8mm rolled steel angle frame.

- 50mm x 25mm steel rebates welded to door frame.

- Frame profiles and fixings to suit a variety of opening constructions.

- Door panel mounted within door frame and hung on a substantial continuous hinge.

- Glass retaining frames are fitted on the corridor side of the door only.

- Standard 22.3mm thick toughened laminated glass.

- Home Office Type 1 slam action lock with adjustable keep.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

  - 0.8mm thick rigidised stainless steel bright annealed skin with glass retaining frame to cell side of door panel (Pattern ref: 12LG).

CELL DOORS

60 minute Fire Resisting.

  - Recessed Handles when used in conjunction with a type 2 heavy duty mortice deadlock.

  - Intelli-glass privacy glazing.

  - Home Office Type 2 heavy duty deadlock.

  - Bullet resisting glass.

  - Electric Strike.

  - 2 + 1 Position hatch.

SECTION

24

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Holding Room Door	1

SECTION 1

POLICE STATION (HOL)

Specification

- Door is constructed from a solid timber core sheeted both sides and all edges with 1.6mm thick zintec steel skins pressure bonded to core, interlocked and riveted to give a finished door thickness of 50mm.

- Door is mounted within a 125mm x 75mm x 8mm rolled steel and frame and hung from a substantial continuous hinge.

- Frame fixed into the structural opening with concealed 12mm dia sleeve anchors.

- 150mm square x 21mm thick laminated glass vision panel with hinged steel cover.

- Home Office Type 2 heavy duty mortice deadlock with adjustable keep.

- Recessed handle corridor side only.

- Tolerance between door panel and rebate conforms to current Home Office specification not to exceed 2mm.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

- Vertical louvered viewer.

- Circular gasket free viewer.

CELL DOORS

Concealed door closer.

- Frame profiles and fixings to suit a variety of opening constructions.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	25

#### 1	Holding Room Screen

SECTION 1

POLICE STATION

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Door and fixed panels are constructed from 40mm wide steel hollow section framework sheeted both sides with 3mm thick steel skins, mounted within a rolled hollow section frame.

- All glazing is 22.3mm thick toughened laminated glass, the glass retaining frames are only fitted to the corridor side.

- The door is hung from a substantial continuous hinge.

- Fixings to suit a variety of opening constructions.

- The door is supplied as standard with one recessed pull handle on the outside and a Home Office Type 2 heavy duty mortice deadlock.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

  - Home Office Type 1 slam action cell lock.

  - Bullet resisting glass.

  - Intelli-glass privacy glazing.

CELL DOORS

Any size / combination of panels can be provided.

  - 0.8mm thk rigidised stainless steel bright annealed skin to cell side of door panel. (Pattern ref: 12LG)

  - 60 minute Fire Resisting.

SECTION

26

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Cubicle Cell	1

SECTION 1

POLICE STATION

Specification

- Door panel and fixed panels are constructed from 40mm wide steel hollow section framework sheeted both sides with 3mm thick steel skins.

- All glazing is 22.3mm thick toughened laminated glass, the glass retaining frames are secured on the external side of the

unit.

- The door panel is hung from a substantial continuous hinge.

- Fixings to suit a variety of opening constructions.

- The door is supplied as standard with a Home Office Type 1 slam action cell lock.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

- Multiple (side by side) units.

- Privacy glazing.

- None glazed side panels.

CELL DOORS

High level Ventilation Grille.

- Ventilation grille within door panel.

- Cubicle Light.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	27

#### 1	International Cell Gate

SECTION 1

POLICE STATION / JAIL / DETENTION

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Gate is constructed from 50mm x 12mm flat steel framework with 25mm diameter solid vertical bars.

- Mounted on a 70mm x 70mm x 8 rolled steel angle frame with a full height stainless steel hinge pin.

- Top and bottom pivot hinges bushed, base hinge incorporates a ball bearing to reduce hinge friction and giving ease of movement.

- Frame fixings to suit a variety of opening construction.

- Class 1 heavy duty slam action lock.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

  - Galvanized.

  - Polyester Powder Coated.

CELL DOORS

Any arrangement of side or header panels can be provided.

SECTION

28

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### ASTM - Sliding Cell Gate	1

SECTION 1

POLICE STATION / JAIL / DETENTION

Specifications

- Gate panel conforms to ASTM A627.

- Constructed from 2 1/4” x 3/8” flat steel bar framing and horizontals with 7/8” diameter double ribbed interlocking vertical bar.

- Steel subframe fixed to door opening incorporating a full height slam channel and bottom guide rail.

- Manual or electronically driven top track.

- Mechanical frame mounted lock for manual sliding gates or electronic locking for electronically driven gates to ASTM F1643-05.

- Lockable pressed steel canopy incorporating track gear.

- Canopy incorporates a mechanical unlock / override facility for electronically driven gates.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

- Available right or left hand.

CELL DOORS

Tool resistant anti-cut double ribbed round and flat bars to ASTM A627-03.

- Food pass aperture.

- Canopy can incorporate status indicator lights to show open, locked and in travelling positions for electronically driven gates.

- Electronic controls to control multiple gates.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	29

#### 1	Timber Service Duct Door

SECTION 1

POLICE STATION (DUC) / JAIL / DETENTION

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Door is constructed from a 54mm thick solid timber core blank with hardwood lipping to all edges.

- Door is mounted within a hardwood profile frame and hung on 3No. x 100mm butt hinges.

- Frame fixings to suit a variety of opening construction.

- Recessed handle outside.

- Two allen key bolt locks.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

  - Veneer finish.

  - Double door arrangement.

  - Smoke seals.

CELL DOORS

Continuous hinge in lieu of butt hinges.

SECTION

30

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Service Duct Door inc Locker	1

SECTION 1

POLICE STATION / JAIL / DETENTION

Specification

- Door leaf is constructed from a solid timber core sheeted on front face and all edges with 1.6mm thick zintec steel skin giving a finished door thickness of 40mm.

- Door leaf is mounted within a 50mm x 50mm x 6mm rolled steel angle frame and is hung from a substantial continuous hinge.

- Frame fixings to suit a variety of opening construction.

- Supplied as standard with three allen key socket head bolts for secure fastening.

- Integral shoe locker constructed from 2mm pressed mild steel box and frame c/w 1.6mm pressed door cover secured

by a cam lock.

Finish

- One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

- Frame can be provided without threshold for “full height” doors.

- Double leaf doors.

CELL DOORS

Door leaf secured by deadlocks or cam locks in lieu of standard secure fasteners.

- Can be supplied without shoe lockers.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	31

#### Service Duct Door

SECTION 1

POLICE STATION / JAIL / DETENTION

#### CellSecurity.co.uk

custodial and detention solutions

Specification

  - Door panel constructed from 38mm wide rolled hollow steel sections with 2mm thick steel door skins seam welded

along all edges.

  - 60 minutes fire rated from within the service duct.

  - Mineral wool insulation to all internal voids.

  - Full height continuous hinge.

  - Four sided rolled steel angle frame fixed with heavy duty expansion bolts.

  - Supplied with or prepared for Class 3 lock.

Finish

    - One coat factory applied high build zinc phosphate primer to all mild steel surfaces.

Options

    - Double leaf doors with central mullion.

CELL DOORS

Smoke seals.

SECTION

32

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Cell Door	1

SECTION 1

POLICE STATION

CELL DOORS

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	33

#### Custody Perimeter / Circulation Doors

SECTION 2

#### CellSecurity.co.uk

custodial and detention solutions

SECTION 2

Cell Security Limited provide a comprehensive range of Custody Perimeter and Circulation Doors for areas where secure access and control are essential.

All doors when closed secure the area and allow custody officers to transfer detainees throughout custody. Built to a high specification, comply with the Home Office Standards for Police Stations, Magistrates Courts, Crown Courts, Prisons, HM Armed Forces, Custom and Excise, and any buildings that require a detention facility.

Doors are also available that meet Worldwide and European Standards.

Doors are constructed to suit their application, and can either be a solid timber core with door skins bonded and riveted or an all welded steel construction, all are hung on a continuous or pivot hinge and supplied in a substantial steel frame. All welded steel doors are internally insulated with mineral wool.

Each door will either be supplied with, or prepared for, a secure Home Office approved custody lock. Where applicable, we can supply electronic lock releases remotely activated from the custody desk or control room to entry doors, secure lobby or corridor doors.

CSL0201

Entrance Door (PER)

Police Station

CSL0202

Sliding Door

Police Station / Jail

CSL0203

Pass / Exercise Yard Door

Police Station / Magistrates Court Crown Court (Type Ba & Bb)

CSL0204

Perimeter Door Magistrates & Crown Court (Type Bc)

CSL0205

Dock Entry Door

Crown Court (Type G)

CSL0206

60 Minute Fire Resisting Door Police Station / Magistrates & Crown Court / Armed Force / Custom & Excise

CIRCULATION DOORS

CSL0207

Glazed 60 Minute Fire Resisting Door Custody Screen

Police Station

CSL0208

Custody Fire Escape Door

Police Station / Magistrates Court

CSL0209

Single Action Corridor Door

London Police Station (SD26)

SECTION

34

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Custody Perimeter / Circulation Door	2

SECTION 2

SECTION 2

CSL0210

Double Action Corridor Door

London Police Station (SD9)

CSL0212

English Circulation Door

Prison

CSL0213

Circulation Door

Preparation for palm reader & audio visual unit

Irish Prison

CSL0214

C & R Door

Prison

CIRCULATION DOORS

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	35

#### 2	Entrance Door

SECTION 2

POLICE STATION (PER)

Specification

#### CellSecurity.co.uk

custodial and detention solutions

- Door is constructed from a solid timber core sheeted both sides with 1.6mm thick zintec steel skins pressure bonded to core, interlocked and riveted to give a finished door thickness of 50mm.

- Door is mounted within a 100mm x 50mm rolled steel channel frame and hung from a substantial continuous hinge.

- Frame fixings to suit a variety of opening construction.

- Supplied as standard with 150mm square x 21mm thick laminated glass vision panel with hinged steel privacy cover.

- Recessed pull handles on both sides of door.

- Home Office Type 2 heavy duty mortice deadlock working in conjunction with a 12V DC electric strike fitted in the frame with optional closer.

Finish

  - One coat factory applied high build zinc phosphate primer.

CIRCULATION DOORS

Options

  - Rolled steel angle frame or profile frame to suit site conditions.

  - Drip strip / water bar on external doors.

  - Weather seals.

  - Vertical louvred viewer.

  - Circular gasket free viewer.

SECTION

36

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Sliding Door	2

SECTION 2

POLICE STATION / JAIL

Specification

- Door constructed from a 40mm wide steel hollow section framework sheeted both sides with 3mm thick steel skins

seam welded.

- Mineral wool insulation to all internal voids.

- Door panel hung on pulley wheels with roller bearings and mounted within a horizontal steel top track.

- Two recessed pull handles.

- Bottom guide rail.

- Supplied as standard with a 150mm square x 9.5mm thick laminated glass vision panel.

- Supplied with a full height slam channel and interlocking angles, securing door when in closed position.

- Remotely operated electric motor with rack and pinion gear.

- Supplied as standard with motor / track canopy.

Finish

CIRCULATION DOORS

One coat factory applied high build zinc phosphate primer.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	37

#### 2	Pass / Exercise Yard Door

SECTION 2

POLICE STATION / MAGISTRATES COURT / CROWN COURT (TYPE Ba & Bb)

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Door is constructed from a solid timber core sheeted both sides with 1.6mm thick zintec steel skins pressure bonded to core, interlocked and riveted to give a finished door thickness of 50mm.

- Door is mounted within a 100mm x 50mm rolled steel channel frame and hung from a substantial continuous hinge.

- Frame fixings to suit a variety of opening construction.

- Supplied as standard with 150mm square, 9.5mm thick laminated glass vision panel (no privacy cover).

- Recessed pull handles on both sides of door.

- HMCS/Home Office Type 2 heavy duty mortice deadlock.

- PSD / Ba - Pass or circulation door.

- PSE / Bb - Pass or circulation door with hinged cover on vision panel.

Note - Doors for Crown Courts, the lock is supplied and fitted by HMCS.

CIRCULATION DOORS

Finish

  - One coat factory applied high build zinc phosphate primer.

Options

  - Rolled steel angle or pressed profile frame to suit site conditions.

  - Hydraulic door closer.

  - Drip strip / water bar on external doors.

  - Electric strike in frame.

  - Weather seals.

  - Hinged privacy cover on vision panel.

SECTION

38

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Perimeter Door	2

SECTION 2

MAGISTRATES / CROWN COURT (TYPE Bc)

Specification

- Door is constructed from a solid timber core sheeted both sides with 1.6mm thick zintec steel skins pressure bonded to core, interlocked and riveted to give a finished door thickness of 50mm.

- Door is mounted within a 100mm x 50mm rolled steel channel frame and hung from a substantial continuous hinge.

- Frame fixings to suit a variety of opening construction.

- Supplied as standard with 150mm square x 9.5mm thick laminated glass vision panel with hinged steel privacy cover.

- Five lever mortice deadlock, one fixed knob handle to secure side only.

- Preparation for a HMCS Class 2 heavy duty mortice deadlock supplied and fitted by HMCS on site.

Finish

- One coat factory applied high build zinc phosphate primer.

CIRCULATION DOORS

Options

- Rolled steel angle or pressed profile frame to suit site conditions.

- Hydraulic door closer.

- Drip strip / water bar on external doors.

- Electric strike in frame.

- Weather seals.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	39

#### 2	Dock Entry Door

SECTION 2

CROWN COURT (TYPE G)

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Door leaf constructed from a solid timber core sheeted both sides with 1.6mm thick zintec steel skins pressure bonded to core, interlocked and riveted to give a finished door thickness of 50mm.

- Door leaf mounted within a 100mm x 50mm rolled steel channel frame and hung from a substantial continuous hinge.

- Frame fixings to suit a variety of opening constructions.

- Supplied as standard with two magnification viewers to provide viewing from either side.

- Five lever mortice deadlock, one fixed knob handle to secure side only.

- Preparation for a HMCS Class 2 heavy duty mortice deadlock supplied and fitted by HMCS on site.

Finish

  - One coat factory applied high build zinc phosphate primer.

CIRCULATION DOORS

Options

  - Door leaf can be veneered both sides to match courtroom panelling.

  - Rolled steel angle or pressed profile frame to suit site conditions.

  - Fire rated version available.

SECTION

40

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### 60 Minute Fire Resisting Door	2

SECTION 2

POLICE STATION / MAGISTRATES / CROWN COURT / ARMED FORCES / CUSTOM & EXCISE

BS EN1364-1 Testing

Specification

- Tested in accordance with BS EN 1364-1 to give up to 60 minute fire resistance, the door is constructed from 40mm wide rolled hollow steel section with 3mm thick steel door skins.

- Mineral wool insulation to all internal voids.

- 125mm x 75mm x 8mm rolled steel angle frame.

- Frame fixings to suit a variety of opening constructions.

- Door panel mounted within door frame hung on a substantial continuous hinge.

- Recessed handles on both sides of door.

- Home Office Type 2 heavy duty mortice deadlock.

- 150mm square vision panel with fire resisting laminated glass.

Finish

CIRCULATION DOORS

One coat factory applied high build zinc phosphate primer.

Options

- Hinged privacy cover to vision panel.

- Hydraulic door closer.

- Type 1 Cell Lock.

- 12V DC electric strike.

- Smoke seals.

- Veneer finish

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	41

#### 2	Glazed 60 Minute Fire Resisting Door Custody Screen	CellSecurity.co.uk

SECTION 2

POLICE STATION	custodial and detention solutions

Specification

- Tested in accordance with BS EN 1364-1 to give up to 60 minute fire resistance.

- Door and fixed panels are constructed from 40mm wide steel hollow section framework sheeted both sides with 3mm thick steel skins, mounted within a 100mm x 60mm rolled hollow section frame.

- All glazing is laminated, fire resisting and complies with BS 5544:1978 specification for anti-bandit glazing.

- The door is hung from a substantial continuous hinge.

- Fixings to suit a variety of opening constructions.

- The door is supplied as standard with a recessed pull handles on both sides of the door and fitted with a Home Office Type 2 heavy duty mortice deadlock.

Finish

  - One coat factory applied high build zinc phosphate primer.

CIRCULATION DOORS

Options

  - Home Office Type 1 slam action cell lock.

  - Smoke seals.

  - Hydraulic door closer.

  - 12V DC electric strike.

SECTION

42

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Custody Fire Escape Door	2

SECTION 2

POLICE STATION / MAGISTRATES COURT

Specification

- Door is constructed from a solid timber core sheeted both sides and all edges with 1.6mm thick zintec steel skins pressure bonded to core, interlocked and riveted to give a finished door thickness of 50mm.

- Door is mounted within a 100mm x 50mm rolled steel channel frame and hung from a substantial continuous hinge.

- Frame fixings to suit a variety of opening construction.

- Recessed handles on both sides of door.

- Drip strip on outside face of door, weather bar threshold.

- Home Office Type 2 heavy duty mortice deadlock.

Finish

- One coat factory applied high build zinc phosphate primer.

CIRCULATION DOORS

Options

- Rolled steel angle frame or pressed profile frame to suit site conditions.

- Hydraulic door closer.

- 12V DC electric strike.

- Weather seals.

- Vision panel.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	43

#### 2	Single Action Corridor Door

SECTION 2

LONDON POLICE STATION (SD26)

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Door is constructed from a solid timber core sheeted both sides with Armaglaze panels with Ash Lipping to all four edges, giving a finished door thickness of 60mm.

- Door is mounted within an Ash frame and pivots on a single action floor spring set into the floor.

- Frame fixings to suit a variety of opening construction.

- Supplied as standard with a 215mm x 250mm x 6mm thick polycarbonate vision panel.

- Heavy duty adjustable roller catch, one ‘D’ handle to pull side, push plate to push side.

- Home Office Type 2 heavy duty mortice deadlock.

Finish

  - Armaglaze to any colour within the BS 4800 colour range.

  - All timber to have two coats of water-borne varnish.

CIRCULATION DOORS

Options

  - Smoke and / or intumescent seals.

  - 24.3mm Pyrobel laminated glass (non-certified 60 minute fire rated).

SECTION

44

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Double Action Corridor Door	2

SECTION 2

LONDON POLICE STATION (SD9)

Specification

- Door is constructed from a solid timber core sheeted both sides with Armaglaze panels with Ash Lipping to all four edges, giving a finished door thickness of 60mm.

- Door is mounted within an Ash frame and pivots on a double action floor spring set into the floor.

- Frame fixings to suit a variety of opening construction.

- Supplied as standard with a 215mm x 250mm x 6mm thick polycarbonate vision panel.

- Heavy duty adjustable roller catch.

- Push plates both sides.

- Home Office Type 2 heavy duty mortice deadlock.

Finish

- Armaglaze to any colour within the BS 4800 colour range.

CIRCULATION DOORS

All timber to have two coats of water-borne varnish.

Options

- Smoke and / or intumescent seals.

- 24.3mm Pyrobel laminated glass (non-certified 60 minute fire rated).

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	45

#### Irish Circulation Door

SECTION 2

PRISON

#### CellSecurity.co.uk

custodial and detention solutions

Specification

  - Door constructed from a 40mm wide steel hollow section framework sheeted both sides with 2mm thick steel skins seam

welded.

  - Mineral wool insulation to all internal voids.

  - Door panel mounted within a steel hollow section frame and hung from a full height tubular steel pivot hinge, and is fixed

with chemical resin anchors.

  - Preparation for a MoJ Class 1 electric lock or MoJ Class 2 heavy duty mortice deadlock supplied and fitted by HMPS on

site.

Finish

  - One coat factory applied high build zinc phosphate primer.

CIRCULATION DOORS

Options

    - If required the header is provided with a removable panel to allow access to electrical wiring. (All removable panels are secured with tamper resisting screws, tamper proof switches can be supplied fitted and wired by others to monitor plate removal).

    - Vision panel.

SECTION

46

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### C & R Door	2

SECTION 2

PRISON

Specification

- Door leaf constructed from 38mm wide rolled hollow steel section with 2mm thick steel door skins.

- Mineral wool insulation to all internal voids.

- Steel hollow section door frame.

- Frame fixed by chemical resin anchors.

- Door leaf mounted within door frame and hung on a full height tubular steel pivot hinge.

- One welded D handle, two heavy duty barrel bolts.

- Rectangular laminated glass vision panel with hinged steel cover.

- Preparation for a MoJ Class 2 heavy duty prison mortice deadlock supplied and fitted by HMPS on site.

Finish

- One coat factory applied high build zinc phosphate primer.

CIRCULATION DOORS

Options

- Cast in type frame.

- Smoke seals.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	47

#### Personnel Doors

SECTION 3

SECTION 3

#### CellSecurity.co.uk

custodial and detention solutions

PERSONNEL DOORS

SECTION

48

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Personnel Doors	3

SECTION 3

SECTION 3

Cell Security Limited provide a comprehensive range of personnel doors for specific areas where the highest security and resistance to abuse is essential.

CSL0301

Surgeon / Medical Door

Police Station

CSL0302

Search Room Door (SRCH)

Police Station

CSL0303

Solid Timber Door Police Station (PLC) Crown Court (Type C)

CSL0304

Half Glazed Timber Door Police Station (PLB) Crown Court (Type D)

CSL0305

Acoustic Door (39 or 44 db)

Police Station

CSL0306

Acoustic Door (55db)

Police Station

CSL0307

Steel Personnel Door

Police Station

CSL0308

Decency Door Police Station (PMA) Crown Court (Type E)

CSL0309

Armoury Door

Police Station / Armed Forces

PERSONNEL DOORS

CSL0310

Strongroom Door

Police Station / Armed Forces

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	49

#### 3	Surgeon / Medical Door	CellSecurity.co.uk

SECTION 3

POLICE STATION	custodial and detention solutions

Specification

- Door is constructed from a solid timber core sheeted both sides with 1.6mm thick zintec steel skins pressure bonded to core, interlocked and riveted to give a finished door thickness of 50mm.

- Door is mounted within a 3mm thick steel profile frame and hung from a substantial continuous hinge.

- Frame fixings to suit a variety of opening construction.

- Supplied as standard with 150mm square x 9.5mm thick laminated glass window with hinged steel privacy cover.

- Recessed pull handle on each side of the door.

- Home Office Type 2 heavy duty mortice deadlock.

Finish

  - One coat factory applied high build zinc phosphate primer.

Options

PERSONNEL DOORS

Hydraulic door closer.

SECTION

50

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Search Room Door	3

SECTION 3

POLICE STATION (SRCH)

Specification

- Door is constructed from a 54mm thick solid timber core blank with hardwood lipping to all edges.

- Door is mounted within a hardwood profile frame and hung on a substantial continuous hinge.

- Frame fixings to suit a variety of opening construction.

- Magnification viewer.

- Home Office Type 2 heavy duty mortice deadlock.

Finish

- One coat factory applied primer.

Options

- Veneer finish.

PERSONNEL DOORS

Coloured impact resistant smooth PVC cladding.

- Recessed handle.

- Concealed door closer.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	51

#### 3	Solid Timber Door

SECTION 3

POLICE STATION (SOL) / CROWN COURT (TYPE C)

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Door is constructed from 54mm thick solid timber core faced both sides with plywood veneer and 9mm lipping to vertical and horizontal edges.

- Door is mounted within a hardwood profile frame and is hung from 4 No x 100mm triple knuckle butt hinges.

- Frame fixings to suit a variety of opening construction.

- 5 lever mortice locking latch operable from both sides.

Finish

  - One coat factory applied primer.

Options

  - Steel profile frame.

PERSONNEL DOORS

One coat varnish finish instead of primer.

  - Veneer finish.

  - Hydraulic door closer.

  - Anti-ligature hardware.

  - Vision panel.

  - Flush ring handle(s) instead of knob furniture.

  - 60 minute Fire Rating.

  - Double door arrangement.

  - Electro-magnetic hold open closer.

  - Vertical louvred viewer.

SECTION

52

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Half Glazed Timber Door	3

SECTION 3

POLICE STATION (GLZ) / CROWN COURT (TYPE D)

Specification

- Door is constructed from 54mm thick solid timber core faced both sides with plywood veneer and 9mm lipping to vertical and horizontal edges.

- Door is mounted within a hardwood profile frame and is hung from 4 No x 100mm triple knuckle butt hinges.

- Frame fixings to suit a variety of opening construction.

- Supplied as standard with 11.5mm thick laminated glass vision panel (maximum size 0.45 M.)

- 5 lever mortice locking latch operable from both sides.

Finish

- One coat factory applied primer.

Options

PERSONNEL DOORS

Steel profile frame.

- One coat varnish finish instead of primer.

- Veneer finish.

- Anti-ligature hardware.

- Hydraulic door closer.

- Vision panel.

- Flush ring handle(s) instead of knob furniture.

- 60 minute Fire Rating with door closer, fire rated glass, intumescent seals and smoke seals.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	53

#### 3	Acoustic Door (39 or 44 db)	CellSecurity.co.uk

SECTION 3

POLICE STATION	custodial and detention solutions

Specification

- Door is constructed from a 54mm solid timber core faced both sides with plywood veneer and 9mm lipping to vertical and horizontal edges.

- Door is mounted within a hardwood profile frame and is hung from 4 No x 100mm triple knuckle butt hinges.

- Frame fixings to suit a variety of opening construction.

- Acoustic and intumescent seals with rubber fins to all rebates.

- Drop seal to bottom edge of door.

- Magnification Viewer.

- 5 lever mortice locking latch operable from both sides.

Finish

  - One coat factory applied primer.

PERSONNEL DOORS

Options

  - Available in 39db or 44db rating.

  - One coat varnish finish instead of primer.

  - Veneer finish.

  - Anti-ligature hardware.

  - Flush ring handles instead of knob furniture.

  - 60 minute Fire Rating

  - Intumescent seals and/or smoke seals.

  - Hydraulic door closer.

SECTION

54

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Acoustic Door (55 db)	3

SECTION 3

custodial and detention solutions	POLICE STATION

Specification

- Door is constructed from a 67mm leaf with a range of corner and single rebate frame profiles.

- 150mm x 20mm diameter heavy duty hinges (2 per leaf).

- Frame fixings to suit a variety of opening construction.

- Acoustic and intumescent seals with rubber fins to all rebates.

- Drop seal to bottom edge of door.

- 5 lever mortice locking latch operable from both sides.

Finish

- One coat factory applied primer.

Options

PERSONNEL DOORS

One coat varnish finish instead of primer.

- Flush ring handles instead of knob furniture.

- Intumescent seals and/or smoke seals.

- Hydraulic door closer.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	55

#### 3	Steel Personnel Door

SECTION 3

POLICE STATION (PMA) / CROWN COURT (TYPE E)

#### CellSecurity.co.uk

custodial and detention solutions

CSL0307A - 240 minute Fire Resistance (Non-Insulated) CSL0307B - 240 minute Fire Resistance LPS SR2 CSL0307C - 240 minute Fire Resistance LPS SR3 CSL0307D - 240 minute Fire Resistance LPS SR4

Specification

- Door leaf constructed from 1.2mm thick zinc coated steel skins bonded to a honeycomb or rigid thermally-insulating

core.

- Door leaf is mounted within a 1.6mm thick zinc coated steel double rebate frame and is hung from 3 or 4, stainless steel, ball bearing butt hinges.

- Reinforcing fitted within the door frame to suit various ironmongery options. “T” section astragal provided on double doors. 2.5mm heavy duty reinforcing channel incorporating an adjustable bottom rail allowing 16mm of height adjust- ment for uneven floors or smoke control purposes on fire resisting doors.

- Additional locking functions, attack skins and reinforcing on Gold and Silver doors.

Finish

  - One coat factory applied primer.

PERSONNEL DOORS

Options

  - Special frame to give up to 50mm width adjustment.

  - 1.1/2 pairs or double leaf door sets.

  - Over / side panels.

  - Anti-ligature hardware.

  - Threshold.

  - Square / rectangular / circular window.

  - Weather / smoke seals.

  - Louvres.

  - Factory finished to any B.S. or RAL colour.

SECTION

56

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Decency Door	3

SECTION 3

POLICE STATION (DEC)

CSL0308A - Decency Door	CSL0308B - Decency Door (DEC)

Specification

- Door is constructed from a 44mm thick solid timber core faced both sides in Formica with Ash lipping to all four edges.

- Door is mounted between two Ash posts and hung on three 100mm brass butt hinges.

- Frame fixings to suit a variety of opening construction.

- Supplied as standard with a heavy duty adjustable roller catch.

Finish

- White Formica to door faces.

- All hardwood to have two coats of water-borne varnish.

Options

PERSONNEL DOORS

Different coloured Formica to match site colour scheme.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	57

#### Armoury Door

SECTION 3

POLICE STATION

#### CellSecurity.co.uk

custodial and detention solutions

Specification

  - Door leaf is constructed from 10mm MS thick plate.

  - Frame constructed from 100 x 75 x 10 RSA and 100 x 10 flat.

  - Leaf hung on 3 No. strap hinges complete with 15 pins.

  - Supplied with 2No. Bramah locks.

  - Fitted with ‘D’ handle welded to outside.

  - Dog bolts fitted to inside to prevent leaf removal.

  - Frame fixings to suit a variety of opening constructions.

  - Constructed to comply with The Home Office Firearms Security Handbook 2005 and Health & Safety Executive Circular to Chief Officers of Police (No 1/2005).

Finish

PERSONNEL DOORS

One coat factory applied high build zinc phosphate primer.

SECTION

58

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Strongroom Door	3

SECTION 3

custodial and detention solutions	POLICE STATION

Specification

- Door leaf constructed from 40mm rolled hollow framework sheeted both sides with 3mm thick steel skins.

- Mineral wool insulation to all internal voids.

- 125mm x 75mm x 8mm rolled steel angle frame.

- Frame fixings to suit a variety of opening constructions.

- Door leaf mounted within door frame and hung on a substantial continuous hinge.

- Supplied with two ASSA deadlocks suited to differ.

- Fitted with one ‘D’ handle on outside, one cabin hook, one heavy duty barrel bolt on inside.

- Security strip on leading edge to resist forced entry.

Finish

- One coat factory applied high build zinc phosphate primer.

PERSONNEL DOORS

Options

- Rolled steel angle frame or profile frame to suit site conditions.

- Can be supplied with a Frazer Bar (free issue from the MOD) instead of the ASSA locks when used as an MOD emergency escape door.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	59

#### Grille Gates

SECTION 4

#### CellSecurity.co.uk

custodial and detention solutions

SECTION 4

Cell Security Limited provide a comprehensive range of Custody Grille Gates for cell or circulation corridors where the highest security and visual surveillance is essential.

All gates when closed secure the detention area. Built to a high specification, comply with the Home Office Standards for Police Stations, Magistrates Courts, Crown Courts, Prisons, HM Armed Forces, Custom and Excise, and any build- ings that require a detention facility.

Gates are a welded construction from substantial steel bars and hung on a pivot hinges.

Each gate will either be supplied with, or prepared for, a secure Home Office approved custody lock. Where applica- ble, we can supply electronic lock releases remotely activated from the custody desk to control circulation.

CSL0401

Medium Duty Gate

Police Station / Magistrates Court

Armed Forces

CSL0402

Medium Duty Gate & Fixed Panel Police Station / Magistrates Court Armed Forces

CSL0403

Heavy Duty Gate (GAT)

Police Station (PSF) / Magistrates Court Crown Court (Fa) / Prison

CSL0404

Heavy Duty Gate with Side Panel (GAT) Police Station (PSF) / Magistrates Court Crown Court (Fb) / Prison

CSL0405

Heavy Duty Gate

Prison

CSL0406

Surface Mounted Heavy Duty Gate

Prison

GRILLE GATES

CSL0408

Overhead Sliding Gate

Prison

CSL0409

Manhole Gate

Prison

CSL0410

Heavy Duty Gate (Type A)

Prison

SECTION

60

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Grille Gates	4

SECTION 4

SECTION 4

CSL0411

Heavy Duty Gate (Type C)

Prison

CSL0412

Heavy Duty Gate (Type M)

Prison

CSL0413

Sliding Gate

Police Station / Jail

GRILLE GATES

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	61

#### 4	Medium Duty Grille Gate

SECTION 4

POLICE STATION / MAGISTRATES COURT / ARMED FORCES

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Gate is constructed from 50mm wide steel RHS framework with 20mm diameter solid vertical bars.

- Mounted on strong pivot hinges.

- Frame fixings to suit a variety of opening construction.

- Home Office Type 2 heavy duty mortice deadlock.

Finish

  - One coat factory applied high build zinc phosphate primer.

Options

  - Surface mounted on ‘Outside’ of opening.

  - Electric release to lock.

  - Hydraulic Door Closer.

GRILLE GATES

Galvanized finish.

SECTION

62

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Medium Duty Gate & Fixed Panel	4

SECTION 4

POLICE STATION / MAGISTRATES COURT / ARMED FORCES

Specification

- Gate is constructed from 50mm wide steel RHS framework with 20mm diameter solid vertical bars.

- Mounted on strong pivot hinges.

- Frame fixings to suit a variety of opening construction.

- Home Office Type 2 heavy duty mortice deadlock.

Finish

- One coat factory applied high build zinc phosphate primer.

Options

- Any arrangement of header / side panels to suit requirements.

- Double leaf gate with joggle bolts.

- Hinged side panel.

- Electric release to lock.

- Hydraulic Door Closer.

GRILLE GATES

Galvanized finish.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	63

#### 4	Heavy Duty Gate

SECTION 4

POLICE STATION (GAT) / MAGISTRATES COURT / CROWN COURT (Fa) / PRISON

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Gate is constructed from 65mm x 12mm flat steel framework with 30mm diameter solid vertical bars.

- Mounted on Home Office pattern pivot hinges.

- Frame fixings to suit a variety of opening construction.

- Home Office Type 2 heavy duty mortice deadlock. (Except when gate used in Prisons or Crown Courts where the lock is supplied and installed by HMPS).

Finish

  - One coat factory applied high build zinc phosphate primer.

Options

  - 12V DC electric strike.

  - Hydraulic door closer.

GRILLE GATES

Galvanized finish.

SECTION

64

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Heavy Duty Gate with Side Panel	4

SECTION 4

POLICE STATION (GAT) / MAGISTRATES COURT / CROWN COURT (Fb) / PRISON

Specification

- Gate is constructed from 65mm x 12mm flat steel framework with 30mm diameter solid vertical bars.

- Mounted on Home Office pattern pivot hinges.

- Frame fixings to suit a variety of opening construction.

- Home Office Type 2 heavy duty mortice deadlock. (Except when gate used in Prisons or Crown Courts where the lock is supplied and installed by HMPS).

Finish

- One coat factory applied high build zinc phosphate primer.

Options

- Any arrangement of side or header panels can be provided.

- 12V DC electric strike.

- Hydraulic door Closer.

- Double leaf gate with joggle bolts.

GRILLE GATES

Galvanized finish.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	65

#### 4	Heavy Duty Gate

SECTION 4

PRISON

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Gate constructed from a 65mm x 12mm steel flat framework with 30mm diameter solid steel vertical bars and mounted within a steel hollow section frame, fixed by chemical resin anchors and hung from a full height tubular steel pivot hinge.

- Both gate and frame can be supplied with preparation for any mechanical and electrical locking variation specified.

- The frame is provided with removable panels to allow access to electrical wiring. (All removable panels are secured with tamper resistant screws, tamper proof switches can be supplied fitted and wired by others to monitor plate removal).

- Locks and electrical components are supplied and fitted by others.

Finish

  - One coat factory applied high build zinc phosphate primer.

Options

  - Galvanized finish.

GRILLE GATES

SECTION

66

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Surface Mounted Heavy Duty Gate	4

SECTION 4

PRISON

Specification

- Gate is constructed from 65mm x 12mm flat steel framework with 30mm diameter solid vertical bars.

- Mounted on Home Office pattern pivot hinges.

- The Gate is mounted within a RHS and RSA frame and is fixed by chemical resin anchors.

- Preparation for a Home Office Class 1 or 2 heavy duty mortice deadlock supplied and fitted by HMPS.

Finish

- One coat factory applied high build zinc phosphate primer.

Options

- Galvanized finish.

GRILLE GATES

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	67

#### 4	Heavy Duty Gate (Type A)

SECTION 4

PRISON

#### CellSecurity.co.uk

custodial and detention solutions

Specification

- Gate constructed from a 65mm x 12mm steel flat framework with 30mm diameter solid steel vertical bars and mounted within a steel hollow section frame, fixed by chemical resin anchors and hung from a full height tubular steel pivot hinge.

- Both gate and frame can be supplied with preparation for any mechanical and electrical locking variation specified.

- The frame is provided with removable panels to allow access to electrical wiring. (All removable panels are secured with tamper resistant screws, tamper proof switches can be supplied fitted and wired by others to monitor plate removal).

- Locks and electrical components are supplied and fitted by others.

Finish

- One coat factory applied high build zinc phosphate primer.

Options

- Galvanized finish.

GRILLE GATES

SECTION

70

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Heavy Duty Gate (Type C)	4

SECTION 4

PRISON

Specification

- Gate constructed from a 65mm x 12mm steel flat framework with 30mm diameter solid steel vertical bars and mounted within a steel hollow section frame, fixed by chemical resin anchors and hung from a full height tubular steel pivot hinge.

- Both gate and frame can be supplied with preparation for any mechanical and electrical locking variation specified.

- The frame is provided with removable panels to allow access to electrical wiring. (All removable panels are secured with tamper resistant screws, tamper proof switches can be supplied fitted and wired by others to monitor plate removal).

- Locks and electrical components are supplied and fitted by others.

Finish

- One coat factory applied high build zinc phosphate primer.

Options

GRILLE GATES

Galvanized finish.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	71

#### Heavy Duty Gate (Type M)

SECTION 4

PRISON

#### CellSecurity.co.uk

custodial and detention solutions

Specification

  - Gate constructed from a 65mm x 12mm steel flat framework with 30mm diameter solid steel vertical bars and mounted within a steel hollow section frame, fixed by chemical resin anchors and hung from a full height tubular steel pivot hinge.

  - Both gate and frame can be supplied with preparation for any mechanical and electrical locking variation specified.

  - The frame is provided with removable panels to allow access to electrical wiring. (All removable panels are secured with tamper resistant screws, tamper proof switches can be supplied fitted and wired by others to monitor plate removal).

  - Locks and electrical components are supplied and fitted by others.

Finish

    - One coat factory applied high build zinc phosphate primer.

Options

    - Galvanized finish.

GRILLE GATES

SECTION

72

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk

custodial and detention solutions

#### Sliding Gate	4

SECTION 4

PRISON/JAIL

Specification

- Gate constructed from 65mm x 12mm steel flat framework with 30mm diameter solid steel vertical bars.

- Full height steel angle slam post.

- Pressed steel sliding track with trolley hangers.

- Concealed bottom guide track.

- The gate is supplied as standard with a Class1 or 2 hook bolt lock.

Finish

- One coat factory applied high build zinc phosphate primer.

Options

- A pressed steel canopy can be provided to conceal the sliding track gear.

- Galvanized finish.

GRILLE GATES

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	73

#### Furniture

SECTION 5

#### CellSecurity.co.uk

custodial and detention solutions

SECTION 5

Cell Security Limited provide a comprehensive range of detention furniture for Cell or Custody use.

All furniture is designed and built to a high specification and complies with the Home Office Standards for Police Stations, Magistrates Courts, Crown Courts, Prisons, HM Armed Forces, Custom and Excise and any buildings that has a detention facility.

Cell Benches & Beds

CSL0501

Lambeth Cell Bench

CSL0502

Holding Room Bench

CSL0503

Cell Bench

CSL0504

Low Level Cell Bench

CSL0505

Cell Bench Top

CSL0506

Crown Court Cell Bench

CSL0507

Secure Hospital Bed

CSL0508

Wall Fixed Cell Single / Double Bunk Bed

CSL0509

Cell Double Bunk Bed

CSL0510

Wall Fixed Cell Single Bunk Bed

CSL0511

Wall Fixed Cell Double Bunk Bed

CSL0512

Cell Prison Double Bunk Bed

FURNITURE

CSL0513

Single Prison Cell Bed

CSL0514

Double Prison Cell Bunk Bed

CSL0515

Triple Prison Cell Bunk Bed

SECTION

74

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Furniture	5

custodial and detention solutions	SECTION 5

SECTION 5

Cell Wardrobes & Shelves

CSL0520

Cell Wardrobe

CSL0521

Open Cell Wardrobe

CSL0522

Cell Shelving Unit

CSL0523

Cell Shelf Unit With Snap Hooks

CSL0524

Cell Shelf

CSL0525

Ligature Resisting Snap Hooks

Chairs, Stools & Seats

CSL0530

Interview Chair

CSL0531

Plastic Cell Chair

CSL0532

Single Steel Waiting Chair

CSL0533

Multiple Steel Waiting chairs

CSL0534

Visits Table / Chairs

CSL0535

Folding Steel Stool

CSL0536

Steel Stool

CSL0537

Wall Mounted Steel Stool

CSL0538

FURNITURE

Timber Stool

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	75

#### 5	Furniture

SECTION 5

SECTION 5

#### CellSecurity.co.uk

custodial and detention solutions

CSL0539

Steel Mushroom Stool

CSL0540

Adjustable Height Photo Stool

CSL0541

Timber Peninsular Seat

CSL0542

Timber Island Seat

CSL0543

Timber Bench Seat

CSL0544

Timber Waiting Bench

CSL0545

Floor Mounted Pedestal Steel Waiting Bench

CSL0546

Steel Waiting Bench With Handcuff Hoops

CSL0547

Wall Mounted Steel Bench

Tables & Desks

CSL0550

Interview Table

CSL0551

Interview Table with Central Pedestal

CSL0552

Steel Cell Desk

FURNITURE

CSL0553

Steel Cell Writing Table

CSL0554

Steel Cell Writing Table with Combined Stool

CSL0555

Steel Cell Desk with Swivel Stool

SECTION

76

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Furniture	5

SECTION 5

custodial and detention solutions	SECTION 5

CSL0556

Solid Surface Cell Desk

CSL0557

Picnic Table

CSL0558

4 Person Dining Table

CSL0559

6 Person Dining Table

Secure Cupboards & Gun Cupboards

CSL0570

Medical Cabinet

CSL0571

Medicine Cabinet

CSL0572

Medicine Safe

CSL0573

Gun Cabinets

CSL0574

Ammunition Cabinet

CSL0575

FURNITURE

Secure Fire Cabinets

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	77

#### 5	Furniture

SECTION 5

SECTION 5

Secure Screens & Transfer Units

#### CellSecurity.co.uk

custodial and detention solutions

CSL0580

Interview Screen

CSL0581

Electronic Interview Screen

CSL0582

25mm Document Passing Tray

CSL0583

114mm Document Passing Tray

CSL0584

Electronic Transfer Units

CSL0585

Cell TV Cover

CSL0586

Charge Desk Monitor Cover

CSL0587

Mobile Workstation

Secure Ceilings

CSL0590

Secure Steel Ceiling

CSL0591

FURNITURE

Secure Mesh Ceiling

CSL0592

Corian Bulkhead

SECTION

78

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### 5	Cell Benches & Beds

SECTION 5

#### Lambeth Cell Bench

CellSecurity.co.uk

custodial and detention solutions

SECTION 5

Purpose

A cell bench complying with the Home Office and Metropolitan Police specifications to meet the criteria for maintaining a safe, secure and functional cell bench within the environment of a Police cell.

Specification

- Bench top is constructed from Corian solid surface material bonded to double 25mm thick Birch plywood sheet to give a finished thickness of 63mm with a special Home Office approved nosing detail.

- Supporting steel underframe is constructed from RHS framework with 2mm thick stainless steel pressed panels, the upper panel having ligature resisting perforations.

Finish

- Corian to be white.

- Stainless steel to be satin finished.

Options

- Non-perforated front panel if no under bench heating is required.

#### Holding Room Bench

Purpose

A holding room bench complying with the Home Office specifications to meet the criteria for maintaining a safe, secure and functional bench within the environment of a police holding room.

Specification

- Bench top is constructed from Corian solid surface material bonded to double 25mm thick birch plywood sheet to give a finished thickness of 63mm with a special Home Office approved nosing detail.

- Supporting steel underframe is constructed from RHS framework with 2mm thick stain- less steel pressed front panels with ligature resisting perforations.

Finish

- Corian to be white.

- Stainless steel to be satin finished.

Options

- Non-perforated front panel if no under bench heating is required.

FURNITURE

SECTION

80

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Cell Benches & Beds	5

SECTION 5

custodial and detention solutions	SECTION 5

#### Cell Bench

Purpose

A cell bench historically used in Police Stations and Magistrates Courts.

Specification

- Bench top is constructed from either 45mm thick Iroko hardwood battens or from Corian solid surface material bonded to plywood sheet to give a finished thickness of 38mm.

- Underside of bench top is protected from heat generated by the heating pipes with a layer of 6mm thick heat resisting fibreboard.

- Supporting steel underframe is constructed from 80mm x 60mm x 6 rolled steel angles and 3mm thick mild steel pressed panels, fully rebated all around to prevent passage of contraband.

- The two hinged panels allowing access to the heating pipes are held firmly shut by two security fixings.

Finish

- Timber bench top, two coats of water borne varnish.

- Corian in white. Other colours available at additional cost.

- Steelwork has one coat factory applied high build zinc phosphate primer.

Options

- Ash or Maple battens can be supplied.

- Corian supplied from a range of standard

colours.

- Bench can be supplied any length.

- 500mm wide bench for use in Magistrates

Courts.

- Fixed steel front without perforations for installations without heating.

Purpose

A low level cell bench historically used for ‘drunken prisoners’

Specification

#### Low Level Cell Bench

- Low level seating is constructed from either 45mm thick Iroko hardwood battens or Corian solid surface material bonded to plywood sheet to give a finished thickness of 38mm.

- Supporting steel underframe is constructed from rolled steel channel legs and 3mm thick mild steel pressed panels, fully rebated all around to prevent passage of contraband.

- Anti-ligature perforations are supplied within the top panels which is sloped to prevent

climbing.

- Underside of bench top is protected from heat generated by the heating pipes with a layer of 6mm thick heat resisting fibreboard.

- The hinged panels allowing access to the heating pipes are held firmly shut by two security fixings.

Finish

- Timber bench top, two coats of water borne varnish.

- Corian in white. Other colours available.

- Steelwork has one coat factory applied high build zinc phosphate primer.

FURNITURE

Options

- Ash or Maple battens can be supplied.

- Corian supplied from a range of standard

colours.

- Bench can be supplied any length.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	81

#### 5	Furniture

SECTION 5

#### Cell Bench Top

CellSecurity.co.uk

custodial and detention solutions

SECTION 5

Purpose

A cell bench top for use as a new or replacement to an existing bench top.

Specification

- Bench top is constructed from either 45mm thick Iroko hardwood battens or from Corian solid surface material bonded to plywood sheet to give a finished thickness of 38mm.

- Steel supports are constructed from 76mm x 38mm rolled steel channels, with fully welded fixings lugs at face and rear.

Finish

- Timber bench top, two coats of water borne varnish.

- Corian in white. Other colours available.

- Steelwork has one coat factory applied high build zinc phosphate primer.

Options

  - Ash or Maple battens can be supplied.

  - Corian supplied from a range of standard

colours.

  - Bench can be supplied any length.

  - 500mm wide bench for use in Magistrates

Courts.

#### Crown Court Cell Bench

Purpose

A cell bench complying with the specification of the Court Standard and meeting the criteria for maintaining a safe, secure and functional cell bench seat within the environment of an HM Crown Court designated cell unit.

Specification

- Bench top and backrest are constructed from 45mm thick Iroko hardwood battens.

- Supporting steel underframe is constructed from rolled steel angle legs with fully welded fixing lugs at face and rear.

Finish

- Timber bench top, two coats of water borne varnish.

- Steelwork has one coat factory applied high build zinc phosphate primer.

Options

- Ash or Maple battens can be supplied.

- Corian top.

- Bench can be supplied any length.

FURNITURE

SECTION

82

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Furniture	5

SECTION 5

custodial and detention solutions	SECTION 5

#### Secure Hospital Bed

Purpose

A secure steel bolt down bed with safety edge and restraints loops designed specifically for secure hospitals.

Specification

- Bed is constructed from a mild steel tube frame with 3mm thick mild steel mattress base com- plete with ventilation holes and central 50mm wide rolled hollow section stiffener.

- The frame legs are securely fixed to the floor through 6mm mild steel plates with heavy duty expansion bolts.

- Two restraint loops are provided along each side of the bed.

- A 100mm thick bed mattress can be contained within the bed frame.

Finish

- Polyester paint Oyster Grey.

Options

- Other polyester paint finishes available.

- Fire and vandal proof secure mattresses, sheets, blankets, and pillow cases, can be supplied.

#### Wall Fixed Single / Double Cell Bunk Bed

Purpose

Secure steel cantilevered bunk bed(s). Designed specifically for use within a prison cell.

Specification

- Bed(s) welded steel construction from 3mm thick mild steel sheet with folded safety

edge.

- Cantilevered design and securely fixed to wall through 5mm thick mild steel plates with heavy duty expansion bolts.

Finish

- Polyester paint Oyster Grey.

Options

- Integral ladder when double beds are required

- Other polyester paint finishes available.

FURNITURE

Fire and vandal proof secure mattresses, sheets, blankets, and pillow cases, can be supplied.

CSL0508A - Single

CSL0508B - Double with ladder

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	83

#### 5	Furniture

SECTION 5

#### Cell Double Bunk Bed

CellSecurity.co.uk

custodial and detention solutions

SECTION 5

Purpose

A secure steel double bunk bed with integral lockable storage for prisoner use. Designed specifically for use within a prison dormitory cell.

Specification

- Beds are constructed from 3mm thick mild steel sheet with formed safety edges.

- Robust welded steel framework.

- Incorporated ladders each end.

- Top bunk guard.

- Beds secured to the floor via heavy duty tamper resistant expansion bolts.

Finish

- Polyester paint Black.

Options

  - Can be provided either left or right hand (left hand shown).

  - Other polyester paint colours available.

  - Fire and vandal proof secure mattresses (pictured), pillows, sheets, blankets, and pillow cases, can be supplied.

#### Wall Fixed Cell Single Bunk Bed

Purpose

A secure steel cantilevered bunk bed with a safety edge. Designed specifically for use within a prison cell.

Specification

- Bed top is constructed from 3mm thick mild steel sheet with circular section safety edge welded around the perimeter of the front and side edges.

- Fixed directly to wall through a continuous fixing flange along the back and through rein- forcing gusset via tamper resistant expansion bolts.

- The exposed end of the bed has a pressed steel reinforcing gusset.

Finish

- Polyester paint Oyster Grey.

Options

- Can be provided either left or right hand (left hand shown).

- Other polyester paint colours available.

- Fire and vandal proof secure mattresses, seats, blankets, and pillow cases, can be supplied.

FURNITURE

SECTION

84

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Furniture	5

SECTION 5

custodial and detention solutions	SECTION 5

#### Wall Fixed Cell Double Bunk Bed

Purpose

A secure steel cantilevered double bunk bed with safety edges and top bunk safety guard. Designed specifically for use within a prison cell.

Specification

- Bed top is constructed from 3mm thick mild steel sheet with circular section safety edge welded around the perimeter of the front and side edges.

- Fixed directly to the wall through a continuous fixing flange along the back and through reinforcing gusset via tamper resisting expansion bolts.

- The exposed ends of the beds have pressed steel reinforcing gussets.

- The top bunk incorporates a welded steel safety guard.

- All steel construction ladder bolts between both bunks with security fixings.

Finish

- Polyester paint Oyster Grey.

Options

- Can be provided either left or right hand (right hand shown).

- Other polyester paint colours available.

- Fire and vandal proof secure mattresses, sheets, blankets, and pillow cases, can be supplied.

#### Wall Fixed Cell Double Bunk Bed

Purpose

A secure steel ‘assemble on site’ cantilevered bunk bed with integral ladder. Designed specifically for use within a prison cell.

Specification

- Beds are constructed from 3mm thick mild steel sheet with formed safety edges, mattress ventilation holes and bolted to the rigid endplate with security fixings.

- Endplate contains integral ladder.

- Beds secured to walls via heavy duty expansion bolts.

Finish

- Polyester paint Oyster Grey.

Options

- Can be provided either left or right hand (left hand

shown).

- Other polyester paint colours available.

FURNITURE

Fire and vandal proof secure mattresses, sheets, blankets, and pillow cases, can be supplied.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	85

#### 5	Furniture

SECTION 5

#### Single Prison Cell Bed

CellSecurity.co.uk

custodial and detention solutions

SECTION 5

Purpose

A secure steel floor standing bed designed specifically for use within a prison cell.

Specification

- Constructed from 50mm x 50mm x 5mm steel angle framework with a 3mm mild steel folded sheet bed top.

- Secured to the floor with heavy duty expansion bolts.

Finish

- Polyester paint Oyster Grey.

Options

- Other polyester paint colours available.

#### Double Prison Cell Bunk Bed

Purpose

A secure steel floor standing double bunk bed designed specifically for use within a prison cell.

Specification

- Constructed from 50mm x 50mm x 5mm steel angle framework with a 3mm mild steel folded sheet bed tops.

- Bed ends can be used as ladders for top bunk access.

- Secured to the floor with heavy duty expansion bolts.

Finish

- Polyester paint Oyster Grey.

Options

  - Other polyester paint colours available.

FURNITURE

SECTION

86

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Furniture	5

SECTION 5

custodial and detention solutions	SECTION 5

#### Triple Prison Cell Bunk Bed

Purpose

A secure steel floor standing triple bunk bed designed specifically for use within a prison cell.

Specification

- Constructed from 50mm x 50mm x 5mm steel angle framework with a 3mm mild steel folded sheet bed tops.

- Bed ends can be used as ladders for top bunk access.

- Secured to the floor with heavy duty expansion bolts.

Finish

- Polyester paint Oyster Grey.

Options

- Other polyester paint colours available.

#### Cell Wardrobe

Purpose

A robust secure steel floor standing wardrobe designed specifically for use within a prison cell.

Specification

- Constructed from 50mm x 50mm x 6mm steel angle framework with 3mm mild steel folded sheet.

- Hinged doors with magnetic catches, welded pull handles and welded continuous hinges.

- Wardrobe incorporates two welded shelves and clothes rail.

- Doors are perforated at high and low level for ventilation.

- Secured to the floor with heavy duty expansion bolts.

Finish

- Polyester paint Oyster Grey.

Options

- Other polyester paint colours available.

FURNITURE

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	87

#### 5	Furniture

SECTION 5

#### Open Cell Wardrobe

CellSecurity.co.uk

custodial and detention solutions

SECTION 5

Purpose

A robust secure steel floor standing wardrobe designed specifically for use within a prison cell.

Specification

- Constructed from 50mm x 50mm x 6mm steel angle framework with 3mm mild steel folded sheet.

- Wardrobe incorporates two welded shelves and clothes rail.

- Secured to the floor with heavy duty expansion bolts.

Finish

- Polyester paint Oyster Grey.

Options

- Other polyester paint colours available.

#### Cell Shelving Unit

Purpose

A robust secure steel wall shelving unit designed specifically for use within a prison cell.

Specification

- Construction from 3mm welded mild steel sheet.

- Secured to the wall with heavy duty expansion bolts.

Finish

- Polyester paint Oyster Grey.

Options

- Other polyester paint colours available.

FURNITURE

SECTION

88

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Furniture	5

SECTION 5

custodial and detention solutions	SECTION 5

#### Cell Shelf Unit with Snap Hooks

Purpose

A robust secure stainless steel wall shelf incorporating ‘snap hooks’ which collapse under a pre-set load to prevent ligature attachment. Suitable for use within a prison cell.

Specification

- Welded construction from 3mm thick grade 304 stainless steel.

- Secured to the wall with tamper proof expansion bolts.

- Three load adjustable snap hooks supplied as standard secured to the shelf from the rear to prevent tampering.

Finish

- Satin stainless steel.

Options

- Grade 316 stainless steel.

#### Cell Shelf

Purpose

A robust secure stainless steel wall shelf suitable for use within a prison cell.

Specification

- Welded construction from 3mm thick grade 304 stainless steel.

- Secured to the wall with tamper proof expansion bolts.

Finish

- Satin stainless steel.

Options

- Grade 316 stainless steel.

FURNITURE

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	89

#### 5	Furniture

SECTION 5

#### Ligature resisting snap hooks

CellSecurity.co.uk

custodial and detention solutions

SECTION 5

Purpose

Ligature resisting stainless steel snap hooks for clothes and towels which collapse under a predetermined load.

Specification

- Machined grade 304 stainless steel.

- Load adjustable.

- Type A front fixing.

- Type B rear fixing.

Finish

- Machine finish grade 304 stainless steel

CSL0525A - Front fixing CSL0525B - Rear fixing

Options

- Available in grade 316 stainless steel.

FURNITURE

CSL0538 - Timber Stool	CSL0542 - Timber Island Seat

SECTION

90

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Furniture	5

SECTION 5

custodial and detention solutions	SECTION 5

#### Interview Chair

Purpose

An Interview Room chair for maintaining the safety, security and functionality within a police station.

Specification

  - Chair seating is constructed from 45mm thick Ash battens, mounted on 40 x 40 x 3 RHS framework bolted to floor with heavy duty expansion bolts.

Finish

  - Timber has two coats of water borne varnish.

  - Steelwork is polyester paint black.

Options

  - Seats can be provided in alternative hardwood or Corian.

  - Steel frame can be provided in a range of colours.

#### Plastic Cell Chair

Purpose

A plastic chair commonly used in prison cells.

Specification

  - Moulded one piece plastic.

Finish

  - Available in black or white.

Options

  - Other colours available upon request.

FURNITURE

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	91

#### 5	Furniture

SECTION 5

#### Single Steel Waiting Chair

CellSecurity.co.uk

custodial and detention solutions

SECTION 5

Purpose

A robust steel chair suitable for waiting areas or interview rooms.

Specification

- Seat constructed from perforated 3mm thick mild steel.

- Chair base secured to the floor via heavy duty expansion bolts.

Finish

- Black and silver polyester paint.

Options

- Other colours available upon request.

#### Multiple Steel Waiting Chairs

Purpose

A robust row of three steel chairs suitable for waiting areas.

Specification

- Seat constructed from perforated 3mm thick mild steel.

- Chair bases secured to the floor via heavy duty expansion bolts.

Finish

- Black and silver polyester paint.

Options

- Other colours available upon request.

- Other seat multiples can be provided in a single unit.

FURNITURE

SECTION

92

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Furniture	5

SECTION 5

custodial and detention solutions	SECTION 5

#### Visits table & chairs

Purpose

A robust visits table with 4 or 6 chair assembly suitable for prison visits halls.

Specification

- Steel framework mandrel bent 3mm thick ERW MS tube, 75mm diameter.

- Completely welded construction with height adjustable feet to allow for uneven flooring.

- Table constructed from 32mm thick laminated MDF 950mm diameter and secured to frame using Pin Torx security machine screws, fixing through plate into concealed inserts.

- Chairs constructed from 10mm thick laminated beech ply shells. Secured to frame using Torx Pin security machine screws, fixing through plate into tamper resistant blind bushes.

Finish

- Steel framework white polyester paint.

- Table top in natural beech finish with lacquered edge.

Options

- Seat colour options below.

CSL0534A

4 Chair Arrangement

CSL0534B

6 Chair Arrangement

#### Folding Steel Stool

Purpose

A robust secure folding steel stool suitable for use within prison cells or detention areas.

Specification

- One piece welded construction.

- Seat constructed from 3mm thick mild steel.

- Can be secured in horizontal or vertical positions via spring loaded plunger.

- Secured to the wall with heavy duty expansion bolts.

Finish

- Polyester paint Oyster Grey.

Options

FURNITURE

Other colours available upon request.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	93

#### 5	Furniture

SECTION 5

#### Steel Stool

CellSecurity.co.uk

custodial and detention solutions

SECTION 5

Purpose

A robust secure steel stool suitable for use within prison cells or detention areas.

Specification

- One piece welded construction.

- Seat constructed from 3mm thick mild steel.

- Secured to the floor with heavy duty expansion bolts.

Finish

- Polyester paint Oyster Grey.

Options

- Other colours available upon request.

- Stainless steel seat.

#### Wall Mounted Steel Stool

Purpose

A robust wall mounted fixed and swing steel stool suitable for use within prison cells and detention areas.

Specification

CSL0537A

FIXED STEEL STOOL

CSL0537B

SWING STEEL STOOL

- Wall bracket constructed from welded steel.

- Seat constructed from stainless steel grade 304.

Finish

- Bracket - Polyester paint Oyster Grey.

- Seat - Satin stainless steel.

Options

- Other colours available upon request.

FURNITURE

SECTION

94

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Furniture	5

SECTION 5

custodial and detention solutions	SECTION 5

#### Timber Stool

Purpose

A single timber stool seat suitable for use with interview screens or in a detention area.

Specification

  - Seating is constructed from 45mm thick Iroko hardwood battens.

  - Mounted on tubular steel posts bolted to floor.

Finish

  - Timber bench top, two coats of water borne varnish.

  - Steel post is polyester powder coated, Black.

Options

  - Ash or Maple battens can be supplied.

#### Steel Mushroom Stool

Purpose

A robust steel mushroom stool suitable for use in detention areas.

Specification

  - Constructed from 3mm thick mild steel secured to the floor via an adaptor plate and heavy duty expansion bolts.

Finish

  - Polyester paint Oyster Grey.

Options

  - Other colours available upon request.

  - Stainless steel grade 304.

FURNITURE

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	95

#### 5	Furniture

SECTION 5

#### Adjustable Height Photo Stool

CellSecurity.co.uk

custodial and detention solutions

SECTION 5

Purpose

A robust adjustable height photo stool designed specifically for police stations.

Specification

- Welded tubular seat post secured to the floor with heavy duty expansion bolts.

- Seat constructed from 3mm thick stainless steel grade 304.

- Height adjustable by rotating seat.

- The seat can’t be removed from the post.

Finish

- Seat post polyester paint black.

- Seat satin stainless steel.

Options

  - Other colours available upon request.

#### Timber Peninsular Seat

Purpose

A robust timber peninsular seat designed specifically for use with interview screens.

Specification

- Welded steel tubular support post for securing to the floor plus steel angle bracket for fixing to the wall.

- Hardwood seat constructed from solid battens of Ash.

- Secured to the floor and wall with heavy duty expansion bolts.

Finish

- Timber seat top, two coats of water borne varnish.

- Steel posts is polyester paint, black.

Options

  - Seat can be provided in alternative hardwoods or

Corian.

  - Steel supports can be provided in a range of colours.

FURNITURE

SECTION

96

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Furniture	5

SECTION 5

custodial and detention solutions	SECTION 5

#### Timber Island Seat

Purpose

A robust timber island seat suitable for use with interview screens or in a detention area.

Specification

- Welded steel tubular support post for securing to the floor plus steel angle bracket for fixing to the wall.

- Hardwood seat constructed from solid battens of Ash.

- Secured to the floor and wall with heavy duty expansion bolts.

Finish

- Timber seat top, two coats of water borne varnish.

- Steel posts is polyester paint, black.

Options

- Seat can be provided in alternative hardwoods or Corian.

- Steel supports can be provided in a range of colours.

#### Timber Bench Seat

Purpose

A robust timber bench seat suitable for use with desks or in waiting areas.

Specification

- Steel frame constructed from 50mm x 50mm x 6mm angle steel seat support with 40mm x 40mm RHS legs.

- Hardwood seat constructed from solid battens of Ash.

- Secured to the floor and wall with heavy duty expansion bolts.

Finish

- Timber seat top, two coats of water borne varnish.

- Steel posts is polyester paint, black.

Options

- Seat can be provided in alternative hardwoods or Corian.

- Steel supports can be provided in a range of colours.

FURNITURE

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	97

#### 5	Furniture

SECTION 5

#### Waiting Bench

CellSecurity.co.uk

custodial and detention solutions

SECTION 5

Purpose

A robust timber waiting bench suitable for use in holding areas within detention facilities.

Specification

- Steel frame constructed from welded 40 x 40 RHS.

- Hardwood seat constructed from solid battens of ash.

- Secured to the floor and wall with heavy duty expansion bolts.

Finish

- Timber seat top, two coats of water borne varnish.

- Steel posts is polyester paint, black.

Options

- Seat can be provided in alternative hardwoods or Corian.

- Steel frame can be provided in a range of colours.

- Different lengths can be provided.

- Welded handcuff loops can be added to the steel underframe.

#### Floor Mounted Pedestal Steel Waiting Bench

Purpose

A robust steel floor mounted pedestal waiting bench. Suitable for use in holding / waiting areas within a detention facilities.

Specification

- Welded steel tubular support posts.

- Seat constructed from 3mm think mild steel folded sheet.

- Secured to the floor with heavy duty expansion bolts.

Finish

- Polyester paint Oyster Grey.

Options

- Other polyester colours available.

- Seat can be provided in stainless steel.

- Different lengths can be provided.

- Welded handcuff loops can be provided.

FURNITURE

SECTION

98

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Furniture	5

SECTION 5

custodial and detention solutions	SECTION 5

#### Steel Waiting Bench with Handcuff Loops

Purpose

A robust steel floor mounted waiting bench with handcuff loops. Suitable for use in holding / waiting areas within a detention facility.

Specification

- Welded steel frame constructed from 50mm x 50mm RHS.

- Seat constructed from 3mm thick mild steel folded sheet.

- Secured to floor with heavy duty expansion bolts.

- Two handcuff loops welded to the frame.

Finish

- Polyester paint Oyster Grey.

Options

- Other polyester colours available.

- Seat can be provided in stainless steel.

- Different lengths can be provided.

#### Wall Mounted Steel Bench

Purpose

A robust wall mounted steel bench suitable for use in holding / waiting areas within a deten- tion facility.

Specification

- Bench constructed from 3mm mild steel folded and welded to 5mm thick mild steel sup- port gussets.

- Secured to wall with heavy duty expansion bolts.

Finish

- Polyester paint Oyster Grey.

Options

- Other polyester colours available.

- Seat can be provided in stainless steel.

- Different lengths can be provided.

FURNITURE

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk	99

#### 5	Furniture

SECTION 5

#### Interview Table

CellSecurity.co.uk

custodial and detention solutions

SECTION 5

Purpose

A robust interview table suitable for use within police station and prison interview rooms.

Specification

- Table constructed from a solid timber top with Formica on top side and lipped on all four

edges with Ash.

- Mounted on 50 x 50 steel angle frame bolted floor.

- Secured to the floor with heavy duty expansion bolts.

Finish

- Steelwork polyester paint black.

- Formica is white.

Options

  - Table top can be constructed from solid timber battens to match seating.

  - Table top can be provided in Corian.

  - Formica can be to any colour within the B.S. range.

  - Steel frame can be provided in alternate paint colours.

#### Interview Table with Central Pedestal

Purpose

A robust interview table with central pedestal. Suitable for use within police station and prison interview rooms where disabled access is required.

Specification

- Table constructed from a solid timber top with Formica on top side and lipped on all four

edges with Ash.

- Mounted on substantial centralised steel pedestal bolted to the floor with heavy duty expansion bolts.

Finish

- Steel pedestal polyester paint black.

- Formica is white.

Options

- Table top can be constructed from solid timber battens to match seating.

- Table top can be provided in Corian.

- Formica can be to any colour within the B.S. range.

FURNITURE

Steel pedestal can be provided in alternate paint colours.

SECTION

100

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Furniture	5

SECTION 5

custodial and detention solutions	SECTION 5

#### Steel Cell Desk

Purpose

A robust steel cell desk suitable for use in a prison cells.

Specification

- Desk constructed from pressed 3mm thick mild steel sheet with no sharp edges.

- Welded steel brackets secured to wall with tamper resistant expansion bolts.

Finish

- Polyester paint Oyster Grey.

Options

- Other polyester colours available.

- Desk can be provided in stainless steel.

- Different lengths can be provided.

#### Steel Cell Writing Table

Purpose

A robust steel cell writing table suitable for use in prison cells.

Specification

- Table is constructed from pressed 3mm thick mild steel sheet with no sharp edges.

- Fixed directly to wall with tamper resistant expansion bolts.

Finish

- Polyester paint Oyster Grey.

Options

- Other polyester colours available.

- Table can be provided in stainless steel.

- Different lengths can be provided.

FURNITURE

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

101

#### 5	Furniture

SECTION 5

#### CellSecurity.co.uk

custodial and detention solutions

#### Steel Cell Writing Table with Combined Stool

SECTION 5

Purpose

A robust steel cell writing table with combined stool suitable for use in prison cells.

Specification

- Welded steel tubular support frame.

- Table and seat constructed from stainless steel grade 304.

- Secured to floor and wall with tamper resistant expansion bolts.

Finish

- Tubular support frame - polyester paint Oyster Grey.

- Table and seat satin stainless steel.

Options

  - Tubular support frame can be provided in alternative colours.

  - Table and seat can be provided in mild steel with paint finish.

#### Steel Cell Desk with Swivel Stool

Purpose

A robust steel cell desk incorporating a swivel stool and shelves suitable for use in prison cells.

Specification

- Desk is constructed from pressed 3mm thick mild steel sheet with no sharp edges.

- Strong swivel stool with stainless steel seat grade 304.

- Desk incorporates two shelves.

- Secured to floor and wall with tamper resistant expansion bolts.

- Available left or right handed (left hand shown).

Finish

- Desk - Polyester paint Oyster Grey.

- Seat - Satin stainless steel.

Options

- Other polyester paint colours available.

- Desk can be provided without a swivel seat for disabled access.

FURNITURE

SECTION

102

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Furniture	5

custodial and detention solutions	SECTION 5

SECTION 5

Solid Surface Cell Desk

Purpose

A robust solid surface cell desk suitable for use in high security cells.

Specification

- Desk constructed from marine birch ply with all faces and edges laminated with 12mm thick Corian solid service material.

- Desk incorporates two shelves.

- Secured to floor and wall with concealed heavy duty expansion bolts.

- Available left or right handed (left hand shown).

Finish

- Fabricated solid surface Corian.

Options

- Various Corian colour options available.

#### Picnic Table

Purpose

A robust 6 person picnic table suitable for use in a prison dining and association areas.

Specification

- Seats and table top constructed from pressed 3mm thick grade 304 stainless steel.

- Underframe supports constructed pressed 3mm thick mild steel.

- Assembled on site with tamper proof bolts.

- Secured to the floor with heavy duty expansion bolts.

Finish

- Seats and table top satin stainless steel brushed finish.

- Underframe supports polyester paint Oyster Grey.

Options

- Underframe supports can be provided in stainless steel.

FURNITURE

Other polyester paint colours available.

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

103

#### 5	Furniture

SECTION 5

#### Person Dining Table

CellSecurity.co.uk

custodial and detention solutions

SECTION 5

Purpose

A robust 4 person dining table suitable for use in a prison dining and association areas.

Specification

  - Seats and table top constructed from pressed 3mm thick grade 304 stainless steel.

  - Welded underframe supports constructed from steel rolled hollow sections.

  - Assembled on site with tamper proof bolts.

  - Secured to the floor with heavy duty expansion bolts.

Finish

  - Seats and table top satin stainless steel brushed finish.

  - Underframe supports polyester paint Oyster Grey.

Options

  - Underframe supports can be provided in stainless steel.

  - Other polyester paint colours available.

#### 6 Person Dining Table

Purpose

A robust 6 person dining table suitable for use in a prison dining and association areas.

Specification

- Seats and table top constructed from pressed 3mm thick grade 304 stainless steel.

- Welded underframe supports constructed from steel rolled hollow sections.

- Assembled on site with tamper proof bolts.

- Secured to the floor with heavy duty expansion bolts.

Finish

- Seats and table top satin stainless steel brushed finish.

- Underframe supports polyester paint Oyster Grey.

Options

- Underframe supports can be provided in stainless steel.

- Other polyester paint colours available.

FURNITURE

SECTION

104

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Furniture	5

SECTION 5

custodial and detention solutions	SECTION 5

#### Medical Cabinet

Purpose

The Medical Storage Cabinets are suitable for storing general First Aid items.

Specification

- General First Aid equipment storage.

- 1.5mm steel cabinet with flush closing door.

- Lockable inner compartment c/w 2 keys.

- Supplied with 2 removable shelves.

- Supplied with security lock and 2 keys.

- Dimensions external H550mm x W380mm x D205mm

Finish

- Easy clean gloss white paint finish will suit most environments.

#### Medicine Cabinet

Purpose

The Medicine Cabinets are perfect for controlled drugs that are governed by the Home Office Drugs Inspectorate direction of control.

Specification

- High security cabinet for the safekeeping of controlled drugs / medicines.

- 4mm Solid steel electrically welded body.

- 8mm thick door fitted with VdS class 1 double bitted safe lock & 2 keys.

- Supplied with 2 adjustable shelves.

- Suitable for rear or floor fixing - Bolts supplied.

- Dimensions external H532mm x W374mm x D230mm

Finish

- Easy clean gloss white paint finish will suit most environments.

FURNITURE

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

105

#### Furniture

SECTION 5

#### Medicine Safe

CellSecurity.co.uk

custodial and detention solutions

SECTION 5

Purpose

The Medicine Safe are perfect for controlled drugs that are governed by the Home Office Drugs Inspectorate direction of control.

Specification

- Maximum security for the safekeeping of controlled drugs / medicines.

- 3-way square locking boltwork, with continuous hinge side protection.

- Door opens to 205 degrees for maximum access.

- Lightweight construction - Ideal for upper floor installation.

- Fitted with VdS class 1 double bitted safe lock & 2 keys.

- Optional combination or electronic locking available.

- Internal door pan is locked to prevent tampering.

- Supplied with adjustable (50mm intervals) / removable shelf.

- Twin fixing positions - Bolts supplied.

Finish

- Easy clean satin grey paint finish will suit most environments.

Dimensions

- Width 410 mm

- Height 415 mm

- Depth 345 mm

- Weight 75 kg

- Capacity 27 litres

#### Gun Cabinets

Purpose

These cabinets exceed the UK standard for storing firearms and utilises multi point locking with a VdS class safe lock. The cabinet includes a Master 90D trigger lock to prevent unauthor- ised use of your weapon during transportation.

Specification (Type A)

Type A

Type B

- High security cabinet for the safekeeping of shotguns and rifles.

- Exceeds the UK standard for storing firearms.

- 4mm solid steel body and door.

- Multi point locking with VdS class safe lock and 2 keys.

- Weapons holder included for up to 7 weapons.

- 4 handy storage compartments built into the rear of the door.

- Base and rear fixing holes provided.

- Includes FREE Master Lock 90D gun trigger lock.

Specification (Type B)

- High security cabinet for the safekeeping of shotguns and rifles.

- Exceeds the UK standard for storing firearms.

- 4mm solid steel body and door.

- Multi point locking with VdS class safe lock and 2 keys.

- Weapons holder included for up to 9 weapons.

- Includes single fixed shelf.

- 4 handy storage compartments built into the rear of the door.

- Base and rear fixing holes provided.

- Includes FREE Master Lock 90D gun trigger lock.

Finish

- Colour - Metallic Dark Grey RAL7001.

FURNITURE

SECTION

106

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Furniture	5

SECTION 5

custodial and detention solutions	SECTION 5

#### Ammunition Cabinet

Purpose

High security cabinet for the safekeeping of firearms ammunition.

Specification

  - 4mm Solid steel electrically welded body.

  - 8mm thick door fitted with VdS class 1 double bitted safe lock and 2 keys.

  - Supplied with 2 adjustable shelves.

  - Suitable for rear or floor fixing - Bolts supplied.

Finish

  - Colour - Metallic Dark Grey RAL7001.

#### Secure Fire Cabinets

Purpose

Our range of secure fire and burglary resistant storage cabinets are an affordable solution for the safekeeping of both valuables and documents.

The cabinets are double-walled with protective filling, providing 30 minutes resistance at tem- peratures of up to 750˚C.

Each model is fitted as standard with five heavy-duty round bolts secured on three sides and a VdS approved safe lock.

Specification

  - Ideal in the office, for the safe keeping of paper documents and valuables.

  - Suitable for up to £2,000 cash (10 x valuables).

  - Independently tested and certified to EN 14450 S1 by VdS test house.

  - Double wall construction with fire resistant fill.

  - Up to 30 minute protection for documents at 750 degrees.

  - Heavy duty 3-way locking mechanism.

  - Full height dog bar protects hinge side.

  - Standard locking - CEN/VdS approved double bitted lock and 2 keys.

  - Optional inner compartments and pull-out frames available.

Finish

  - Light Grey finish complements any environment.

CSL0575A

1950(H) x 930(W) x 520(D)

CSL0575B

1580(H) x 600(W) x 520(D)

CSL0575C

1220(H) x 600(W) x 520(D)

CSL0575D

FURNITURE

860(H) x 600(W) x 520(D)

CSL0575E

1950(H) x 1200(W) x 520(D)

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

107

#### 9	Stainless Steel WC Pans

SECTION 9

SECTION 9

#### WC Pan - Horizontal Outlet

Specification

#### CellSecurity.co.uk

custodial and detention solutions

- Constructed in one piece without joints or crevices from 2mm thick grade 304 stainless

steel.

- Ligature resisting security flushing rim with a gap of less than 2mm.

- Filled with sound deadening fire retardant expanding foam to prevent drumming.

- Supplied with concealed floor fixing plate and four M12 x 330mm long fixing bolts.

- Ergonomically designed contoured seat.

Compatible With

Pneumatic system CSL0981 Cistern CSL0982 Push Button

Electrical system

CSL0981 Cistern

CSL0983 Electrical Control Button CSL0985 Electrical Syphon CSL0986 Electrical Control Box

Other

CSL0915 Dry Cell Toilet Cover

Finish

- External surface of bowl is polished to a satin finish.

- Internal surface polished to a ultra smooth finish that is hygienic, easy to clean and resist- ant to scale build-up and deposits.

Options

- Also available in grade 316 stainless steel to order.

- Stove enamelled.

#### WC Pan - Vertical Outlet

Specification

Compatible With

Pneumatic system CSL0981 Cistern CSL0982 Push Button

Electrical system

CSL0981 Cistern

CSL0983 Electrical Control Button CSL0985 Electrical Syphon CSL0986 Electrical Control Box

SANITARY WARE

Other

CSL0915 Dry Cell Toilet Cover

- Constructed in one piece without joints or crevices from 2mm thick grade 304, stainless

steel.

- Ligature resisting security flushing rim with a gap of less than 2mm.

- Filled with sound deadening fire retardant expanding foam to prevent drumming.

- Removable side panel to allow for easy outlet access.

- Supplied with concealed floor fixing plate and four M12 x 330mm long fixing bolts.

- Ergonomically designed contoured seat.

Finish

- External surface of bowl is polished to a satin finish.

- Internal surface polished to a ultra smooth finish that is hygienic, easy to clean and resist- ant to scale build-up and deposits.

Options

- Also available in grade 316 stainless steel to order.

- Stove enamelled.

SECTION

150

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Stainless Steel WC Pans	9

SECTION 9

custodial and detention solutions	SECTION 9

#### WC Pan - Disabled

Specification

- Constructed in one piece without joints or crevices from 2mm thick grade 304, stainless

steel.

- Ligature resisting security flushing rim with a gap of less than 2mm.

- Filled with sound deadening fire retardant expanding foam to prevent drumming.

- Removable side panel to allow for easy outlet access.

- Supplied with concealed floor fixing plate and four M12 x 330mm long fixing bolts.

- Ergonomically designed contoured seat.

Finish

- External surface of bowl is polished to a satin finish.

- Internal surface polished to a ultra smooth finish that is hygienic, easy to clean and resist- ant to scale build-up and deposits.

Options

- Also available in grade 316 stainless steel to order.

- Stove enamelled.

Compatible With

Pneumatic system CSL0981 Cistern CSL0982 Push Button

Electrical system

CSL0981 Cistern

CSL0983 Electrical Control Button CSL0985 Electrical Syphon CSL0986 Electrical Control Box

Other

CSL0915 Dry Cell Toilet Cover CSL1011 Doc M Pack

Specification

#### WC Pan - Disabled

- Constructed in one piece without joints or crevices from 2mm thick grade 304 stainless

steel.

- Ligature resisting security flushing rim with a gap of less than 2mm.

- Self draining and an ergonomically designed contoured seat. Filled with sound deadening fire retardant expanding foam to prevent drumming.

- Removable cistern top with tamper proof fixings.

- Removable side panel to allow for easy outlet access.

- Supplied in electronic or pneumatic operation.

Finish

- External surface of bowl is polished to a satin finish.

- Internal surface polished to a ultra smooth finish that is hygienic, easy to clean and resist- ant to scale build-up and deposits.

Options

- Also available in grade 316 stainless steel to order.

- Stove enamelled.

CSL0907E

Electrical Operation

CSL0907P

SANITARY WARE

Pneumatic Operation

CSL0907DE

Disabled Electrical Operation

CSL0907DP

Disabled Pneumatic Operation

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

151

#### CellSecurity.co.uk	Sanitary Ware	9

SECTION 9

custodial and detention solutions	SECTION 9

#### Dry Cell Toilet Cover

Specification

- A unique robust tamper resistant wc pan cover.

Transforms a normal cell into a dry cell to prevent the flushing of contraband.

- Ergonomically designed to reduce ligature risks.

- Easy fitment with only a single tamper proof security fixing.

- Installation / removal tool included.

- Compatible with CSL0901, CSL0902, CSL0903, CSL0904, CSL0905, CSL0906 & CSL0907.

Finish

- Polyester powder coated.

- Stainless steel tamper proof security fixing.

#### Drugs Interceptor

Specification

- Constructed from 1.5mm grade 304 stainless steel with all exposed surfaces polished to a satin finish.

- The unit incorporates 3 support legs with height adjustable feet, a wash down inlet con- nection pipe, waste inlet pipe for connection to WC in cell, an integral contraband collec- tion tray, central strainer and a “P” trap outlet that can be rotated to any position.The unit is supplied with two heavy duty rubber gloves and a clear toughened glass lid for sealed access and inspections.

SITE INSTALLATION INFORMATION

- When installing the unit the following considerations to be made.

- Height of foul water inlet and outlet.

- Ensure there is an adequate fall of the toilet waste pipe.

- Height adjustment of support legs 40mm maximum.

Finish

- Satin polished stainless steel.

SANITARY WARE

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

153

#### 9	Sanitary Ware

SECTION 9

SECTION 9

#### Compact Drugs Interceptor

Specification

#### CellSecurity.co.uk

custodial and detention solutions

- Constructed from 1.5mm grade 304 stainless steel with all exposed surfaces polished to a satin finish

- The unit incorporates a wash down inlet connection pipe operated from the front, waste inlet pipe for connection to WC in the cell, an integral hinged contraband collection tray and an acrylic collection removable pot.

- The unit is supplied with one heavy duty right handed rubber glove and a clear polycar- bonate inspection lid.

- Inspection light included, requires AC voltage.

- The hinged collection tray can be left in the vertical position which allows the WC to be

used normally.

- Compact interceptor ideal for use within the duct area.

SITE INSTALLATION INFORMATION

- When installing the unit the following considerations to be made.

- Height of foul water inlet and outlet.

- Ensure there is an adequate fall of the toilet waste pipe.

Finish

- Satin polished stainless steel.

SANITARY WARE

CSL0901 - Stainless Steel Horizontal WC Pan Stove Enamelled

SECTION

154

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Sanitary Ware	9

SECTION 9

custodial and detention solutions	SECTION 9

#### Combination Units

Specification

- Constructed 1.5mm thick grade 304 stainless steel.

- Filled with sound deadening fire retardant expanding foam to prevent drumming.

- Incorporated soap dish.

- Direct flushing as standard.

- Included time flow control valve for washbasin and flushing valve.

- The toilet pan is hung off the combination unit for ease of cleaning under- neath.

- Secured to the wall via fixings supplied.

Finish

- External surface of bowl is polished to a satin finish.

- Internal surface polished to a ultra smooth finish that is hygienic, easy to clean and resistant to scale build-up and deposits.

Options

CSL0931A - Straight Unit CSL0931B - Right Handed Unit CSL0931C - Left Handed Unit

- Available in straight, right and left configurations.

- Toilet roll holder.

- Drinking Water option with third flow control valve.

- Other models are available upon request.

- Also available in grade 316 stainless steel to order.

Right Handed Unit

(B)

Left Handed Unit

(C)

Left Handed Unit

(C)

(OPTIONAL)

Right Handed Unit

(B)

(STD)

(STD)

SANITARY WARE

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

155

#### 9	Stainless Steel Washbasins

SECTION 9

SECTION 9

#### Washbasin - Wall Hung

Specification

#### CellSecurity.co.uk

custodial and detention solutions

Pictured with taps for

illustration purposes only

- Constructed as a totally enclosed unit in one piece without joints or crevices from 2mm grade 304

Stainless Steel.

- Exposed surfaces of the basin are polished to a satin finish.

- The rear upstand is incorporated for ease of cleaning and hygiene.

- 1”1/4 BSP male waste outlet connector.

- Two tap holes.

- Blanking plug is also included if only one tap is required.

Finish

- Polished stainless steel.

Options

- Can be supplied with removable access cover on the front panel.

- Grade 316 stainless steel available to order.

- Stove enamelled.

Optional (with removable access cover)

#### Washbasin - Floor Standing

Specification

Pictured with taps for

illustration purposes only

- Constructed as a totally enclosed unit in one piece without joints or crevices from 2mm grade 304

Stainless Steel with access panel secured with security fixings.

- Exposed surfaces of the basin are polished to a satin finish.

- The rear upstand is incorporated for ease of cleaning and hygiene.

- The basin’s front panel extends downward with an access panel which protects the supply

and waste pipes with removable access cover secured with tamper proof fixing screws.

- 1”1/4 BSP male waste outlet connector.

- Two tap holes.

- Blanking plug is also included if only one tap is required.

Finish

  - Polished stainless steel.

Options

  - Grade 316 stainless steel available to order.

  - Stove enamelled.

SANITARY WARE

SECTION

156

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

#### CellSecurity.co.uk	Stainless Steel Washbasins	9

SECTION 9

custodial and detention solutions	SECTION 9

#### Washbasin - Wall Hung Corner

Specification

- Constructed in one piece without joints or crevices from 2mm grade 304 Stainless Steel.

- Exposed surfaces of the basin are polished to a satin finish.

- The rear upstand is incorporated for ease of cleaning and hygiene.

- 1”1/4 BSP male waste outlet connector.

- Two tap holes.

- Blanking plug is also included if only one tap is required.

Finish

- Polished stainless steel.

Options

- Grade 316 stainless steel available to order.

- Stove enamelled.

#### Washbasin - Floor Standing Corner

Pictured with taps for illustration purposes only

Specification

- Constructed in one piece without joints or crevices from 2mm grade 304 stainless steel.

- Exposed surfaces of the basin are polished to a satin finish.

- The rear upstand is incorporated for ease of cleaning and hygiene.

- The basin’s front panel extends downward with an access panel which protects the supply and waste pipes with removable access cover secured with tamper proof fixing screws.

- 1”1/4 BSP male waste outlet connector.

- Two tap holes.

- Blanking plug is also included if only one tap is required.

Finish

- Polished stainless steel.

Options

- Grade 316 stainless steel available to order.

SANITARY WARE

Stove enamelled.

Pictured with taps for illustration purposes only

Cell Security Limited - Lynstock Way - Lostock - Bolton - BL6 4SA - England

Tel +44 (0) 1204 669690 Fax +44 (0) 1204 669650 Email sales@cellsecurity.co.uk Web www.cellsecurity.co.uk

157